| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0061 |
| --- | --- |

| Title: | AHG7: Slice header prediction for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The values of many slice header parameters of the view component of the non-base views in an access unit are identical to those of the base view in the same access unit. A slice header prediction scheme is proposed in this document in order to take advantage of this observation. In the proposed scheme, for non-base views, some of the slice header parameters may not be explicitly present, but are rather predicted or inferred from the first slice header of the view component of the base view.

# Introduction

The values of many slice header syntax elements are identical across different slices of the same picture. The same is also true for slice header syntax elements across multiple views. Predicting the information for these parameters in the non-base views from those in the base views could improve signaling efficiency. When prediction is not possible, the slice header syntax may allow for the syntax element to be explicitly signalled in the slice header.

Slice header prediction has been previously discussed in JCT-VC in JCTVC-I0070 and JCTVC-J0109. It was discussed that slice-header prediction would be promising for non-base views with no change to the base view.

Not all the syntax elements in the slice header may be predictable and it might be unnecessary to control the prediction of different syntax elements separately. Hence, syntax elements are categorized based on the functionality and each category of syntax elements can be independently selected to be signalled or to be predicted. A detailed description of the proposal is given in the following section.

# Proposal

The first slice header of the view component of the base view is chosen for predicting the slice header syntax elements in the non-base views. The syntax elements of the slice header which can be predicted are categorized into four types:

- Common information – these include syntax elements that do not change for all the slices in all the views; e.g., POC LSB and reference picture set information

- Reference picture list related information – these include the syntax elements that are related to the reference picture lists, including list sizes and list indices

- Prediction weight table information – these include the syntax elements that are used to signal the parameters for weighted prediction.

- Deblocking parameters information – these include syntax elements that are related to signaling of the deblocking parameters.

Syntax elements in the slice header are categorised such that the syntax elements in each category are most likely to be predicted together. The changes to the syntax table are highlighted below; the syntax elements that have been moved to help the categorisation are highlighted in green and the rest of the changes are highlighted in yellow.

## Slice header syntax for non-base views

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| common_info_pred_flag | u(1) |
| if( !common_info_pred_flag ) { |  |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| } |  |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| for ( i = 0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_undetermined_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( !common_info_pred_flag ) |  |
| common_info_table() |  |
| ref_pic_list_related_info_pred_flag | u(1) |
| if( !ref_pic_list_related_info_pred_flag ) |  |
| ref_pic_list_related_info_table() |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) { |  |
| pred_weight_pred_flag | u(1) |
| if( !pred_weight_pred_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| deblocking_para_table_pred_flag | u(1) |
| if( !deblocking_para_table_pred_flag ) |  |
| deblocking_para_table() |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

## Slice header semantics

common_info_pred_flag equal to 0 indicates that the syntax elements in the common_info_table() and no_output_prior_pics_flag and slice_pic_parameter_set_id are not predicted from any slice header of a base view and are explicitly present. common_info_pred_flag equal to 1 indicates that syntax elements in the common_info_table() and no_output_prior_pics_flag and slice_pic_parameter_set_id are the same as in the first slice of the base view in the same access unit.

ref_pic_list_related_info_pred_flag equal to 0 indicates that the syntax elements in the ref_pic_list_related_info_table() are not predicted from any slice header of a base view and are explicitly present. ref_pic_list_related_info_pred_flag equal to 1 indicates that syntax elements in the ref_pic_list_related_info_table() are the same as in the first slice of the base view in the same access unit.

pred_weight_pred_flag equal to 0 indicates that the syntax elements in the pred_weight_table() are not predicted from any slice header of a base view and are explicitly present. pred_weight_pred_flag equal to 1 indicates that syntax elements in the pred_weight_table() are the same as in the first slice of the base view in the same access unit.

deblocking_para_table_pred_flag equal to 0 indicates that the syntax elements in the deblocking_para_table() are not predicted from any slice header of a base view and are explicitly present. deblocking_para_table_pred_flag equal to 1 indicates that syntax elements in the deblocking_para_table() are the same as in the first slice of the base view in the same access unit.

## Common information table

| common_info_table() { |  |
| --- | --- |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps && num_long_term_ref_pics_sps > 1) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The existing semantics of each syntax element x in common_info_table() applies with the following addition.

When common_info_pred_flag is zero, the value of x is inferred to be equal to the value of the syntax element x in the first slice header of the base view.

## Reference picture list-related information table

| ref_pic_list_related_info_table() { |  |
| --- | --- |
| if( !IdrFlag ) |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| } |  |

The existing semantics of each syntax element x in ref_pic_list_related_info_table() applies with the following addition.

When ref_pic_list_related_info_pred_flag is zero, the value of x is inferred to be equal to the value of the syntax element x in the first slice header of the base view.

## Deblocking parameters table

| deblocking_para_table() { |  |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| s slice_disable_deblocking_filter_flag | u(1) |
| if( !slice_disable_deblocking_filter_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The existing semantics of each syntax element x in deblocking_para_table() applies with the following addition.

When deblocking_para_pred_flag is zero, the value of x is inferred to be equal to the value of the syntax element x in the first slice header of the base view.

## Prediction weight table semantics

The existing semantics of each syntax element x in pred_weight_table() applies with the following addition.

When pred_weight_pred_flag is zero, the value of x is inferred to be equal to the value of syntax element x in the first slice header of the base view.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).