| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0064 |
| --- | --- |

| Title: | Temporal motion vector prediction hook for efficient merge mode in MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang Vadim Seregin Li Zhang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Kemal Ugur Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com yekuiw@qti.qualcomm.com vseregin@qti.qualcomm.com lizhang@qti.qualcomm.com martak@qti.qualcomm.com kemal.ugur@nokia.com miska.hannuksela@nokia.com |
| Source: | Qualcomm Incorporated and Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution is submitted to JCT-3V to collect feedback, although the proposal is more essential to JCT-3V as it targets at changing the HEVC version 1, the proposal itself is mainly for more efficient MV-HEVC coding. The following is a copy of the content of document proposed to JCT-VC, as in JCTVC-L0257.

[Start of the copied content of the JCTVC-L0257]

# Abstract

In the context of multiview or 3DV coding, reference index equal to zero in merge mode may correspond to the reference picture in the same view, while the motion vector (MV) of the co-located PU may point to an inter-view reference picture which is marked as long-term. In this case, TMVP candidate is considered as unavailable. To address this issue, it is proposed that in this case the motion vector is still available, with a changed target reference index (which is non-zero). For multiview video coding (MV-HEVC), the proposed method provides about 0.94% average bitrate saving for the all the views and 2.5% bitrate saving for the non-base views. As only high-level syntax changes are allowed for MV-HEVC, the change is proposed for HEVC version 1.

The contribution proposes the following options for deriving the changed target reference index for the merge mode:

- In HEVC version 1, the changed target reference index is always set equal to 0 during the invocation of the reference picture list construction process. This option has reportedly the smallest impact among all the options for decoding processes.

- Indicating the changed target reference index in the slice segment header. This option has reportedly the second smallest impact among all the options for decoding processes, as no derivation of the changed target reference index is required.

- Deriving the changed target reference index to be equal to the smallest reference index which has a different marking (as used as short-term or long-term reference) from that of reference index 0. This option was presented in JCTVC-K0239.

- In the case the co-located PU points to a reference picture having a different layer identifier (equal to layerA) than that for reference index 0, deriving the changed target reference index to be equal to the smallest reference index that has layer identifier equal to layerA. This option can reportedly support more than one potential inter-view prediction source (e.g. for B views) unlike the previous two options. Furthermore, the derivation of the changed target reference index is never invoked for HEVC v1 bitstreams.

One of the options is proposed for adoption in HEVC version 1.

# Introduction

## Multiview video coding

MV-HEVC is the multiview extension of HEVC to support multiview video coding and it is being developed under JCT-3V. Similar to MVC (the multiview extension of AVC), MV-HEVC doesn’t contain any low-level changes beyond HEVC. In MV-HEVC, an inter-view reference picture is marked as long-term.

In MV-HEVC, one view can be predicted from the other by the so-called disparity motion compensation. It is realized in a way that a decoded view component from a different view but in the same time instance can be added in a reference picture list of the current view component. For example, as shown in Figure 1, wherein the vertical indices (V1, V0 and V2) of the pictures correspond to view identifiers and the horizontal indices (T0 through T11) correspond to POC values. Each square is a view component and when a vertical arrow links view component A (e.g., T5/V0) to another view component B (e.g., T5/V1, in the same time instance), view component A can be added into the reference picture list 0 of view component B.

Figure 1:  Typical motion prediction structure in 3DV (texture only).

Note that in the above example, both view component A and view component B have the same POC value (5).

A motion vector pointing to a view component of a different view is called disparity motion vector in this document and this view component is referred to as an inter-view reference picture.

In MV-HEVC, an inter-view reference picture is marked as long-term.

## Merge mode in HEVC/MV-HEVC

In HEVC, when the current mode is merge, the target reference index is always set to 0. The motion vector in the co-located PU, if referring to a short-term (ST) reference picture, is scaled to form a merge candidate of the current PU (PU0), as shown in Figure 2, wherein MV0 is scaled to MV0’ during the merge mode.

However, if the co-located PU has a motion vector (MV1) referring to an inter-view reference picture, marked as long-term, the motion vector is not used to predict the current PU (PU1).

Figure 2:  TMVP in MV-HEVC (merge mode).

In MV-HEVC, the temporal motion vector prediction process is the same as in HEVC, due to the fact that no modifications in the coding unit or lower level can be introduced.

However, there might be significant amount of co-located PUs (in the co-located picture) which contain motion vectors referring to an inter-view reference picture while the target reference index (being equal to 0) indicates a short-term reference picture. Therefore, disabling prediction from those motion vectors makes the merge mode less efficient.

# Proposal

Figure 3:  Proposed TMVP in MV-HEVC (merge mode).

It is proposed that in the case mentioned above, motion vector in the co-located picture is still considered as available. However, a new additional reference index (ref_idx_add), i.e. the changed reference index, is assigned for that motion candidate. This applies only to the merge mode. As shown in Figure 3, MV1 of the co-located block of PU1 can be used to form a disparity motion vector merging candidate.

In general, when the reference index equal to 0 represents a short-term reference picture, the newly added target reference index is used to represent a long-term reference picture. When the reference index equal to 0 represents a long-term reference picture, the newly added target reference index is used to represent a short-term reference picture.

The contribution proposes the following options for deriving the changed target reference index (ref_idx_add) for the merge mode:

- Setting the changed target reference index equal to 0 during the invocation of the reference picture list construction process.

- Indicating the changed target reference index in the slice segment header.

- Deriving the changed target reference index to be equal to the smallest reference index which has a different marking (as used as short-term or long-term reference) from that of reference index 0. This option was presented in JCTVC-K0239.

- In the case the co-located PU points to a reference picture having a different layer identifier (equal to layerA) than that for reference index 0, deriving the changed target reference index to be equal to the smallest reference index that has layer identifier equal to layerA.

The options are described in details in the next sub-sections.

## Option 1: Setting the additional target reference index equal to 0

No syntax change is associated with this option. The additional target reference index is always set equal to 0 during the invocation of the reference picture list construction process in HEVC version 1. The value can be set to other values in HEVC extensions with high-level syntax changes (e.g. also during the invocation of the reference picture list construction process).

## Option 2: Indicating the additional target reference index in the slice segment header

The newly introduced additional reference index (ref_idx_add_lX, with X equal to 0 or 1) may be always set to 0 in the HEVC and can be set to other values in HEVC extensions with high-level syntax changes.

A flag may be introduced in slice header to indicate the presence of ref_idx_add_lX, when not present, ref_idx_add_lX is inferred to be equal to 0.

## Option 3: Deriving the additional target reference index to be equal to the smallest reference index which has a different marking

In general, when the reference index equal to 0 represents a short-term reference picture, the additional target reference index is used to represent a long-term reference picture. When the reference index equal to 0 represents a long-term reference picture, the additional target reference index is used to represent a short-term reference picture.

Note that similar to the current HEVC specification, if the target reference index corresponds to a long-term reference picture, the motion vector in a co-located block, if considered as available, is not scaled.

## Option 4: Deriving the additional reference index on the basis of layer identifier

It is checked whether the picture used as a reference for the co-located block resides in the same layer (i.e. has the same nuh_reserved_zero_6bits value) as that of the default target picture with reference index 0. If these two pictures are in the same layer, the default target picture is used as the target picture. If these two pictures are in different layers, additional target reference index ref_idx_add is derived as the first picture in the reference picture list having the same layer identifier value as the picture used as a reference for the co-located block. If a picture meeting the mentioned criteria for the additionally target picture is not available, the TMVP candidate is unavailable.

# Proposed spec. text changes in HEVC

The proposed specification text changes are as follows, with newly added/modified text highlighted and deleted text marked as red strikethrough (red strikethrough). The following colors are used for highlighting:

- Common for all options

- Option 1

- Option 2

- Option 3

- Option 4

- 7.3.8.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| add_merge_ref_idx_flag | u(1) |
| if ( add_merge_ref_idx_flag ) { |  |
| ref_idx_add_l0 | ue(v) |
| if( slice_type = = B ) |  |
| ref_idx_add_l1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

7.4.8.1	General slice segment header semantics

add_merge_ref_idx_flag equal to 1 specifies that ref_idx_add_l0 and for B slices, ref_idx_add_l1 are present. add_merge_ref_idx_flag equal to 0 specifies that neither of ref_idx_add_l0 and ref_idx_add_l1 is present. The value of add_merge_ref_idx_flag shall be equal to 0 in this version of this Specification. Although the value of add_merge_ref_idx_flag is required to be equal to 0 in this version of this Specification, decoders shall allow the value 1 to appear in the syntax.

ref_idx_add_lX specifies the additional reference index used as a target reference index to reference picture list X (with X equal to 0 or 1) during the merge mode. When not present, ref_idx_add_lX is inferred to be equal to 0.

- 8.3.4.	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause Error: Reference source not found. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

For option 1:

At the beginning of the decoding process for each slice, the variable RefIdxL0A is set to 0, and for B slices, RefIdxL1A is set to 0, and the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows. Although the value of RefIdxL0A and RefIdxL1A are required to be equal to 0 in this version of this Specification, decoders shall allow others values of the variables in the range of 1 to 15, inclusive.

For option 2:

At the beginning of the decoding process for each slice, the variable RefIdxL0A is set to ref_idx_add_l0, and for B slices, RefIdxL1A is set to ref_idx_add_l1, and the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

For option 3:

This process is invoked when the current slice is a P or B slice. Additional target reference indices RefIdxL0A and RefIdxL1A are derived.

- Set variables RefIdxL0A and RefIdxL1A both to -1.

- The following applies to derive RefIdxL0A.

- bZeroIdxLongTermFlag = RefPicList0[ 0 ] is a short-term reference picture ? 0 : 1
bFound	= 0
for( i = 1; i <= num_ref_idx_l0_active_minus1&&! bFound; i++)
	if ( (bZeroIdxLongTermFlag && RefPicList0[ i ] is a short-term reference picture) | |
		(!bZeroIdxLongTermFlag && RefPicList0[ i ] is a long-term reference picture) ) {
			RefIdxL0A = i
			bFound =1
		}

- When the slice is a B slice, the following applies to derive RefIdxL1A.

- bZeroIdxLongTermFlag = RefPicList1[ 0 ] is a short-term reference picture ? 0 : 1
bFound	= 0
for( i = 1; i <= num_ref_idx_l1_active_minus1&&! bFound; i++)
	if ( (bZeroIdxLongTermFlag && RefPicList1[ i ] is a short-term reference picture) | |
		(!bZeroIdxLongTermFlag && RefPicList1[ i ] is a long-term reference picture) ) {
			RefIdxL1A = i
			bFound =1
		}

For option 4:

This process is invoked when the current slice is a P or B slice. Additional target reference indices RefIdxL0A[ layerId ] and RefIdxL1A[ layerId] are derived for values of layerId in the range of 0 to 63, inclusive.

- Set variables RefIdxL0A[ layerId ] and RefIdxL1A[ layerId ] both to -1 for all values of layerId in the range of 0 to 63, inclusive.

- The following applies to derive RefIdxL0A[ layerId ].

- for( i = 0; i <= num_ref_idx_l0_active_minus1; i++) {
	layerId = nuh_reserved_zero_6bits value for RefPicList0[ i ] 
	if( RefIdxL0A[ layerId ] < 0 ) 
		RefIdxL0A[ layerId ] = i
}

- When the slice is a B slice, the following applies to derive RefIdxL1A.

- for( i = 0; i <= num_ref_idx_l1_active_minus1; i++) {
	layerId = nuh_reserved_zero_6bits value for RefPicList1[ i ] 
	if( RefIdxL1A[ layerId ] < 0 ) 
		RefIdxL1A[ layerId ] = i
}

## Changes in TMVP

- 8.5.3.1.8		Derivation process for collocated motion vectors

Inputs to this process are

- currPb specifying the current prediction block,

- colPic specifying the collocated picture,

- colPb specifying the collocated prediction block inside the collocated picture specified by colPic,

- a luma location ( xPCol, yPCol ) specifying the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic,

- a variable mergeTMVP,

- a reference index refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagLXCol[ x ][ y ], mvLXCol[ x ][ y ] and refIdxLXCol[ x ][ y ] are set equal to the corresponding arrays of the collocated picture specified by colPic, PredFlagLX[ x ][ y ], MvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

  - If predFlagL0Col[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL1Col[ xPCol ][ yPCol ], refIdxL1Col[ xPCol ][ yPCol ], and L1, respectively.

  - Otherwise if predFlagL0Col[ xPCol ][ yPCol ] is equal to 1 and predFlagL1Col[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL0Col[ xPCol ][ yPCol ], refIdxL0Col[ xPCol ][ yPCol ], and L0, respectively.

  - Otherwise (predFlagL0Col[ xPCol ][ yPCol ] is equal to 1 and predFlagL1Col[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If DiffPicOrderCnt( currPic, pic ) is less than or equal to 0 for every picture pic in every reference picture list of the current slice, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xPCol ][ yPCol ], refIdxLXCol[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xPCol ][ yPCol ], refIdxLNCol[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows.

  - When If LongTermRefPic( currPic, currPb, refIdxLX, LX ) is not equal to LongTermRefPic( colPic, colPb, refIdxCol, listCol ), or if LongTermRefPic( currPic, currPb, refIdxLX, LX ) and LongTermRefPic( colPic, colPb, refIdxCol, listCol ) are equal to 1 and nuh_reserved_zero_6bits for the RefPicListX[ refIdxLX ] (being assigned to colRefLayerId) is not equal to nuh_reserved_zero_6bits for the refPicListCol[ refIdxCol ], the following applies.

      - If refIdxLXA[ colRefLayerId ] is greater than or equal to 0, and mergeTMVP is equal to 1, refIdxLX is set to RefIdxLXA[ colRefLayerId ] and availableFlagLXCol is set to 1.

      - Otherwise, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise, When the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing prediction block currPb in the picture colPic, and the following applies.

colPocDiff = DiffPicOrderCnt( colPic, refPicListCol[ refIdxCol ] )	(8-1)

currPocDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(8-2)

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or colPocDiff is equal to currPocDiff, mvLXCol is derived as:

mvLXCol = mvCol	(8-3)

      - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below.

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(8-4)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-5)

mvLXCol =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCol ) * 		
		( ( Abs( distScaleFactor * mvCol ) + 127 ) >> 8 ) )	(8-6)

where td and tb are derived as

td = Clip3( −128, 127, colPocDiff )	(8-7)

tb = Clip3( −128, 127, currPocDiff )	(8-8)

## TMVP Invocations

Changes in this sub-section are for the invocations of 8.5.3.1.8.

### Adding a new input for 8.5.3.1.7

- 8.5.3.1.7		Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable mergeTMVP,

- a reference index refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPb specifies the current luma prediction block at luma location ( xP, yP ).

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If slice_temporal_mvp_enable_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply.

- Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

- The bottom right collocated motion vector is derived as follows

xPRb = xP + nPbW                                                      (8-9)

yPRb = yP + nPbH                                                       (8-10)

- If ( yP >> Log2CtbSizeY ) is equal to ( yPRb >> Log2CtbSizeY ), and xPRb is less than pic_width_in_luma_samples, the following applies.

- The variable colPb specifies the luma prediction block covering the modified location given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the collocated picture specified by colPic.

- The luma location ( xPCol, yPCol ) is set equal to the top-left sample of the of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic.

- The derivation process for collocated motion vectors as specified in subclause Error: Reference source not found is invoked with currPb, colPic, colPb, ( xPCol, yPCol ), mergeTMVP, and refIdxLX as inputs and the output being assigned to mvLXCol and availableFlagLXCol.

- Otherwise, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- When availableFlagLXCol is equal to 0, the central collocated motion vector is derived as follows.

xPCtr = xP + ( nPbW >> 1 )                                             (8-11)

yPCtr = yP + ( nPbH >> 1 )                                              (8-12)

- The variable colPb specifies the luma prediction block covering the modified location given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- The luma location ( xPCol, yPCol ) is set equal to the top-left sample of the of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic.

- The derivation process for collocated motion vectors as specified in subclause Error: Reference source not found is invoked with currPb, colPic, colPb, ( xPCol, yPCol ), mergeTMVP, and refIdxLX as inputs and the output being assigned to mvLXCol and availableFlagLXCol.

### Invocation of 8.5.3.1.7 for merge

- 8.5.3.1.1		Derivation process for luma motion vectors for merge mode

This process is only invoked when CuPredMode[ xC ][ yC ] is equal to MODE_SKIP or CuPredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- …

- ...

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, mergeTMVP equal to 1, and refIdxLXCol as the inputs and the output being the availability flags availableFlagLXCol and the temporal motion vectors mvLXCol (with X being 0 or 1, respectively). The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-13)

predFlagLXCol = availableFlagLXCol	(8-14)

…

### Invocation of 8.5.3.1.7 for AMVP

- 8.5.3.1.5		Derivation process for luma motion vector prediction

Inputs to this process are

…

The motion vector predictor mvpLX is derived in the following ordered steps.

- …

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0, otherwise, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, mergeTMVP equal to 0, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

…

# Information: proposed spec. text changes in MV-HEVC:

Changes in this MV-HEVC to support the proposed technology are given in this section as an example. The changes in this section are only needed when the option 2 in HEVC is utilized to support the motion hook. Similar syntax changes may be applicable to 3D-HEVC as well. The text changes in MV-HEVC are highlighted in pink.

- 7.3.8.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| ref_idx_add_l0 | ue(v) |
| if( slice_type = = B ) |  |
| ref_idx_add_l1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

- 7.4.8.1	General slice segment header semantics

ref_idx_add_lX specifies the additional reference index used as a target reference index to reference picture list X (with X equal to 0 or 1) during the merge mode. When not present, ref_idx_add_lX is inferred to be equal to 0.

- 8.3.4.	Decoding process for reference picture lists construction

…

At the beginning of the decoding process for each slice, the variable RefIdxL0A is set to ref_idx_add_l0, and for B slices, RefIdxL1A is set to ref_idx_add_l1, and the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

# Simulation results

This section provides simulation results of the proposed method based on the 3DV-HTM software, under the JCT-3V common test conditions for coding texture views.

The coding gain of the proposed method compared to the anchor for texture views coding in 3-veiw case is reported in this section. Table 1 shows the average coding gain of the proposed method with respect to HTM5.0 anchor. The column denoted by ‘Video only’ lists the coding gain of all three texture views where the bitrates represent the total bitrates of bitstreams containing three texture views and the PSNR values are the average PSNR values of the three decoded texture views. Bitrate savings for “Video 1” and “Video 2” are also listed in this table. Note that “Video 0” corresponds to the base view, while “Video 1” and “Video 2” indicate the non-base views (texture only).

As shown in Table 1, the overall average bitrate saving is around 0.94% for all texture views and the saving of the non-base views is about 2.5%.

Table 1: Coding gain with respect to MV-HEVC anchor

|  | Video 0 | Video 1 | Video 2 | Video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.16% | -2.93% | -1.40% |
| Kendo | 0.0% | -2.50% | -2.22% | -1.03% |
| Newspapercc | 0.0% | -1.52% | -1.20% | -0.59% |
| GhostTownFly | 0.0% | -5.70% | -5.49% | -1.55% |
| PoznanHall2 | 0.0% | -1.92% | -1.03% | -0.67% |
| PoznanStreet | 0.0% | -2.14% | -2.06% | -0.71% |
| UndoDancer | 0.0% | -1.89% | -2.00% | -0.69% |
| 1024x768 | 0.0% | -2.39% | -2.12% | -1.01% |
| 1920x1088 | 0.0% | -2.91% | -2.64% | -0.91% |
| average | 0.0% | -2.69% | -2.42% | -0.95% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[End of the copied content of the JCTVC-L0257]