| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0065 |
| --- | --- |

| Title: | CE5.h : Inter-view SAO process in 3DV coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taesup Kim Jin Heo Moonmo Koo Sehoon Yea #221, Yangjae-dong, Seocho-gu, Seoul, 137-130, Korea | Tel: Email: | +82-10-2440-6393 taesup.kim@lge.com jin78.heo@lge.com moonmo.koo@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

- In this contribution, SAO process in the 3D HEVC (HTM 5.0.1) is migrated to that in the current HEVC (HM 8.2). The main modification is that SAO parameters are derived from LCU-based optimization (interleaving mode) rather than picture-based optimization (APS mode). Moreover, some other modifications are included to increase the performance of SAO process. Furthermore, inter-view SAO process is proposed and modeled on the top of it to confirm the benefit of sharing base view’s SAO parameters on the dependent views, and reportedly shows 1.0% and 0.8% gains on view 1 and 2, respectively.

# Introduction

In the current Common Test Condition (CTC) of 3D HEVC (HTM 5.0.1) [1], SAO process is designated to implement on APS mode that derives SAO parameters from picture-based optimization with quad-tree structure in the picture. Moreover, SAO parameters are transmitted on the APS, as a set of SAO parameters is assigned to each partition of quad-tree structure in the picture. Otherwise, SAO process has an alternative approach with LCU-based optimization, which is called interleaving mode, that derives SAO parameters from each LCU and transmits them on the slice.

LCU-based optimization approach is, however, the current CTC of HEVC (HM 8.2) [2] with a couple of modifications. Therefore, it is an important experiment to confirm the benefit and compatibility of inter-view SAO process [3] also in the current HEVC, and it was also noted on the mandates of Core Experiment 5 of the 2nd JCT-3V meeting [4].

In this contribution, a couple of modifications on SAO process due to the migration to the current HEVC are listed and the benefit of inter-view SAO process is also confirmed on it.

# SAO Process on HEVC (HM 8.2)

As APS is removed from the current HEVC, SAO process is implemented by using LCU-based optimization that derives and transmits SAO parameters on the LCU level. It is based on the interleaving mode of the current 3D HEVC. Moreover, there are some additional modifications that improve the performance.

- JCTVC-J0044 : SAO process is turned on/off on a frame level based on a statistical criterion of the previous frame. The same criterion is used but applied separately for luma and chroma.

- JCTVC-J0355 : SAO process uses a single merge left flag to signal whether all color components are merge left, and to use a single merge up flag to signal whether all color components are merge up.

- JCTVC-H0273 : In order to avoid storing all pixels of one LCU row, the last four pixel rows in each LCU are not used during SAO parameter estimation because they are not yet processed by deblocking until the lower LCU comes in practical LCU-based encoding systems.

# Inter-view SAO Process

Inter-view SAO process was proposed during the 2nd JCT-3V meeting with an assumption that SAO parameters could be shared between neighbored views without any additional process. As depicted on Figure 1, SAO parameters are only derived and transmitted from the base view (view 0), and all views including dependent views (view 1, 2) share the base view’s derived SAO parameters. On dependent views, each LCU reuses SAO parameters of the collocated LCU on the base view.

Figure 1. SAO parameters are only derived from each LCU of the base view and co-located LCUs of dependent views just reuse them for SAO process.

# Simulation Results

## SAO Process on HEVC (HM 8.2)

### Compression Performance

In this contribution, only SAO process of the current 3D HEVC (HTM 5.0.1) is replaced by that of the current HEVC (HM 8.2). The coding performance of it is provided by comparing it with the anchor of current 3D HEVC (HTM 5.0.1). The result is shown in Table 2.

Table 2. Rate-distortion results on 3-view coding with depth of ‘Migration of SAO process of the current 3D HEVC (HTM 5.0.1) to the current HEVC (HM 8.2)’

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.9% | 0.3% | 0.5% | 0.7% | 0.6% | 0.7% |
| Kendo | 0.9% | 0.0% | 0.6% | 0.7% | 0.4% | 0.6% |
| Newspapercc | 0.2% | -0.3% | -0.1% | 0.1% | 0.0% | 0.0% |
| GhostTownFly | 0.3% | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% |
| PoznanHall2 | 0.4% | 0.8% | -0.2% | 0.4% | 0.3% | 0.3% |
| PoznanStreet | 0.1% | -0.3% | -0.2% | 0.2% | -0.1% | 0.0% |
| UndoDancer | 0.1% | -0.1% | -0.3% | 0.0% | 0.2% | 0.1% |
| 1024x768 | 0.7% | 0.0% | 0.3% | 0.5% | 0.4% | 0.4% |
| 1920x1088 | 0.2% | 0.1% | -0.2% | 0.2% | 0.1% | 0.1% |
| average | 0.4% | 0.1% | 0.0% | 0.3% | 0.2% | 0.3% |

### Complexity Analysis

Table 3 shows the ratios of encoding, decoding and rendering time of the proposal in comparison with the anchor. The results indicate no evident change on complexity compared to the anchor’s running time.

Table 3. Running time ratio of ‘Migration of SAO process of the current 3D HEVC (HTM 5.0.1) to the current HEVC (HM 8.2)’

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 100.2% | 111.8% | 101.8% |
| Kendo | 100.3% | 115.9% | 99.0% |
| Newspapercc | 100.1% | 121.0% | 95.9% |
| GhostTownFly | 100.0% | 109.5% | 100.0% |
| PoznanHall2 | 100.2% | 105.3% | 101.1% |
| PoznanStreet | 99.9% | 109.9% | 100.6% |
| UndoDancer | 100.0% | 116.1% | 101.1% |
| 1024x768 | 100.2% | 116.2% | 98.9% |
| 1920x1088 | 100.0% | 110.1% | 100.7% |
| Average | 100.1% | 112.7% | 99.9% |

## Inter-view SAO Process

### Compression Performance

Inter-view SAO process was implemented and evaluated on the top of SAO process of the current HEVC. The coding performance of it is provided by comparing it with the anchor of current 3D HEVC along with SAO process migration to the current HEVC. The result is shown in Table 4. Consequently, it is confirmed that the benefit of inter-view SAO process is still effective in SAO process in the current HEVC. All dependent views appeared with consistent gains on all sequences, and the average gains were 1.0% and 0.8% on video 1, 2, respectively. Moreover, 0.1% gains were reported for overall videos.

Table 4. Rate-distortion results on 3-view coding with depth of ‘Inter-view SAO process on the top of migrated SAO process’

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -0.7% | -0.2% | -0.3% | -0.2% |
| Kendo | 0.0% | -0.8% | -0.8% | -0.3% | -0.3% | -0.2% |
| Newspapercc | 0.0% | -0.8% | -0.4% | -0.1% | -0.2% | -0.1% |
| GhostTownFly | 0.0% | -1.6% | -1.4% | -0.2% | 0.1% | 0.0% |
| PoznanHall2 | 0.0% | -1.3% | -1.2% | -0.3% | -0.4% | -0.4% |
| PoznanStreet | 0.0% | -0.6% | -0.1% | 0.1% | 0.1% | 0.1% |
| UndoDancer | 0.0% | -0.8% | -0.7% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | -0.8% | -0.7% | -0.2% | -0.2% | -0.2% |
| 1920x1088 | 0.0% | -1.1% | -0.8% | -0.1% | -0.1% | -0.1% |
| average | 0.0% | -1.0% | -0.8% | -0.1% | -0.1% | -0.1% |

### Complexity Analysis

Table 5 shows the ratios of encoding, decoding and rendering time of the proposal in comparison with the anchor. The results shows no additional complexity for inter-view SAO.

Table 5. Running time ratio of ‘inter-view SAO process on the top of migrated SAO process’

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 100.0% | 102.1% | 99.3% |
| Kendo | 100.0% | 99.0% | 100.4% |
| Newspapercc | 100.0% | 99.2% | 101.6% |
| GhostTownFly | 99.9% | 99.5% | 102.1% |
| PoznanHall2 | 100.3% | 99.0% | 101.2% |
| PoznanStreet | 100.3% | 97.8% | 105.6% |
| UndoDancer | 100.0% | 98.9% | 103.3% |
| 1024x768 | 100.0% | 100.1% | 100.4% |
| 1920x1088 | 100.1% | 98.8% | 103.0% |
| Average | 100.1% | 99.3% | 101.9% |

# Conclusion

In this contribution, it is reported that the benefit of inter-view SAO process is still effective in the upcoming version of SAO process, which is CTC of the current HEVC.

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1005, Oct. 2012

[2] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 8”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1003, Jul. 2012

[3] T. Kim, J. Heo, M. Koo, S. Yea, “Inter-view SAO process in 3DV coding”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1105, Oct. 2012

[4] Y.-L. Chang, S. Yea, “Description of Core Experiment 5 (CE5) on Inter-view/motion prediction”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1105, Oct. 2012

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

### 7.3.8 Slice segment header syntax

### 7.3.8.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| for ( i = 0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_undetermined_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| if(ViewId==0) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) [Ed. (GJS): Does the logic of this condition check make sense when the slice is not a B slice?] |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_disable_deblocking_filter_flag | u(1) |
| if( !slice_disable_deblocking_filter_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

### 7.3.9 Slice segment data syntax

- 7.3.9.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRS % PicWidthInCtbsY ) << Log2CtbSizeY |  |
| yCtb = ( CtbAddrInRS / PicWidthInCtbsY ) << Log2CtbSizeY |  |
| CtbAddrInSliceSeg = CtbAddrInRS − slice_segment_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) { |  |
| if (ViewIdx == 0) { |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| } |  |
| } |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

### 8.7.3 Sample adaptive offset process

#### 8.7.3.1 General

Inputs of this process are the reconstructed picture sample arrays prior to sample adaptive offset recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture sample arrays after sample adaptive offset saoPictureL, saoPictureCb and saoPictureCr.

This process is performed on a coding tree block basis after the completion of the deblocking filter process for the decoded picture.

The sample values in the modified reconstructed picture sample arrays after sample adaptive offset saoPictureL, saoPictureCb and saoPictureCr are initially set equal to the sample values in the reconstructed picture sample arrays prior to sample adaptive offset recPictureL, recPictureCb and recPictureCr.

If ViewId is not equal to 0, the following applies:

- slice_sao_luma_flag and slice_sao_chroma_flag are resued.

- All sample adaptive offset syntaxs are reused.

For every coding tree unit with coding tree block location ( rx, ry ), where rx = 0..PicWidthInCtbsY − 1 and ry = 0..PicHeightInCtbsY − 1, the following applies:

–	When slice_sao_luma_flag of the current slice is equal to 1, the coding tree block modification process as specified in subclause Error: Reference source not found is invoked with recPicture set equal to recPictureL, cIdx set equal to 0, ( rx, ry ) and nS set equal to ( 1 << Log2CtbSizeY ) as inputs and the modified luma picture sample array saoPictureL as output.

–	When slice_sao_chroma_flag of the current slice is equal to 1, the coding tree block modification process as specified in subclause Error: Reference source not found is invoked with recPicture set equal to recPictureCb, cIdx set equal to 1, ( rx, ry ) and nS set equal to ( 1 << (Log2CtbSizeY − 1) ) as inputs and the modified chroma picture sample array saoPictureCb as output.

–	When slice_sao_chroma_flag of the current slice is equal to 1, the coding tree block modification process as specified in subclause Error: Reference source not found is invoked with recPicture set equal to recPictureCr, cIdx set equal to 2, ( rx, ry ) and nS set equal to ( 1 << (Log2CtbSizeY − 1) ) as inputs and the modified chroma picture sample array saoPictureCr as output.