| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: Geneva, CN, 17–23 Jan 2013 | Document: JCT3V-C0067 |
| --- | --- |

| Title: | 3D-CE6.h: Improved Simplified Depth Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo, Sehoon Yea, | Email: | jin78.heo@lge.com, sehoon.yea@lge.com, |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents a depth intra coding method for Simplified Depth Coding (SDC). First, the proposed method substitutes the intra-predicted Wedgelet partitioning method of DMM mode 2 described in JCT3V-A1005 with the region boundary chain coding method described in JCT3V-A0070. Second, the proposed method uses the concatenation of basic binarization schemes for residual index coding. Consequently, this proposal uses three context models to code residual index. Third, since this contribution uses only one kind of context model for SDC residual flag and SDC residual sign flag, it can reduce the number of context models. Under both common test conditions and all-intra configuration this method provides 0.2% BD-rate gain with no impact on the encoder/decoder complexity.

# Introduction

The Simplified Depth Coding (SDC) described in JCT3V-B0036 [1] adds an alternative coding path for each CU and is restricted to a set of four intra prediction modes; DC, DMM1, DMM2, and planar. However DMM1 and DMM2 are both based on the same approach for predicting a depth block by partitioning it into two disjoint regions separated by a straght line. Therefore, this contribution uses a region boundary chain coding proposed in JCT3V-A0070 [2] instead of DMM2 to describe an arbitrary shape of the partition. Moreover, the SDC uses a fixed length binarization to code the residual index generated by a depth lookup table (LUT). In this contribution, a concatenation of the unary binarization for prefix part and the fixed length binarization for suffix part is applied to signal the residual index. Last, this contribtuion uses only one kind of context model to code residual flag and residual sign flag regardless of the segment.

# Proposed Method

## Region Boundary Chain Coding

The proposed method uses the region boundary chain coding in substitute for DMM2 among four prediction modes in SDC. The region boundary chain coding also applies the prediction process for separating the depth block into two constant-valued regions. However, since the method uses several traverse types to separate the depth block, it can handle an arbitrary shape of the partition. Therefore it can well describe even a curved depth pattern, compared with DMM2. Moreover, this proposal changes the SDC prediction mode order; Planar, DMM1, CHAIN, and DC.

## Concatenate Binarization

Instead of coding a residual depth value directly, it is mapped to its corresponding residual index in a depth lookup table which is constructed by analyzing a certain number of frames of the input sequence. The calculated residual index is coded with a sign flag and a magnitude of the residual index using the fixed length (FL) binarization without transform and quantization process.

Typically FL binarization is applied to symbol with a nearly uniform distribution. However, since the statistical distribution of the residual index is highly skewed on small residual index, as shown in Figure 1, it is not suitable for coding the residual index. Considering this observation, this contribution uses the unary code to generate the binary sequence of the residual index. Consequently, this proposal assigns shorter codeword to a small residual index that occurs more frequently instead of using uniformly assigned n bits for all residual indexes. Therefore, the unary binarization generates smaller number of bins to encode by the entropy coder than the FL binarization.

Figure 1 Statistical distribution of residual index, QP=25

Table 1 shows the comparison of the number of bins for FL binarization and unary binarization when QP is equal to 25 and 40.

Table 1 Comparison of the number of bins (All intra 25 frames)

Since the probability of bin ‘0’ and ‘1’ in the binary sequence generated by FL binarization has random characteristics, several adequately designed sets of context models for each position are used. As a result, 20 context models are employed to code bin at each position. However, it is easy to expect the bin value in unary binarization because most of bins except for the last bin have the same bin value. Therefore, only one kind of context model for residual index is used in this proposal.

Table 2 Comparison of the number of context models for residual index

As shown in Figure 1, the statistical distribution of some sequences; Newspaper, GT_Fly, and Poznan_Street have the different statistical distribution of unary binarization. The statistical distribution of them is quite wide; note the large variation and wide tails. In order to shove this problem, this contribution proposes a concatenation of basic binarization schemes; unary binarization and FL binarization. For residual index binarization, the concatenate binarization is constructed by the prefix part of the unary code and the suffix part of the FL code.

## Modifications of Context Model for SDC Residual Flag and SDC Residual Sign Flag

In the current SDC, different context models are used to code SDC residual flag and SDC residual sign flag in each segment. However, the proposed method uses only one context model to code both flags regardless of segment region.

Table 3 Comparison of the number of context models for residual flag and residual sign flag

# Experimental Results

According to the CE6.H description [3] and the common test conditions [4], three tests are conducted under random access (CTC) and intra-only configuration. First test is the result of region boundary chain coding for DMM2 as presented in Section 2.1, and second test is the result of the combination of concatenate binarization as presented in Section 2.2 and modification of context model as presented in Section 2.3. The last test is the result of the combination for Test 1 and Test 2.

## Test 1: results of region boundary chain coding for DMM2

Table 4 Result for 3-view scenario under CTC

Table 5 Result for 3-view scenario under intra-only configuration

## Test 2: results of concatenate binarization

Table 6 Result for 3-view scenario under CTC

Table 7 Result for 3-view scenario under intra-only configuration

## Test 3: results of the combination for Test 1 and Test 2

Table 8 Result for 3-view scenario under CTC

Table 9 Result for 3-view scenario under intra-only configuration

# Working Draft Modifications

## Modifications to Coding Unit

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I && !MotionInhFlag[ x0 ][ y0 ]) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if( !MotionInhFlag[ x0 ][ y0 ] ) { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( sdc_pred_mode = = 2) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign[ x0 ][ y0 ] | ae(v) |
| edge_start_left_flag | ae(v) |
| edge_start_position | ae(v) |
| edge_count_minus1 | ae(v) |
| for( k = 0; k <= edge_count_minus1; k++) |  |
| edge_code[k] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( enable_DMM_flag && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( resPredEnableFlag ) |  |
| res_pred_flag | ae(v) |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if ( MotionInhFlag[ x0 ][ y0 ] && TextureCtDepth[ x0 ][ y0 ] > ctDepth) { |  |
| coding_tree( x0, y0, log2CbSize, ctDepth ) |  |
| } else { |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ¦ ¦ sdc_pred_mode = = 2 ) ? 2 : 1 ) |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_prefix[ x0 ][ y0 ][ i ] | ae(v) |
| if ( prefix_count = = PrefixThreshold ) |  |
| sdc_residual_suffix[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Table G-1 – Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_DepthPartition( 41 ) |
| 3 | Intra_Planar |
| 0 | Intra_Planar |
| 1 | MODE_DMM_WFULL( 35 ) |
| 2 | MODE_CHAIN( 43 ) |
| 3 | Intra_DC |

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_prefix[ x0 ][ y0 ][ i] and sdc_residual_suffix[ x0 ][ y0 ][ i ] specify the value of a residual index.

sdc_residual_sign_flag[ x0 ][ y0 ][ i ], sdc_residual_prefix[ x0 ][ y0 ][ i ], and sdc_residual_suffix[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

The parsing process for these syntax elements begins with reading the bits starting at the current location in the bitstream up to PrefixThreshold and including the first zero bit, and counting the number of leading bits (prefix_count) that are equal to 1.

PrefixThreshold = ( (num_depth_values_in_dlt * 3) >> 2 )

If prefix_count is not equal to PrefixThreshold, the parsing process for SdcResidual[ x0 ][ y0 ][ i ] terminates. Otherwise, the parsing process for sdc_residual_suffix[ x0 ][ y0 ][ i ] begins with reading the bits starting at the current location in the bitstream up to BitsPerSuffix and suffix_value is derived.

BitsPerSuffix = ceil( Log2( num_depth_values_in_dlt – prefixThreshold) )

–	If prefix_count is not equal to PrefixThreshold

SdcResidual[ x0 ][ y0 ][ i ] = ( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( prefix_count + 1 )

–	Otherwiese prefix_count is equal to PrefixThreshold

SdcResidual[ x0 ][ y0 ][ i ] = ( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( prefix_count + suffix_value + 1 )

# Decoding Process

## Decoding Process for Simplified Depth Coding

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is:

- a variable IntraPredMode[ xB ][ yB ] specifying the prediction mode for the current prediction block

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

If sdc_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

- If SdcPredMode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1

- If SdcPredMode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 0

- If SdcPredMode[ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 35

- If SdcPredMode[ xB ][ yB ] is equal to 3, IntraPredMode[ xB ][ yB ] is set equal to 41

- If SdcPredMode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 0

- If SdcPredMode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35

- If SdcPredMode[ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 43

- If SdcPredMode[ xB ][ yB ] is equal to 3, IntraPredMode[ xB ][ yB ] is set equal to 1

### Depth value reconstruction process

Inputs to this process are:

- a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nT specifying the prediction size

- predicted samples predSamples[ x ][y ], with x, y =0..nT−1

- a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as per the following ordered steps.

  - Depending on intraPredMode the array wedgePattern[ x ][ y ] and SDCPattern[ x ][ y ] with x, y =0..nT−1 specifying the binary segmentation pattern is derived as follows.

    - If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

        - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

        - SDCPattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

    - Otherwise, if intraPredMode is equal to Intra_DepthPartition( 41 ), the following applies.

      - [Ed. (GT): Specify derivation for continuous intra direction wedgelet partition.]

    - If intraPredMode is equal to Intra_Chain( 43 ), the following applies.

- SDCPattern = edgePattern[ x ][ y ]

    - Otherwise ( intraPredMode is not equal to Intra_DepthPartition( 35 ) and  intraPredMode is not equal to Intra_DepthPartition( 41 ) Intra_Chain( 43 )), the following applies.

      - For x, y = 0..nT−1 wedgePattern[ x ][ y ] SDCPattern[ x ][ y ] is set equal to 0.

  - For x, y = 0..nT−1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following.

    - If dlt_flag is equal to 0, the following applies.

      - resSamples[ x ][ y ] = predSamples[ x ][ y ] + SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]

      - resSamples[ x ][ y ] = predSamples[ x ][ y ] + SdcResidual[ xB ][ yB ][ SDCPattern[ x ][ y ] ]

    - Otherwise ( dlt_flag is equal to 1 ), the following applies.

        - dltIdxPred = DepthValue2Idx[ predSamples[ x ][ y ] ] 	(G-1)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(G-2)
dltIdxResi = SdcResidual[ xB ][ yB ][ SDCPattern[ x ][ y ] ]	(G-3)
resSamples[ x ][ y ] = Idx2DepthValue[ dltIdxPred + dltIdxResi ]	(G-4)

### Initialization process for context variables

Table G-2 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| coding_unit() | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
|  | sdc_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Table G-3 | 0..1 0 | 2..3 1 | 4..5 2 |
|  | sdc_residual_sign_flag | Table G-4 | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Table G-5 | 0..19 | 20..39 | 40..59 |
|  | sdc_residual | Table G-5 | 0 | 1 | 2 |
|  | sdc_pred_mode | Error: Reference source not found | 0..2 | 3..5 | 6..8 |

Table G-3 – Values of variable initValue for sdc_residual_flag ctxIdx

| Initialization variable | sdc_residual_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 251 154 | 255 154 | 0 154 | 0 | 56 | 0 |

Table G-4 – Values of variable initValue for sdc_residual_sign_flag ctxIdx

| Initialization variable | sdc_residual_sign_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 255 154 | 255 154 | 0 154 | 252 | 55 | 0 |

Table G-5 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

Table G-68 – Values of variable initValue for sdc_residual ctxIdx

| Initialization variable | sdc_residual |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 155 | 155 | 155 |

# Conclusion

This contribution proposes an improved depth intra coding scheme for Simplified Depth Coding (SDC) using two methods; region boundary chain coding instead of intra-predicted Wedgelet partitioning method of DMM mode 2 and concatenate binarization. The improved scheme achieves 0.2% and 0.2% gains under common test conditions and all-intra configuration, respectively.

# Reference

- Fabian Jäger, “3D-CE6.h: Simplified Depth Coding with an optional Depth Lookup Table,” JCT3V-B0036, Shanghai, CN, Oct. 2012.

- Jin Heo, Eunyong Son, Sehoon Yea, “CE6.h Region Boundary Chain Coding for Depth-map,” JCT3V-A0070, Stockholm, SE, July 2012.

- Dmytro Rusanovskyy, Karsten Müller, and Anthony Vetro, “Common Test Conditions for 3DV Core Experiments,” JCT3V-A1100, Stockholm, Sweden, July 2012.

- Philipp Merkle, " Description of Core Experiment 6 (CE6) on Depth Intra Coding", JCT3V-A1106, Stockholm, July 2012.

# Patent rights declaration(s)

LG Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).