| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0078 |
| --- | --- |

| Title: | - AHG13: On disparity vector constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JCT3V-B0037. Coding efficiency impact of disparity vector constraint is studied for input video having vertical offset as AHG13 activity. Non-rectified MVC test sequences and CTC sequences with artificial vertical offset are studied. Both MV-HEVC and 3D-HEVC condition are studied using AHG13 software.

It is reported that the BD-rate difference is 0.0% in total when the offset is smaller than the constraint. On the other hand, the difference is more than about 40% in total when the offset is larger than the constraint.

The proposal is to define two profiles as MVC to satisfy the various requirements. One is namely Stereo-profile and the other is Multi-view profile, where the disparity vector constraints are mandated in the former but not mandated in the latter. Thus, it is asserted that low delay feature is realized in Stereo profile and high flexibility to the input is ensured in Multi-view profile.

Regarding the constraint value, 56 [pixel] is proposed since the significant coding loss is not observed in non-rectified MVC sequences while one LCU line delay functionality is achieved for the maximum LCU size.

Finally, to clarify the constraint, it is proposed to add a 1-bit sequence level flag in SPS extension. It is asserted that the flag is useful to transcode bitstream between the profiles.

# Introduction

This contribution is a follow-up of JCT3V-B0037.

JCT3V-B_Notes_d7 says, regarding JCT3V-B0037,

In real applications, HE-MVC might need to encode video from non-rectified cameras – further study necessary what amount of vertical restrictions would be useful. We would not intend to define an HE-MVC profile only for rectified cameras.

Thus, coding efficiency impact of disparity vector constraint is studied for input video having vertical offset as AHG13 activity.

AHG13 software was released in JCT-3V reflector on 13 Dec. 2012. It is based on HTM5.1, and has a functionality to shift input image of dependent view in a vertical direction before encode. Using the software, common test condition sequences with artificial vertical offset are studied. Also, non-rectified or large vertical offset MVC test sequences are studied. The experimental results are reportedly shown in Section 2 for both MV-HEVC and 3D-HEVC conditions.

# Experimental results

The results with the following setting on AHG13 software are reported.

(Note: the results are same as what were announced in JCT-3V reflector on 18 and 20 Dec. 2012. They were cross-checked by Sharp, Mediatek and Sony.)

- Sequence setting

  - CTC with artificial offset

In the configuration file,

VerticalShiftValueinPixel 	:   X		( X= 0, 16, 32 or 64 )

- MVC sequence (See Figure2.1 for the camera position)

  - Flamenco2

    - View0 (Center), View1 (Left), View3 (Right)

    - View2 (Center), View1 (Left), View3 (Right)

  - Akko&Kayo

    - View48 (Center), View46 (Left), View50 (Right)

    - View48 (Center), View26 (Left), View70 (Right)

  - Breakdancers

    - View4 (Center), View1 (Left), View7 (Right)

Figure 2.1 Camera position for MVC sequence

- Macro setting

  - MV-HEVC condition

#define QC_MVHEVC_B0046                   1

- 3D-HEVC condition

#define QC_MVHEVC_B0046                   0

- Configuration setting

  - Anchor

DisparitySearchRangeRestriction  : 0

- Tested

DisparitySearchRangeRestriction  : 1

VerticalDisparitySearchRange    : 56

## MV-HEVC condition result

Table2.1.1 CTC with VerticalShiftValueinPixel = 0

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% |
| Kendo | 0.0% | 0.1% | -0.1% | 0.0% |
| Newspapercc | 0.0% | 0.1% | -0.1% | 0.0% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | 0.2% | -0.2% | 0.0% |
| PoznanStreet | 0.0% | 0.1% | 0.1% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% |
| average | 0.0% | 0.1% | 0.0% | 0.0% |

Table2.1.2 CTC with VerticalShiftValueinPixel = 16

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.1% |
| Newspapercc | 0.0% | -0.1% | 0.0% | 0.0% |
| GhostTownFly | 0.0% | -0.2% | 0.1% | 0.0% |
| PoznanHall2 | 0.0% | 0.3% | 0.1% | 0.1% |
| PoznanStreet | 0.0% | -0.2% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | -0.1% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% |

Table2.1.3 CTC with VerticalShiftValueinPixel = 32

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% |
| Newspapercc | 0.0% | 0.1% | -0.1% | 0.0% |
| GhostTownFly | 0.0% | 0.2% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | -0.1% | -0.3% | -0.1% |
| PoznanStreet | 0.0% | 0.1% | 0.2% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% |

Table2.1.4 CTC with VerticalShiftValueinPixel = 64

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 22.1% | 95.5% | 26.8% |
| Kendo | 0.0% | 61.8% | 85.4% | 33.6% |
| Newspapercc | 0.0% | 28.6% | 121.5% | 32.1% |
| GhostTownFly | 0.0% | 97.1% | 411.7% | 63.7% |
| PoznanHall2 | 0.0% | 27.1% | 91.6% | 27.3% |
| PoznanStreet | 0.0% | 38.4% | 234.9% | 43.9% |
| UndoDancer | 0.0% | 35.5% | 323.5% | 52.2% |
| 1024x768 | 0.0% | 37.5% | 100.8% | 30.8% |
| 1920x1088 | 0.0% | 49.5% | 265.4% | 46.8% |
| average | 0.0% | 44.4% | 194.9% | 39.9% |

Table2.1.5 MVC sequence

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate |
| --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | -0.1% | -0.1% | 0.0% |
| Flamenco2_2_1_3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Akko&Kayo_48_46_50 | 0.0% | 0.0% | 0.0% | 0.0% |
| Akko&Kayo_48_26_70 | 0.0% | 18.1% | 27.0% | 11.1% |
| Breakdancers_4_1_7 | 0.0% | -0.1% | -0.1% | -0.1% |
| average | 0.0% | 3.6% | 5.4% | 2.2% |

## 3D-HEVC condition result

Table2.2.1 CTC with VerticalShiftValueinPixel = 0

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.0% |
| Kendo | 0.0% | -0.1% | 0.0% | 0.0% |
| Newspapercc | 0.0% | 0.0% | 0.1% | 0.0% |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 0.0% |
| PoznanHall2 | 0.0% | 0.1% | -0.6% | -0.1% |
| PoznanStreet | 0.0% | 0.2% | -0.1% | 0.0% |
| UndoDancer | 0.0% | -0.1% | 0.1% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | -0.2% | 0.0% |
| average | 0.0% | 0.0% | -0.1% | 0.0% |

Table2.2.2 CTC with VerticalShiftValueinPixel = 16

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% |
| Kendo | 0.0% | -0.2% | 0.1% | 0.0% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% |
| GhostTownFly | 0.0% | 0.1% | 0.1% | 0.0% |
| PoznanHall2 | 0.0% | 0.0% | -0.1% | 0.0% |
| PoznanStreet | 0.0% | -0.2% | -0.4% | -0.1% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% |
| average | 0.0% | 0.0% | -0.1% | 0.0% |

Table2.2.3 CTC with VerticalShiftValueinPixel = 32

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% |
| Kendo | 0.0% | -0.1% | 0.1% | 0.0% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% |
| GhostTownFly | 0.0% | -0.1% | -0.2% | -0.1% |
| PoznanHall2 | 0.0% | 0.0% | -0.2% | 0.0% |
| PoznanStreet | 0.0% | 0.0% | -0.1% | 0.0% |
| UndoDancer | 0.0% | -0.2% | -0.1% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | -0.1% | -0.2% | 0.0% |
| average | 0.0% | -0.1% | -0.1% | 0.0% |

Table2.2.4 CTC with VerticalShiftValueinPixel = 64

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 139.5% | 128.5% | 53.9% |
| Kendo | 0.0% | 116.0% | 108.8% | 47.4% |
| Newspapercc | 0.0% | 143.6% | 142.0% | 56.3% |
| GhostTownFly | 0.0% | 406.7% | 437.8% | 106.4% |
| PoznanHall2 | 0.0% | 85.6% | 104.0% | 42.2% |
| PoznanStreet | 0.0% | 224.5% | 248.7% | 76.1% |
| UndoDancer | 0.0% | 295.5% | 351.2% | 96.1% |
| 1024x768 | 0.0% | 133.0% | 126.4% | 52.5% |
| 1920x1088 | 0.0% | 253.1% | 285.4% | 80.2% |
| average | 0.0% | 201.6% | 217.3% | 68.3% |

Table2.2.5 MVC sequence

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate |
| --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Flamenco2_2_1_3 | 0.0% | 0.0% | -0.1% | 0.0% |
| Akko&Kayo_48_46_50 | 0.0% | 0.0% | 0.1% | 0.0% |
| Akko&Kayo_48_26_70 | 0.0% | 26.9% | 28.3% | 14.0% |
| Breakdancers_4_1_7 | 0.0% | -0.1% | -0.1% | 0.0% |
| average | 0.0% | 5.3% | 5.6% | 2.8% |

# Discussion

- From the results in Section 2, to enforce the constraint for all input is overkill because the loss is significant when the vertical offset is larger than the constraint.

- No significant loss is observed in non-rectified 1D array case in Table 2.1.5 and 2.2.5. Therefore, regarding the DV range constraint, the value 56 [pixel] is reasonable since it covers most Stereo or 1D array case. It enables one LCU line delay decode in the dependent view from the base view even LCU size is maximum.

It is noted that the value 56 comes from the maximum LCU size 64 minus 8. The value 8 is for in-loop and interpolation filter. Please find the detail in [2], [4].

- Some post processing is necessary when the input has larger vertical offset than the constraint. Otherwise, it is not possible to view (or use) the decoded images directly. Thus, parallel decoding functionality for low-delay may not be necessary for such cases.

- To support vertical camera position is necessary for some application (e.g. light field camera).

# Proposal

Taking into account the discussion in the last section, it is proposed to define two profiles as MVC to satisfy the various requirements. One is namely Stereo-profile and the other is Multi-view profile, where the disparity vector constraints are mandated in the former but not mandated in the latter. Thus, it is asserted that low delay feature is realized in Stereo profile and high flexibility to the input is ensured in Multi-view profile.

Regarding to the constraint value, fixed value as 56 is proposed. The value is fixed in the semantics.

To clarify the constraint, it is proposed to add a 1-bit sequence level flag in SPS extension as disparity_vector_constraint_flag. It is asserted that the flag is useful to enable bitstream-level transcode from Multiview to Stereo profile. Figure 4.1 shows an example. When disparity_vector_constraint_flag is equal to 1 in some views in a Multiview profile bitstream, they can be extracted to a Stereo profile bitstream without full decoding and encoding. If there is no flag in the bitstream, only encoders know the constraint.

Figure 4.1 bitstream level transcoding from Multiview to Stereo profile

# Draft text

##### F.7.3.2.2 Sequence parameter set extension RBSP syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| disparity_vector_constraint_flag | u(1) |
| } |  |

- F.7.4.2.2 Sequence parameter set extension RBSP semantics

disparity_vector_constraint_flag equal to 1 specifies that disparity vector is constrained in the coded video sequence. When disparity_vector_constraint_flag is equal to 1, the vertical component of the disparity vector shall be equal to or less than 56 in pixel. When disparity_vector_constraint_flag is equal to 0, there is no constraint for disparity vector length. When not present, the value is inferred to be 0.

- Profiles

- F.1.1  Stereo profile

Bitstreams conforming to the Stereo profile shall obey the following constraints:

- Sequence parameter set extensions in the non-base view shall have disparity_vector_constraint_flag equal to 1 only.

F.1.2  Multiview profile

Bitstreams conforming to the Multiview profile shall obey the following constraints:

.

# References

- JCT3V-B1004, “MV-HEVC Draft Text 2”

- JCT3V-B0037, “MV-HEVC: Vertical length restriction of inter-view vector for HEVC simple 3D extension”

- JCT3V-B0087, “3D-CE5.h related: Constrained DV for inter-view data access”

- JCT3V-B0112, ”3D-CE5.h related: Disparity vector restrictions”

- JCT3V-B0113, “3D-CE5.h related: Restricted motion vector coding for inter-view prediction”

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”

- JCT3V-B_Notes_d7, “Meeting report of the second meeting of the Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V), Shanghai, CN, 13-19 Oct. 2012”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).