| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0079 |
| --- | --- |

| Title: | - AHG7: On initialization process for reference picture lists of MV-HEVC and 3DHTM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Yoshitomo Takahashi Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com yoshitomo.takahashi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In the current MV-HEVC draft (JCT3V-B1004_d0), inter-view reference picture is appended at the end of temporal reference picture list. It is noted that reference picture list modification (RPLM) syntax is necessary to put the inter-view reference forward.

This contribution proposes to modify the initialization process for reference picture lists. An inter-view reference picture is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter pictures in creating the temporal list. Since it is a semantics change, no syntax is added to the base spec. The change enables to skip RPLM signalling in the common reference picture structure (e.g. B-pictures in traditional N15M3 GOP structure) without extra syntax.

The proposal is implemented on top of HTM5.0.1. It is reported the BD-rate difference in common test condition is 0.0%, -0.1%, -0.1% and -0.1% for video0, video1, video2 and coded & synthesized, respectively. The difference comes from RPLM signalling bit reduction in the dependent view.

# Problem Statement

In the current MV-HEVC draft (JCT3V-B1004_d0), initialization process for reference picture lists is specified in F.8.3.4.2. As noted, an inter-view reference picture is always appended at the end of temporal reference picture list (RefPicListTempX).

Figure 1 visualizes four types of reference picture to create RefPicListTempX. White box with “C” represents a current picture. Green and red boxes stand for short-term reference pictures which are before and after the current picture in POC, respectively. Purples are long-term pictures, and blues are inter-view pictures. In the figure, the number in the boxes is indexed based on POC or layerId as specified in F.8.3.2.

Figure 1: four types of reference picture to create RefPicListTemp0/1

An example of the current initialization process is reportedly shown in Figure 2.

Firstly, four lists, which are RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr and RefPicSetIvCurr, are created from the reference pictures with used_by_curr_pic_flag. When used_by_curr_pic_sX_flag is equal to 0, the reference picture is not appended to the each list. In the example, two reference pictures with index 0 and 1 are appended to RefPicSetStCurrBefore.

Then, RefPicListTempX is created from the four lists.

The order to be appended for list0 is

RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr and RefPicSetIvCurr.

The order to be appended for list1 is

RefPicSetStCurrAfter, RefPicSetStCurrBefore, RefPicSetLtCurr and RefPicSetIvCurr.

It is noted that RefPicSetIvCurr is appended at the end for both lists.

Finally, reference picture list (RefPicListX) is created from RefPicListTempX. When a reference picture list modification (RPLM) syntax is not present, the first (num_ref_idx_lx_active_minus1+1) pictures in RefPicListTempX is copied to RefPicListX. Otherwise, RefPicListX is created according to the RPLM syntax until the list is full.

Figure 2: an example of the current initialization process for reference picture lists

Here, following issues are raised regarding the current process.

- When num_ref_idx_lx_active_minus1 is small, inter-view reference picture might not be included in RefPicListX without sending RPLM syntax. In the example, the inter-view reference picture (blue box) is not included to the RefPicListX. It is noted that num_ref_idx_lx_active_minus1 is 3 or 4 in usual bitstream as CTC from decoder complexity point of view.

- Generally, it makes coding efficiency better to assign smaller reference index to more correlated reference picture. Since inter-view prediction gives significant coding gain, it is reasonable to append an inter-view picture forward in creating the default list.

- The position of RefPicSetIvCurr should be different in RefPicList0 and RefPicList1 by default. It increases the possibility that the two lists are unique and can contain all reference pictures. It is believed that this helps encoders to choice better prediction mode from the default list.

# Proposal

An inter-view reference picture is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter pictures in creating RefPicListTempX. See Figure 3.

In proposal, the order for list 0 is

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

The inter-view picture list is apeende in the second.

The order for list1 is same as current.

Figure 3: an example of the proposed initialization process

# Experimental results

The proposal is implemented on top of HTM5.0.1. To align the reference picture structure with common test condition, RPLM signalling is modified in the encoder side. Precisely, RPLM is not send for POC 1, 3 and 5 for List0 since inter-view reference picture is appended to the last position of RefPicList0 without RPLM. The setting in the configuration file is shown in Table1.

Table2 shows experimental results. The gain in video1 and video2 comes from the RPLM signalling bits reduction. There is no coding efficiency impact on video0 since inter-view prediction is not used in the base view.

Table1 RPLM signalling to align common test condition

| Config parameter | PicType | POC | List0 |  |  | List1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Curr. | Prop. | Final list of 1st GOP | Curr. | Prop. | Final list of 1st GOP |
| FrameI_v1(v2) | P | 0 | 1 | 1 | [V0 ] | 0 | 0 | [ ] |
| Frame1_v1(v2) | B | 8 | 2 | 2 | [0 V0 ] | 0 | 0 | [0 V0 ] |
| Frame2_v1(v2) | B | 4 | 2 | 2 | [0 V0 8 ] | 0 | 0 | [8 0 V0 ] |
| Frame3_v1(v2) | B | 2 | 2 | 2 | [0 V0 4 ] | 0 | 0 | [4 8 0 ] |
| Frame4_v1(v2) | B | 1 | 2 | 0 | [0 V0 2 ] | 0 | 0 | [2 4 8 ] |
| Frame5_v1(v2) | B | 3 | 3 | 0 | [2 0 V0 ] | 0 | 0 | [4 8 2 ] |
| Frame6_v1(v2) | B | 6 | 3 | 3 | [4 2 V0 ] | 0 | 0 | [8 4 2 ] |
| Frame7_v1(v2) | B | 5 | 3 | 0 | [4 0 V0 ] | 0 | 0 | [6 8 4 ] |
| Frame8_v1(v2) | B | 7 | 3 | 3 | [6 4 V0 ] | 0 | 0 | [8 6 4 ] |

Table2 common test condition result

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Kendo | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| GhostTownFly | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| PoznanStreet | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |

# Draft text

The proposed text is as follows. The change part is yellow highlighted.

- F 8.3.4.2 Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0; i < NumIvCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetIvCurr[ i ]
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(F-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(F-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]	
 for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(F-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumIvCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetIvCurr[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)		(F-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# References

- JCT3V-B1004, “MV-HEVC Draft Text 2”

- JCT3V-B0046, “AHG5: MV-HEVC software for HTM”

- JCTVC-K1003, “High Efficiency Video Coding (HEVC) text specification draft 9”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).