| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0080_r1 |
| --- | --- |

| Title: | AHG13: Cross-check of Disparity vector restrictions (JCT3V-C0083) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document is a crosscheck of JCT3V-C0083.

In the proposal, 1) vertical component of disparity vector is constrained to zero in motion parameter derivation stage, 2) absolute value of vertical component of disparity vector is clipped to 56 in pixel in  motion compensation stage, and 3) 1-bit flag is added to sequence parameter set extensions to control the constraints regarding 1) and 2).

Cross checker confirmed that the software and the text provided by the proponent are consistent. The simulation is conducted under AHG13 condition. The results by the cross checker are identical to the results by the proponent.

In the revision 1, cross check results of shift 16, 32 and 64 are added. The results are matched with the propnents.

# Crosscheck

The software provided by Sharp is implemented on top of HTM5.1. When the macro SHARP_DV_CONSTRAINT is enabled, the following three parts are modified. Cross checker confirmed the provided text is consistent regarding 1.2 and 1.3.

## Sequence parameter set extension flag

disparity_vector_constraint_flag is coded in sequence parameter set. The value at encoder is always equal to 1 when the macro is enabled in the current implementation.

## Derivation process for a temporal inter-view motion vector candidate

When disparity_vector_constraint_flag is equal to 1, vertical component of disparity vector is not used for motion parameter prediction process. Regarding the horizontal component, the process is same as current HTM.

## Decoding process for inter prediction samples

When disparity_vector_constraint_flag is equal to 1, absolute value of vertical component of disparity vector is clipped to 56 in pixel in motion compensation stage. The horizontal component is not clipped.

# Experimental results

The simulation is conducted under AHG13 condition. The results are identical to what the proponent provided.

Table 2.1 Results for MVC sequence under AHG13 condition

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | 6.8% | 6.1% | 3.4% | 3.4% |
| Flamenco2_2_1_3 | 0.0% | 6.6% | 4.7% | 2.2% | 2.2% |
| Akko&Kayo_48_46_50 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Akko&Kayo_48_26_70 | 0.0% | 28.1% | 29.8% | 14.7% | 14.5% |
| Breakdancers_4_1_7 | 0.0% | 0.4% | 0.1% | 0.2% | 0.2% |
| average | 0.0% | 8.4% | 8.2% | 4.1% | 4.1% |

Table 2.2 Results under CTC (VerticalShiftValueinPixel=0)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| GhostTownFly | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| PoznanHall2 | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| PoznanStreet | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| 1920x1088 | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 2.3 Results under CTC (VerticalShiftValueinPixel=16)

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 6.3% | 14.2% | 4.5% |
| Kendo | 0.0% | 4.8% | 10.4% | 3.4% |
| Newspapercc | 0.0% | 3.7% | 6.8% | 2.2% |
| GhostTownFly | 0.0% | 4.7% | 8.8% | 2.1% |
| PoznanHall2 | 0.0% | 1.5% | 4.4% | 1.5% |
| PoznanStreet | 0.0% | 3.2% | 6.5% | 1.7% |
| UndoDancer | 0.0% | 2.4% | 6.8% | 1.5% |
| 1024x768 | 0.0% | 4.9% | 10.5% | 3.4% |
| 1920x1088 | 0.0% | 2.9% | 6.6% | 1.7% |
| average | 0.0% | 3.8% | 8.3% | 2.4% |

Table 2.4 Results under CTC (VerticalShiftValueinPixel=32)

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 6.3% | 14.2% | 4.5% |
| Kendo | 0.0% | 4.8% | 10.4% | 3.4% |
| Newspapercc | 0.0% | 3.7% | 6.8% | 2.2% |
| GhostTownFly | 0.0% | 4.7% | 8.8% | 2.1% |
| PoznanHall2 | 0.0% | 1.5% | 4.4% | 1.5% |
| PoznanStreet | 0.0% | 3.2% | 6.5% | 1.7% |
| UndoDancer | 0.0% | 2.4% | 6.8% | 1.5% |
| 1024x768 | 0.0% | 4.9% | 10.5% | 3.4% |
| 1920x1088 | 0.0% | 2.9% | 6.6% | 1.7% |
| average | 0.0% | 3.8% | 8.3% | 2.4% |

Table 2.5 Results under CTC (VerticalShiftValueinPixel=64)

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 148.2% | 138.0% | 57.4% |
| Kendo | 0.0% | 124.6% | 119.2% | 51.4% |
| Newspapercc | 0.0% | 149.7% | 148.9% | 58.8% |
| GhostTownFly | 0.0% | 446.6% | 494.3% | 117.5% |
| PoznanHall2 | 0.0% | 100.5% | 125.2% | 49.8% |
| PoznanStreet | 0.0% | 234.5% | 261.0% | 79.6% |
| UndoDancer | 0.0% | 304.9% | 366.2% | 99.4% |
| 1024x768 | 0.0% | 140.8% | 135.4% | 55.9% |
| 1920x1088 | 0.0% | 271.6% | 311.7% | 86.6% |
| average | 0.0% | 215.6% | 236.1% | 73.4% |

# Reference

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”

- JCT3V-C0083, “AHG13: Disparity vector restrictions”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).