| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0081 |
| --- | --- |

| Title: | AHG7: On Random access point pictures and picture order counts for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Min Woo Park Jaewon Yoon Chanyul Kim Junghoon Park | Tel: Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

To avoid a missing of inter-view reference pictures and make the random access and splicing easier, we propose several syntax changes on picture order count, NAL unit types, and slice types. First, we propose a restriction on picture order counts (POCs) for all view components of a picture. If the POC values of view components in an access unit are not aligned, it may result in a decoding error or a corrupted decoded picture due to a wrong inter-view prediction. Second, we propose a restriction on NAL unit types that all NAL units of all view components of a picture shall have the same value of nal_unit_type for a clean random access and splcing. Finally, we suggest to change the restriction on slice types for non-base view components of RAP pictures for enabling inter-view prediction.

# Picture order count alignment in an access unit

In this proposal, we propose a restriction on picture order counts (POCs) for all view components of a picture. If the POC values of view components in an access unit are not aligned, it may result in a decoding error or a corrupted decoded picture due to a wrong inter-view prediction. Moreover, when the decoded picture buffer of each view is independently managed, a mis-alignment of POCs makes it difficult to synchronize the outputs of all view components of a picture. Therefore, adding the following text to the MV-HEVC specification text is proposed, to align the POCs of all view components of a picture.

- F.8.3.1	Decoding process for picture order count

The process specified in this subclause is invoked for a particular view. The specifications

in subclause 8.3.1 apply independently for each view, with "picture" being replaced by "view component"

.

The following constraints shall be obeyed:

–It is a requirement of bitstream conformance that the bitstream shall not contain data

that result in different values of PicOrderCntVal for the view components of the access unit.

Or alternatively

–It is a requirement of bitstream conformance that the bitstream shall not contain data

that result in different values of prevPicOrderCntLsb for the view components of the access unit.

–It is a requirement of bitstream conformance that the bitstream shall not contain data

that result in different values of prevPicOrderCntMsb for the view components of the access unit.

# Restriction on NAL unit types

We propose a restriction on NAL unit types that all NAL units of all view components of a picture shall have the same value of nal_unit_type. In MVC, idr_flag is used for indicating the current NAL unit of a view component belongs to an IDR access unit. However, idr_flag is not present in MV-HEVC, and there is no room to signal additional information in NAL unit headers. Therefore, it is natural that the NAL unit types of slice segments of non-base view components in IDR access units are IDR for indiciating those NAL units belong to IDR access units. Similarly, for the alignment of NAL unit types of view components in CRA access units and BLA access units, the NAL unit types of all non-base view components in CRA access unit and BLA access units shall be CRA and BLA, respectively. The alignment of NAL unit types in an access unit make the splicing easiler and avoid a missing of inter-view reference pictures. Therefore, we propose to add the following text to the MV-HEVC specification.

- F.7.4.1.2	NAL unit header semantics

layer_id identifies the layer the NAL unit belongs to. layer_id shall be equal to 0 for any NAL unit of the base view. View order index of each NAL unit is derived to be equal to layer_id.

All coded slice segment NAL units of all view components of an access unit shall have the same value of nal_unit_type.

Or alternatively only for RAP-related pictures

When the NAL unit type of the base view component of a picture is IDR, the NAL unit types of the other view components of the picture shall be IDR.

When the NAL unit type of the base view component of a picture is CRA, the NAL unit types of the other view components of the picture shall be CRA.

When the NAL unit type of the base view component of a picture is BLA, the NAL unit types of the other view components of the picture shall be BLA.

When the NAL unit type of the base view component of a picture is RASL, the NAL unit types of the other view components of the picture shall be RASL.

# Modification on slice type mapping for random access point pictures

To enable that slices of non-base view component in RAP pictures have P or B for inter-view prediction, we propose to change the restriction on slice types of RAP pictures as follows.

#### F.7.4.8.1	General slice segment header semantics

Table F 8-2 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

When nal_unit_type has a value in the range of 16 to 23 and nuh_layer_id is equal to 0, inclusive (RAP picture), slice_type shall be equal to 2.

NOTE – When nal_unit_type has a value in the range of 16 to 23 and nuh_layer_id is not equal to 0, slice_type can be equal to 0 or 1 for inter-view prediction.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).