| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0082 |
| --- | --- |

| Title: | AHG7: Reference picture marking process for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Min Woo Park Jaewon Yoon Chanyul Kim Junghoon Park | Tel: Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

A modified reference picture marking process for inter-view reference pictures is proposed. By marking the inter-view reference pictures as “used for inter-view reference” explicitly, the inter-view reference pictures can be handled in distinction from other inter-prediction reference pictures marked as “used for short-term reference” or “used for long-term reference”. The proposed marking process ensures that the inter-view prediction is done among view components of an access unit without incorrect references and reference picture marking errors.

# Introduction

In the reference picture marking process in the latest MV-HEVC specification text, the decoded view component is marked as “used for short-term reference”, after all slices of the current view component have been decoded. The reference pictures are marked as “unused for reference” or “used for long-term reference” after the reference picture sets in the slice header of the first slice segment of the following picture are decoded. Thus, the reference picture, which will be referenced by other view components of the same picture, is marked as a short-term reference picture.

The current reference picture marking process in MV-HEVC is the straightforward extension of the HEVC reference picture marking process without considering an explicit marking process for inter-view reference pictures. This process has several drawbacks and a critical problem. When another view component accesses to the decoded picture buffer (DPB) of the referred view component, finding out the corresponding inter-view reference picture among many reference pictures in the DPB is not simple, because the inter-view reference picture is marked as “used for short-term reference” same as other inter-prediction reference pictures. To find out the correct inter-view reference picture, all reference pictures marked as “used for short-term reference” shall be inspected. Moreover, if the POC values are not aligned in an access unit due to the loss of NAL units of view components or unequal random access among views, the inter-view prediction will be misbehaved. The most critical problem is that the decoded view component is marked as “used for long-term reference” after it is referred by another view component for inter-view prediction. This picture marked as “used for long-term reference” cannot be returned to “used for short-term reference” because this reference picture can be marked as “used for long-term reference” or “unused for reference” after the first slice header of the following picture is decoded. It results in a serious problem in the buffer management that the short-term reference pictures are marked as “used for long-term reference”.

Therefore, we propose a modified reference picture marking process for inter-view reference pictures. By marking the inter-view reference pictures as “used for inter-view reference” explicitly, the inter-view reference pictures can be handled in distinction from other inter-prediction reference pictures. Furthermore, the proposed marking process ensures that the inter-view prediction is done among view components of an access unit without incorrect references. The proposed reference picture marking process has the following benefits:

- Easy handling of inter-view reference picture in distinction from inter-prediction reference pictures.

- Fast search of inter-view reference picture in DPB.

- Restriction that the inter-view prediction is allowed between view components of a picture.

- Error robustness against POC misalignment between view components.

- No reference picture marking error.

# Proposed reference picture marking process

Figure 1. The current inter-view reference picture marking process in MV-HEVC

In the current inter-view reference marking process, the decoded view component is stored in a DPB after all corresponding slices are decoded. Then, it is marked as “used for short-term reference”. When another view component tries to refer to this decoded picture for inter-view prediction, all reference pictures marked as “used for short-term reference” in the DPB are inspected to find out the correct inter-view reference picture among them. The selected inter-view reference picture is included in RefPicSetIvCurr and makred as “used for long-term reference”. When the following access unit is decoded after all view components of the current picture are decoded, the inter-view reference picture marked as “used for long-term reference” may or may not be marked as “unused for reference” or “used for long-term reference” after the slice header of the first slice segment of the same view is decoded.

Figure 2. The proposed inter-view reference picture marking process

In the proposed inter-view reference marking process, the decoded view component is stored in a DPB after all corresponding slices are decoded. Then, it is marked as “used for inter-view reference”.  For an inter-view prediction of another view component, the reference picture marked as “used for inter-view reference” is included in RefPicSetIvCurr, and marked again as “used for long-term reference”. When the following access unit is decoded after all view components of the current picture are decoded, the inter-view reference picture marked as “used for long-term reference” may or may not be marked as “unused for reference” or “used for short-term reference” after the slice header of the first slice segment of the same view is decoded. The decoded reference pictures marked as “used for inter-view reference”, but not used for an inter-view prediction, are also marked as “unused for reference”, “used for short-term reference”, or “used for long-term reference”.

# Proposed text changes

- F.3	Definitions

- inter-view reference picture: A decoded view component of an inter-view reference component. An inter-view reference picture contains samples that may be used for inter-view prediction in the decoding process of subsequent pictures in decoding order. Inter-view reference pictures is marked as “used for inter-view reference”.

- inter-view subset reference picture set: A set of reference pictures marked “used for inter-view reference” as that may be used for inter-view prediction of the associated picture or any picture following the associated picture in decoding order.

- F.8.1	General decoding process

The decoding process operates as follows for the current view component:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found (which only needs to be invoked for the first slice segment of a view component).

–	The decoding process for reference picture set in subclause Error: Reference source not found is invoked, wherein reference pictures may be marked as "unused for reference", "used for short-term reference", or "used for long-term reference" (which only needs to be invoked for the first slice segment of a view component).

–	When the current view component is a BLA view component or is a CRA view component that is the first view component in the bitstream, the decoding process for generating unavailable reference view component specified in subclause Error: Reference source not found is invoked (which only needs to be invoked for the first slice segment of a view component).

–	PicOutputFlag is set as follows:

–	If the current view component is a RASL view component and the previous RAP view component in decoding order is a BLA view component or is a CRA view component that is the first coded view component in the bitstream, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

–	After all slices of the current view component have been decoded, all the decoded view components in the current access unit are marked as "used for inter-view reference".

- F.8.3.2	Decoding process for reference picture set

…

This process is invoked once per view component, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause F.8.3.3.1. The process may result in marking one or more reference pictures as "unused for reference", "used for short-term reference" or "used for long-term reference".

…

A reference picture can be marked as "unused for reference", "used for short-term reference", "used for long-term reference", or “used for inter-view reference”, but only one among these four. When a reference picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference", "used for long-term reference", or “used for inter-view reference”.

…

- F.8.3.3	Picture decoding, marking and storage

When decoding a coded video sequence conforming to a profile specified in Annex Error: Reference source not found using the decoding process specified in Annex Error: Reference source not found, the decoding process as specified in subclause C.4.3 is repeatedly invoked for each view in ascending order of layer_id, with "picture" being replaced by "view component".

When decoding a coded video sequence conforming to a profile specified in Annex Error: Reference source not found using the decoding process specified in Annex Error: Reference source not found, the following decoding process is repeatedly invoked for each view in ascending order of view order index.

The following happens instantaneously when the last decoding unit of access unit n containing the current view component is removed from the CPB.

The current view component is considered as decoded after the last decoding unit of the view component is decoded. The current decoded view component is stored in an empty view component storage buffer in the DPB, and the following applies.

–	If the current decoded view component has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded view component has PicOutputFlag equal to 0), it is marked as "not needed for output".

The current decoded view component is marked as "used for inter-view reference".

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).