| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0083 |
| --- | --- |

| Title: | AHG13: Disparity vector restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yoshiya Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- In practical usage, it is beneficial if base view and dependent views can be decoded simultaneously. However it cannot be achieved because the dependent view may depend on whole picture of base view in the current scheme.  In this contribution, the vertical range of disparity vector is restricted to zero in inter-view motion derivation stage (IV) and also the vertical range is restricted within 56 pixels in “motion compensation stage”(MC). The restriction in IV reduces memory access complexity and the restriction in MC guarantees the range in both decoder and encoder.  The methods are controlled by a SPS extension flag and the flag shall be set equal to 1 in the proposed Stereo profile. The simulation results shows that the restriction brings no or small coding loss in CTC and non-rectified MVC sequences while there are significant loss in 2D-array sequences.

# Motivation

What is special to 3DV application other than other HEVC extension (f.g. spatial scalable) is that several views shall be presented simultaneously.  Let us assume we design 3DV decoder by re-using the HEVC decoder design, of which performance is designed to only meet for one picture (one view). In that case, if we present several views for 3DV application, the decoder needs to decode each views simultaneously. Otherwise we need to completely renew the design of decoder for 3DV.

We assert that this parallelism scheme is the easiest way among other parallel scheme.  For example, imposing mandatory use of tile or WPP needs encoder side design complexity and could bring coding loss while, while this DV restriction scheme has small complexity or reduce complexity in memory access and there are no coding loss in rectified image.

Figure 1: Practical case

# Proposal

We propose:

- (1) SPS extension flag

- (2) DV restriction in Inter-View motion derivation when the above flag is equal to 1

- (3) DV restriction in Motion Compensation when the above flag is equal to 1

- (4) Two profiles (Stereo profile and Multiview Profile)

- SPS extension flag

##### F.7.3.2.2 Sequence parameter set extension RBSP syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| disparity_vector_constraint_flag | u(1) |
| } |  |

- F.7.4.2.2 Sequence parameter set extension RBSP semantics

disparity_vector_constraint_flag equal to 1 specifies that disparity vector is constrained in the coded video sequence. When disparity_vector_constraint_flag is equal to 1, the vertical component of the disparity vector is restricted in the derivation of inter-view motion vector candidate and decoding process for inter prediction samples.  When not present, the value is inferred to be 0.

- Change of Inter-view motion derivation process

Derivation process for a temporal inter-view motion vector candidate

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) )	(G 127)

When dv_constraint_enabled is equal to 0, the following applies.

yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ))	(G 128)

Otherwise (dv_constraint_enabled is equal to 1) the following applies.

yRef = Clip3( 0, PicHeightInSamplesL – 1, yP)) 	(G 128)

- Change of motion compensation process

      - Decoding process for inter prediction samples

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1.

Outputs of this process are:

…

When dv_constraint_enabled is equal to 1 and predFlagL0 is equal to 1 and ViewIdx is not equal to the view order index of RefPicListL0[ refIdxL0 ], the following applies.

mvL0[1] = Clip3(-56*4, 56*4, mvL0[1])

mvCL0[1] = mvL0[1]

When dv_constraint_enabled is equal to 1 and predFlagL1 is equal to 1 and ViewIdx is not equal to the view order index of RefPicListL1[ refIdxL1 ], the following applies.

mvL1[1] = Clip3(-56*4, 56*4, mvL1[1])

mvCL1[1] = mvL1[1]

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

- Profiles

- F.1.1  Stereo profile

Bitstreams conforming to the Stereo profile shall obey the following constraints:

Sequence parameter set extensions shall have disparity_vector_constraint_flag equal to 1 only.

F.1.2  Multiview profile

Bitstreams conforming to the Multiview profile shall obey the following constraints:

# Experimental results

- The experimental results of our proposal are shown in Table 1 to Table3.

- Table 1 shows that the restriction doesn’t bring coding loss in CTC.

- Table 2 shows that the restriction brings no or small coding loss in some non-rectified MVC sequences while there are significant losses in 2D-array sequences

- Table 3 shows that the restriction brings small or moderate coding loss in vertical shift of 16/32 and significant loss in vertical shift of 64.

- In Table 4, AHG13 experiments (encoder only restriction) is shown. From the comparison between Table3 and Table4, the proposed restriction produce small coding loss in vertical shift is 16 and 32.

Table 1: Experimental results on CTC condition  (Proposal)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 100.1% | 99.0% | 100.4% |
| Kendo | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 99.9% | 100.2% | 99.4% |
| Newspapercc | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 100.1% | 100.7% | 100.0% |
| GhostTownFly | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 99.6% | 99.7% |
| PoznanHall2 | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% | 99.9% | 100.8% | 100.1% |
| PoznanStreet | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 100.8% | 100.3% |
| UndoDancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 99.7% | 100.6% | 100.2% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | 100.0% | 100.0% | 100.0% |
| 1920x1088 | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 100.4% | 100.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.2% | 100.0% |

Table 2: Experimental results on MVC sequences  (Proposal)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flamenco2_0_1_3 | 0.0% | 6.8% | 6.1% | 3.4% | 3.4% |  | 100.9% | 98.2% |  |
| Flamenco2_2_1_3 | 0.0% | 6.6% | 4.7% | 2.2% | 2.2% |  | 100.9% | 97.5% |  |
| Akko&Kayo_48_46_50 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |  | 100.0% | 100.5% |  |
| Akko&Kayo_48_26_70 | 0.0% | 28.1% | 29.8% | 14.7% | 14.5% |  | 97.7% | 97.2% |  |
| Breakdancers_4_1_7 | 0.0% | 0.4% | 0.1% | 0.2% | 0.2% |  | 99.5% | 98.5% |  |
| average | 0.0% | 8.4% | 8.2% | 4.1% | 4.1% |  | 99.8% | 98.4% |  |

Table 3: Experimental results on artificially shifted image (Proposal)

|  | shift = 16 | shift = 32 | shift = 64 |
| --- | --- | --- | --- |
| Balloons | 2.19% | 3.64% | 48.97% |
| Kendo | 1.50% | 2.49% | 41.41% |
| Newspaper_CC | 0.91% | 1.42% | 47.13% |
| GT_Fly | 0.82% | 1.59% | 112.06% |
| Poznan_Hall2 | 0.35% | 0.98% | 44.44% |
| Poznan_Street | 0.67% | 1.39% | 73.90% |
| Undo_Dancer | 0.79% | 0.96% | 28.01% |
| 1024x768 | 1.53% | 2.52% | 45.84% |
| 1920x1088 | 0.66% | 1.23% | 64.60% |
| average | 1.03% | 1.78% | 56.56% |

Table 3: Experimental results on artificially shifted image (encoder only change)

|  | shift = 16 | shift = 32 | shift = 64 |
| --- | --- | --- | --- |
| Balloons | -0.04% | -0.04% | 45.89% |
| Kendo | -0.01% | -0.10% | 38.19% |
| Newspaper_CC | -0.01% | -0.10% | 45.02% |
| GT_Fly | 0.01% | -0.05% | 101.40% |
| Poznan_Hall2 | -0.02% | -0.05% | 37.62% |
| Poznan_Street | -0.09% | -0.05% | 70.51% |
| Undo_Dancer | 0.00% | -0.05% | 92.08% |
| 1024x768 | -0.02% | -0.08% | 43.03% |
| 1920x1088 | -0.02% | -0.05% | 75.40% |
| average | -0.02% | -0.06% | 61.53% |

# Conclusion

- In this contribution, the vertical range of disparity vector is restricted to zero in inter-view motion derivation stage (IV) and also the vertical range is restricted within 56 pixels in “motion compensation stage”(MC). The restriction in IV reduces memory access complexity and the restriction in MC guarantees the range in both decoder and encoder.  The methods are controlled by a SPS extension flag and the flag shall be set equal to 1 in the proposed Stereo profile. The simulation results shows that the restriction brings no or small coding loss in CTC and non-rectified MVC sequences while there are significant loss in 2D-array sequences.

- It is recommended to adopt the proposed method in HTM6.

# References

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, JCT3V-B1005, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

[2] T. Ikai, T. Uchiumi, “3D-HEVC: Disparity vector restrictions”, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).