| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0084 |
| --- | --- |

| Title: | AHG7: POC alignment between layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yoshiya Yamamoto Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In 3D-HEVC Test Model 2 [JCT3V-B1005] reads “An access unit includes all video pictures and depth maps that correspond to the same time instant”.  However there is no specific way or no specific text for this condition in terms of POC.  Specifically coded slice segment NAL units of an access unit can have different POC which results in unnecessary difficulties in presentation time management in decoder.  This contribution provides a specific way to guarantee the same POC in the slices between layers.

# Motivation

If coded slice segment NAL units of an access unit can have different POC, it is difficult to manage presentation time in decoder.  Therefore we propose that All coded slice segment NAL units of an access unit shall have the POC.  Because POC is derived by the several set of process, we provide a specific way of semantics to guarantee the same POC in the slices between layers.

# POC derivation process in HEVC

The POC derivation process in HEVC WD9 is as follows.

- When none of the following conditions is true: [Ed. (GJS): Logical structure of sentence seems strange.]

- The current picture is an IDR

- The current picture is a BLA picture

- The current picture is a CRA picture and is the first coded picture in the bitstream

- the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows. Let prevTid0Pic be the previous reference picture in decoding order that has TemporalId equal to 0. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevTid0Pic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

- The variable PicOrderCntMsb of the current picture is derived as follows.

- If the current picture is an IDR or a BLA picture, or if the first coded picture in the bitstream is a CRA picture and the current picture is the first coded picture in the bitstream, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as

PicOrderCntVal = PicOrderCntMsb + pic_order_cnt_lsb

Figure 1: Derivation process of picture order count value

Specifically the POC is initialized when the NAL unit is IDR or BLR and the POC is calculated with the pic_order_cnt_lsb in slice_header and log2_max_pic_order_cnt_lsb_minus4 in SPS.

# Proposal

We propose

- 1) When the base-layer is IDR or BLR, the nal_unit_type of the associated layer shall be equal to the nal_unit_type of base-layer

- 2) The value of log2_max_pic_order_cnt_lsb_minus4 of the non-base layer shall be the same as that of the associated base layer.

- 3) The value of pic_order_cnt_lsb of the non-base layer shall be the same as that of the associated base layer.

G.7.4.1 NAL unit semantics

…

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Error: Reference source not found.

All coded slice segment NAL units of each layer of an access unit shall have the same value of nal_unit_type. If the layer_id of coded slice segment NAL units is equal to 0 and the nal_unit_type of the NAL units is the range of 16 to 20 (BLA or IDR),  the associated coded slice segment NAL units of which layer_id is not equal to 0 shall has the same nal_unit_type as the coded slice segment NAL units of which layer_id is 0.

G.7.4.5 Slice header semantics

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7-2)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

All picture parameter set NAL units of an access unit shall have the same value of log2_max_pic_order_cnt_lsb_minus4.

G.7.4.5 Slice header semantics

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture.

All coded slice segment NAL units of an access unit shall have the same value of pic_order_cnt_lsb.

# Conclusion

We provide a specific semantics to guarantee the same POC in the slices between layers.

It is recommended to adopt this POC restriction in MV-HEVC and 3D-HEVC.

# References

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, JCT3V-B1005, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).