| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0085 |
| --- | --- |

| Title: | AHG7: RAP picture alignment and slice definition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yoshiya Yamamoto Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In 3DV application, base view and dependent view is due to be presented simultaneously and this requirement is also true when random access is used.  However in the current specification, there are cases that only base picture can be decoded with random access because there are no restriction on RAP pictures usage in dependent layers.  This contribution proposes a restriction that when base layer is a RAP picture, the associated non base layers shall be also the RAP type picture.  In addition, this contribution proposes the I_SLICE restriction in RAP picture is only applied in base layer to improve coding efficiency in the case that both base and dependent layers are RAP pictures.

# Problem statement

In the current specification, there are no restrictions on RAP pictures usage in dependent layers.  When we search for a RAP picture in base layer and try to decode from that picture, it could be possible we cannot start that picture for 3D application because the dependent layers may not be RAP pictures as shown Figure 1.

Figure 1. Problem of the current RAP picture restriction

If we set a restriction that dependent layers shall be RAP pictures when associated base layer is a RAP picture. We can start that picture. However the restriction is supposed to bring coding losses because RAP picture cannot use inter-prediction (inter-layer prediction) because the slice type of RAP picture is restricted to I_SLICE.

Figure 2. Problem of the current slice type restriction on RAP picture

# Proposal

We propose that the associated texture picure and associated depth picture shall be the same type of RAP picture and free I_SLICE restriction in non base layer RAP picture.

Figure 3. Proposed RAP picture and slice restriction

G.7.4.1 NAL unit semantics

…

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Error: Reference source not found.

All coded slice segment NAL units of all layers of an access unit shall have the same value of nal_unit_type.

or

All coded slice segment NAL units of each layer of an access unit shall have the same value of nal_unit_type. If the layer_id of coded slice segment NAL units is equal to 0 and the nal_unit_type of the NAL units is the range of 16 to 23,  the associated coded slice segment NAL units of which layer_id is not equal to 0 shall has the same nal_unit_type as the coded slice segment NAL units of which layer_id is 0.

G.7.4.5 Slice header semantics

When nal_unit_type has a value in the range of 16 to 23 inclusive (RAP picture) and layer_id is equal to 0, slice_type shall be equal to 2.

# Conclusion

This contribution proposes a restriction that when base layer is a RAP picture, the associated non-base layer picture shall be the same type of RAP picture and free I_SLICE restriction in RAP picture. It is recommended to adopt this RAP picture and slice type semantics in MV-HEVC and 3D-HEVC.

# References

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, JCT3V-B1005, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).