| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0086 |
| --- | --- |

| Title: | AHG7: On VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yoshiya Yamamoto Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In the last JCT-VC and JCT-3V meeting, NAL unit header and parameter set designs for HEVC extensions in JCTVC-K1007 and JCT3V-B1007 has been agreed. There vps_extension with Approach 1 in JCTVC-J1007 was also revised.  It is asserted that Approach 1 is suitable for general design for HEVC base extension because it has more flexibility than Approach 2.  Currently the current 3D-HEVC Test Model 2 [JCT3V-B1005] uses Approach 2 in view_id and depth_flag definition. This contribution proposes texts of Approach 1 base vps_extension.  This contribution also proposes valid_layer_flag[ i ] which is useful to indicate some specific layers do not exist in whole coded video sequences.

# Introduction

The current 3D-HEVC Test Model 2 [JCT3V-B1005] uses Approach 2 in view_id and depth_flag, which is highlighted as red. This contribution proposes texts of Approach 1 base vps_extension.

layer_id identifies the layer the NAL unit belongs to. layer_id shall be equal to 0 for any NAL unit of the texture view components of the base view. layer_id shall be equal to 1 for any NAL unit of the depth view components of the base view.

The view order index ViewIdx and DepthFlag of each NAL unit are computed as

ViewIdx = layer_id >> 1	(G-1)

DepthFlag = layer_id%2	(G-2)

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_additional_layer_operation_points | u(8) |
| num_additional_profile_level_sets_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| reserved_zero_4bits_num_types[ i ] | u(4) |
| reserved_zero_4bits_type[ i ] | u(4) |
| view_id[ i ] | u(8) |
| if ( i>0 ) |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ] && i; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| for( i = 0; i <= num_additional_profile_level_sets_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= num_additional_layer_operation_points; i++ ) { |  |
| op_point( i ) |  |
| if (num_additional_profile_level_sets) |  |
| profile_level_idx[ i ] |  |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if ( (i ! = 0) && !( i % 2 ) ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if ( i % 2 ) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposal

We propose two options.

Option1:  Approach 1

Option2:  Approach 2 with Approach 1 design

## Option1: Approach 1

| vps_extension( ) { | Descriptor |
| --- | --- |
| .. |  |
| depth_dimension_id = 0 |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( !dimension_id[ i ][ depth_dimension_id ] ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if( dimension_id[ i ][ depth_dimension_id ] ) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The view order index ViewIdx and DepthFlag of each NAL unit are derived as follows.

ViewIdx = dimension_id[ nuh_layer_id ][ 1 ]	(G-15)

DepthFlag = dimension_id[ nuh_layer_id ][ 0 ]	(G-14)

## Option 2: Approach 2 using Approach 1 design

Even if MV-HEVC/3D-HEVC/SHVC commonly will use Approach 1 in JCTVC-J1007.  MV-HEVC/3D-HEVC can continue to utilize Approach 2 by setting some restrictions in dimension_id semantics. Approach 2 is helpful because it provide a propety that distinguish view_id or texture / depth without specifying the associate VPS and parse the syntax in decoder.  Option2 provide Approach 2 capability using Approach 1 design.

When 3D-HEVC is to allow unpaired MVD data scheme mentioned in JCT3V-B0147, it is needed to have a capability to indicate layers which are not used in the entire coded video sequences. Therefore we propose a syntax valid_layer_flag to indicate such layer.

nuh_layer_id specifies the identifier of the layer.

The view order index ViewIdx and DepthFlag of each NAL unit are computed as

ViewIdx = nuh_layer_id >> 1	(G-3)

DepthFlag = nuh_layer_id %2	(G-4)

| vps_extension( ) { | Descriptor |
| --- | --- |
| .. |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| valid_layer_flag[i] | u(1) |
| if (valid_layer_flag[i]) { |  |
| if( !dimension_id[ i ][ depth_dimension_id ] ) { // (i ! = 0) && !( i % 2 ) |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if( dimension_id[ i ][ depth_dimension_id ] ) { // i % 2 |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

valid_layer_flag[ i ] indicates whether the layer is valid. valid_layer_flag[ i ] equal to 1 specifies the layer may be used in in the coded video sequence. valid_layer_flag[ i ] equal to 0 specifies the layer shall not be used in the coded video sequence.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. When present dimension_id[ layer_id ][ 1 ] shall be equal to nuh_layer_id >>1 and dimension_id[ layer_id ][ 0 ] shall be equal to nuh_layer_id % 2

Figure 1 shows an example of camera setting. In this setting, some Texture camera and Depth camera are missing in some view.  In spite of that non-existence, Approach 2 allocates all views and depth. Therefore some layers do not appear in the coded video sequences. The proposed flag valid_layer_flag indicate the case. Table 2 shows the example of the usage of valid_layer_flag which corresponds to the case in Figure 1.

Figure 1 Example of camera setting

Table 2 Example of valid_layer_flag

| camera | nuh_layer_id | view_id | depth_flag | valid_layer_flag | Note |
| --- | --- | --- | --- | --- | --- |
|  | 0 | View 0 | Texture | 1 |  |
|  | 1 | View 0 | Depth | 0 | No depth camera |
|  | 2 | View 1 | Texture | 0 | No texture camera |
|  | 3 | View 1 | Depth | 1 |  |
|  | 4 | View 2 | Texture | 1 |  |
|  | 5 | View 2 | Depth | 0 | No depth camera |

# Conclusion

This contribution proposes texts of Approach 1 base vps_extension.  This contribution also proposes valid_layer_flag[ i ] which is useful to indicate some specific layers do not exist in whole coded video sequences.

# References

[1] P. Aflaki, D. Rusanovskyy, M. M. Hannuksela, “NAL unit header and parameter set designs for HEVC extensions”, JCT-K1007, JCT, 11th Meeting: Shanghai, CN, 10–19 Oct. 2012

[2] J. Boyce, Y.-K. Wang, “NAL unit header and parameter set designs for HEVC extensions”, JCT3V-B1007, JCT3V  2nd Meeting: Shanghai, CN , 13–19 Oct. 2012

[3] P. Aflaki, D. Rusanovskyy, M. M. Hannuksela, “MVC+D: coding of unpaired MVD data”, JCT3V-B0147, JCT3V  2nd Meeting: Shanghai, CN , 13–19 Oct. 2012

[4] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, JCT3V-B1005, JCT3V 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).