| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3nd Meeting: Geneva, Switzeland, 17–23 Jan. 2013 | Document: JCT3V-C0087 |
| --- | --- |

| Title: | 3D-CE1.h: Forward Warping Block-based View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Yin Zhao Lu Yu | Tel: Email: | felixzyc@gmail.com zhaoyin@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes the implementation of the Forward warping Block-based View Synthesis Prediction (FBVSP) on HTM. FBVSP estimates a window in base view, i.e., reference block, by only using the dependent view depth map. Then pixels in the reference block are warped to a target block in dependent view to be prediction samples. The overall bitrate savings on the coded and synthesized views are +0.4% for XGA sequences and -1.6% for HD sequences compared to HTM-5.1 and the decoding time is 106.5% for all sequences on average, where VSP is turned on for all frames of dependent-view texture and depth. The maximum bitrate savings of texture on dependent left and right views are -15.0% and -13.9%, respectively. In comparison to HTM-5.1-VSP, the overall bitrate savings on the coded and synthesized views are 0.0% for XGA sequences and +0.1% for HD sequences while the decoding time is 55.0% of HTM-5.1-VSP.

# Introduction

Current design of VSP for HTM was proposed in JCT3V-B0102 [2] in which frame-based VSP was employed. When VSP is applied on all frames of dependent view texture and depth, the latest version of HTM with VSP (HTM-5.1-VSP) shows that the overall bitrate savings on the coded and synthesized views against HTM-5.1 are +0.4% for XGA sequences and -1.5% for HD sequences and its decoding time is about 200% of HTM-5.1 on average.

Though frame-based VSP shows a great potential of coding performance improvement, its complexity is still too high for decoder. So the block-based VSP is proposed. In the 2nd JCT-3V meeting, Forward warping Block-based View Synthesis Prediction (FBVSP) was proposed in JCT3V-B0121 [1] to reduce decoding complexity of VSP module and support both texture-first and depth-first coding order of 3D-AVC.

In this contribution, we implement the FBVSP on HTM (HTM-5.1-VSP) and the test result of FBVSP is reported. The result shows that FBVSP achieves about 55.0% decoding complexity reduction against HTM-5.1-VSP without coding performance penalty when VSP is applied on all frames of dependent view texture and depth.

# Forward warping Block-based View Synthesis Prediction

Forward warping Block-based View Synthesis Prediction (FBVSP) is used to generate VSP reference samples block by block by using forward warping method. It estimates a window in reference view (based view), i.e., reference block, by only using the dependent view depth map. Then pixels in the reference block are warped to a target block in dependent view to be prediction samples. More details about FBVSP are described in JCT3V-B121 [1].

In the FBVSP implementation on HTM:

- The size of target block is fixed to be 16x16. For a PU larger than 16x16, multiple target blocks are generated to cover the whole PU and merged together to be its reference block; for a PU smaller than 16x16, the prediction samples are copied from the target block directly;

- Warping size of luma component is 1x1 and chroma components are not upsampled, so depth value of a chroma pixel is the top-left corner depth value of its corresponding 2x2 depth block;

- MV of VSP block is set to zero;

# Simulation Results

FBVSP is integrated into HTM-5.1-VSP software and compared with HTM-5.1-VSP and HTM-5.1. HTM-5.1 is tested under common test conditions (CTC) [3]. HTM-5.1-VSP and FBVSP is tested using configuration files provided by HTM-5.1-VSP.

Table 1 shows the coding performance of FBVSP vs HTM-5.1. The overall bitrate savings on the coded and synthesized views are +0.4% for XGA sequences and -1.6% for HD sequences. The decoding time is 106.5% of HTM-5.1 on average. The maximum bitrate savings of texture on dependent left and right views are -15.0% and -13.9%, respectively.

Table 1. FBVSP vs HTM-5.1

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | 2.2% | 0.6% | 0.4% | 0.4% | 105.2% | 106.5% |
| Kendo | 0.0% | 0.0% | 0.8% | 0.3% | 0.1% | 0.2% | 127.7% | 105.3% |
| Newspapercc | 0.0% | 1.3% | 1.5% | 0.6% | 0.6% | 0.6% | 162.5% | 108.7% |
| GhostTownFly | 0.0% | -8.3% | -8.3% | -2.0% | -1.6% | -1.8% | 101.9% | 107.5% |
| PoznanHall2 | 0.0% | -0.4% | -0.7% | -0.1% | -0.3% | -0.2% | 124.8% | 104.6% |
| PoznanStreet | 0.0% | -3.1% | -2.8% | -0.8% | -0.6% | -0.7% | 121.6% | 105.8% |
| UndoDancer | 0.0% | -15.0% | -13.9% | -4.0% | -3.7% | -3.8% | 103.6% | 107.1% |
| 1024x768 | 0.0% | 0.4% | 1.5% | 0.5% | 0.4% | 0.4% | 129.7% | 106.8% |
| 1920x1088 | 0.0% | -6.7% | -6.4% | -1.7% | -1.6% | -1.6% | 112.5% | 106.2% |
| Average | 0.0% | -3.7% | -3.0% | -0.8% | -0.7% | -0.7% | 119.6% | 106.5% |

Table 2 shows the coding performance of FBVSP vs HTM-5.1-VSP. The overall bitrate savings on the coded and synthesized views are +0.0% for XGA sequences and -0.1% for HD sequences. The decoding time is 55.0% of HTM-5.1-VSP on average.

Table 2. FBVSP vs HTM-5.1-VSP

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | 0.3% | 0.0% | 0.0% | 0.0% | 101.1% | 51.4% |
| Kendo | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 90.7% | 49.5% |
| Newspapercc | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 104.6% | 53.1% |
| GhostTownFly | 0.0% | -1.1% | -0.5% | -0.1% | 0.6% | 0.3% | 99.2% | 53.2% |
| PoznanHall2 | 0.0% | 0.3% | -1.0% | -0.2% | -0.2% | -0.2% | 116.0% | 61.2% |
| PoznanStreet | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | 103.9% | 60.9% |
| UndoDancer | 0.0% | -0.9% | -0.5% | -0.2% | -0.4% | -0.4% | 100.0% | 56.8% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 98.6% | 51.3% |
| 1920x1088 | 0.0% | -0.5% | -0.5% | -0.1% | 0.0% | -0.1% | 104.6% | 57.9% |
| average | 0.0% | -0.3% | -0.2% | -0.1% | 0.0% | -0.1% | 102.0% | 55.0% |

# Conclusion

The proposed FBVSP achieves -0.7% bitrate saving against HTM5.1 with maximum of -15.0% bitrate saving on dependent view texture coding. It reduces decoding complexity of HTM-5.1-VSP (45.0%) without coding performance penalty. We recommend adopting FBVSP into 3D-HEVC.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Yichen Zhang, Yin Zhao, and Lu Yu, “CE1.a related results on forward warping block-based VSP”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-B0121, October 2012, Shanghai, China.

[2] Feng Zou, Dong Tian, Anthony Vetro, Shinya Shimizu, Shiori Sugimoto, and Hideaki Kimata, “CE1.h Results on View Synthesis Prediction”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-B0102, October 2012, Shanghai, China.

[3] Dmytro Rusanovskyy, Kasten Muller, and Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-B1100, October 2012, Shanghai, China.