| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0091 |
| --- | --- |

| Title: | CE3.a related: Improved inside-view motion prediction for 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kwan-Jung Oh Ho-Cheon Wey Du-Sik Park San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do, 446-712, Korea Liu He Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: Tel: Email: | +82-31-280-6612 kwanjung.oh@samsung.com hc.wey@samsung.com dusikpark@samsung.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Samsung Electronics Co., Ltd, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current 3D-AVC, inside-view motion prediction (IVMP) is employed for depth coding. When one of the co-located four macroblocks in the texture view is intra-coded or inter-coded with four partitions, IVMP shall not be applied to current macroblock in the depth view. To solve this problem, JCT3V-B0146 has proposed to decide the motion information from several candidate blocks with undesirable complexity increase. As a follow-up of JCT3V-B0146, this proposal further reduces the computational complexity while keeping almost the same coding gain. Experimental results show that the proposed method achieves 0.1% coding gain for both decoded views and synthesized views.

- Introduction

Instead of disabling the IMVP when the co-located macroblock is intra coded or inter-coded with four partitions as described in Error: Reference source not found, it was proposed in JCT3V-B0146 [1] that the motion information could be derived from the co-located macroblock or its neighbouring macroblocks (MBs) in the texture view. However, the complexity increase may be too high compared to the coding efficiency improvement. In this proposal, the computational complexity is reduced compared to JCT3V-B0146, e.g., the number of checking blocks is reduced, and multiplication operation in the selection criterion is removed.

- Proposed Method

When current 8x8 MB partition in the depth view corresponds to one MB in inter mode with four partitions or intra mode, the following methods are applied to derive the motion information for current 8x8 MB partition in the depth view.

- When current 8x8 MB partition in the depth view corresponds to four 8x8 MB partitions in the texture view component, two steps are used to derive the motion vectors and reference index.

  - A predictive 8x8 MB partition of the co-located texture MB, which has the location that is closest to the center of the four co-located texture MBs in current MB, is chosen. Its reference indices are inherited by current 8x8 partition, as shown in Figure 1: Texture 8x8 MB partition to be used directly for depth partition. . An 8x8 MB partition of the depth MB with a certain color is predicted ONLY by the MB partition of texture MB in the same color. The 8x8 partitions closer to the center are chosen

  - The motion vector with the largest cost, after scaling is inherited by current 8x8 partition.

(a) Four co-located texture MBs

(b) One depth MB

Figure 1: Texture 8x8 MB partition to be used directly for depth partition.

- When current 8x8 MB partition in the depth view corresponds to one MB in Intra mode in the texture view component, the modes and motion vectors of three 4x4 blocks in each neighbouring MB of the MB in Intra mode are checked.

  - If all of the other 3 MBs are intra-coded, set the current 8x8 MB partition reference index equal to 0 and set the motion vector equal to 0 for each reference picture list.

  - If only one of the neighboring MBs is inter-coded, the reference indices and scaled motion vectors of the 4x4 block which is the most close to the center of the texture view component of this non-Intra MB is inherited by current 8x8 partition.

  - Otherwise, (more than one neighboring MB is inter-coded), get the motion vectors for each 4x4 block that is closest to the center of the texture view component from each inter-coded neighboring MB. The motion vector with the medium cost after scaling and its associated reference indices are inherited by current 8x8 partition. Here, the cost is defined as the sum of absolute values of the horizontal and vertical components of the motion vector. When the motion vector is unavailable, the cost is set to -1.

- Compression Performance

This section provides simulation results of the proposed method in comparison with the 3D-ATM anchor. The proposed method is implemented on top of 3D-ATM version 6.0 [2] and all the simulation tests are performed based on the common test conditions [4].

Table 1 provides the results of the proposed method compared with 3D-ATM anchor. The overall average bitrate reduction is 0.9%, 0.1%, 0.1% for decoded depth views, total coded views and synthesized views, respectively.

Table 1: Coding gain with respect to 3D-ATM anchor for 3-view case

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | Enc. Time % | Dec. Time % | Ren.Time % |
| S01 | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 96 | 100 | 106 |
| S02 | 0.0 | 0.0 | -0.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 100 | 101 | 109 |
| S03 | 0.0 | 0.0 | -1.9 | 0.1 | -0.1 | 0.0 | -0.3 | 0.0 | 98 | 100 | 105 |
| S04 | 0.0 | 0.0 | -2.2 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 | 99 | 101 | 103 |
| S05 | 0.0 | 0.0 | -0.5 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 | 100 | 97 | 103 |
| S06 | 0.0 | 0.0 | -0.9 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 | 99 | 100 | 128 |
| S08 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 99 | 101 | 102 |
| Aver | 0.0 | 0.0 | -0.9 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 | 99 | 100 | 108 |

- References

- M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. J. Sullivan, " 3D-AVC draft text 4," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1002, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

- K-J. Oh, J. Lee, D.-S. Park, "3D-AVC results on improved inside view motion prediction," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0146, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

- 3D-ATM version 6.0: http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv6.0/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, 2nd Meeting: Shanghai, CN, 13–19 Oct. 2012.

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).