| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0092 |
| --- | --- |

| Title: | Analysis on depth range based weighted prediction (DRWP) in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kwan-Jung Oh Ho-Cheon Wey Du-Sik Park San 14, Nongseo-dong, Giheung-gu, Yongin-si, Gyeonggi-do, 446-712, Korea | Tel: Email: | +82-31-280-6612 kwanjung.oh@samsung.com hc.wey@samsung.com dusikpark@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In the current 3D-AVC, depth range based weighted prediction (DRWP) is employed for depth coding when Znear and Zfar values are varied. However, the current DRWP shows a negative gain and convectional DC based and LMS based weighted predictions show positive gains. Thus, change of CTC is recommended.

- Recommendation

The depth range based weighted prediction (DRWP) is a kind of implicit weighted prediction method which compensates the illuminate difference by using camera parameters. It is well designed but does not show any coding gain under current SW. On the other hand, existing DC and LMS based weighted predictions show stable coding gains. Thus, change of CTC is recommended.

- Experimental Results

This section provides simulation results of the various weighted prediction methods in comparison with the 3D-ATM anchor for GT_fly sequence.

Table 1: Coding gain with respect to 3D-ATM anchor for 3-view case

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | Enc. Time % | Dec. Time % | Ren.Time % |
| DRWP off | -0.02 | 0.00 | 4.90 | -0.27 | -0.18 | 0.01 | -0.36 | 0.01 | 98.24 | 99.57 | 100 |
| DC | -0.06 | 0.00 | -1.21 | 0.07 | -0.16 | 0.01 | -0.35 | 0.01 | 99.27 | 99.25 | 100 |
| LMS | -0.18 | 0.01 | 3.09 | -0.17 | -0.30 | 0.01 | -0.81 | 0.03 | 98.85 | 100.90 | 100 |

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. does not have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).