| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0096 |
| --- | --- |

| Title: | [3D-CE6.h related results on Improved Simple Depth Coding] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Byung Tae Oh, Ilsoon Lim, Ho-cheon Wey, Du-Sik Park, | Email: | byung.oh@samsung.com ilsoon.lim@samsung.com hc.wey@samsung.com dusikpark@samsung.com |
| Source: | [Samsung Electronics Co., Ltd] |  |  |

_____________________________

# Abstract

An improvement of simple depth coding (SDC) is proposed in the proposal. The current SDC scheme uses four intra-prediction modes, which are DC, Planar, DMM1 and DMM2. However, it is experimentally shown that using only 3 modes without DMM2 is enough to cover most of prediction for depth map. Furthermore, the context model for modified SDC is also proposed in the proposal. Coding experiments for CE6.h using a version of HTM-v5.0.1 including proposed scheme have been conducted and are reported in this document. The results report that using only 3 modes does not impact the coding performance with smaller decoding complexity. When adding the proposed context-based probability model, it reports 0.1% BD-rate gain is achieved.

# Proposed algorithm – A

In current SDC design, four depth intra modes are chosen when SDC mode is on. Those modes are DC, Planar, DMM1 and DMM2. However, the functionality of DMM1 and 2 is very similar, so that it is possible to reduce the size of SDC mode with eliminating DMM2. In the proposed algorithm, the following three depth intra modes are used :

SDC = {DC, Planar, DMM1}

The binary code is similarly assigned as follows

Table 1. Binarization

|  | SDC | Simplified SDC | sdc_pred_mode |
| --- | --- | --- | --- |
| DC | 0 | 0 | 0 |
| DMM1 | 10 | 10 | 1 |
| DMM2 | 110 | - | - |
| PLANAR | 111 | 11 | 2 |

# Proposed algorithm – B

In current SDC design, it does not use the context-adaptive model. However, it is usually beneficial if the probability model based on its local neighborhood information. In the proposed scheme, it analyzes the upper and left PU, and use three bins to classify them according to its upper/left PU condition. The decision is as follows.

Table 2. Proposed context-adaptive probability model

| ctxIdx | SdcPredMode |
| --- | --- |
| 0 | ( sdc_pred_mode[ xL ][ yL ] == 0 && sdc_pred_mode[ xA ][ yA ] == 0 ) ¦¦ ( sdc_pred_mode[ xL ][ yL ] == 0 && sdc_pred_mode[ xA ][ yA ] == 2 ) ¦¦ ( sdc_pred_mode[ xL ][ yL ] == 2 && sdc_pred_mode[ xA ][ yA ] == 0 ) |
| 1 | sdc_pred_mode[ xL ][ yL ] == 1 ¦¦ sdc_pred_mode[ xA ][ yA ] == 1 |
| 2 | sdc_pred_mode[ xL ][ yL ] == 2 && sdc_pred_mode[ xA ][ yA ] == 2 |

If the left or upper PU is not SDC mode, there would be conventional 35 modes including DC, PLANAR, and directional modes. In the case, we simply assume all other directional modes (including DMM and ChainCode) to DMM1 as the following equation.

# Coding experiments

It is implemented in HTM-v5.0.1, and the comparison to anchor is given in Table 1 and 2 for algorithm A, and Table 3 and 4 for algorithm B. As a result, the method A does not impact the performance and complexity with further simplifying the previous SDC method. The method B makes the BD-BR improvement about 0.1% on average with similar complexity.

Table 3: coding results for Method A (CTC condition)

|  | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 96.8% | 93.5% |
| Kendo | -0.1% | -0.1% | 99.9% | 94.2% |
| Newspapercc | -0.1% | -0.1% | 102.0% | 96.3% |
| GhostTownFly | -0.1% | 0.0% | 97.7% | 92.4% |
| PoznanHall2 | -0.1% | -0.1% | 99.4% | 94.4% |
| PoznanStreet | 0.0% | 0.0% | 100.2% | 97.5% |
| UndoDancer | 0.0% | 0.0% | 100.6% | 97.6% |
| 1024x768 | 0.0% | -0.1% | 99.5% | 94.7% |
| 1920x1088 | 0.0% | 0.0% | 99.5% | 95.4% |
| average | 0.0% | 0.0% | 99.5% | 95.1% |

Table 4: coding results for Method A (All Intra condition)

|  | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 99.3% | 100.0% |
| Kendo | 0.1% | 0.1% | 99.9% | 96.3% |
| Newspapercc | 0.1% | 0.1% | 97.8% | 98.2% |
| GhostTownFly | 0.1% | 0.1% | 99.4% | 102.2% |
| PoznanHall2 | 0.0% | 0.0% | 99.0% | 102.2% |
| PoznanStreet | 0.0% | 0.0% | 99.2% | 99.0% |
| UndoDancer | 0.0% | 0.0% | 101.7% | 103.3% |
| 1024x768 | 0.1% | 0.1% | 99.0% | 98.2% |
| 1920x1088 | 0.0% | 0.0% | 99.8% | 101.7% |
| average | 0.0% | 0.0% | 99.5% | 100.1% |

Table 5: coding results for Method B (CTC condition)

|  | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 99.3% | 93.1% |
| Kendo | -0.1% | -0.1% | 98.6% | 91.7% |
| Newspapercc | -0.1% | -0.1% | 100.4% | 94.3% |
| GhostTownFly | -0.1% | 0.0% | 99.1% | 96.8% |
| PoznanHall2 | -0.1% | -0.1% | 98.4% | 96.1% |
| PoznanStreet | 0.0% | 0.0% | 100.3% | 95.7% |
| UndoDancer | -0.2% | -0.1% | 101.2% | 99.1% |
| 1024x768 | -0.1% | -0.1% | 99.4% | 93.0% |
| 1920x1088 | -0.1% | -0.1% | 99.7% | 96.9% |
| average | -0.1% | -0.1% | 99.6% | 95.2% |

Table 6: coding results for Method B (All Intra condition)

|  | synthesized only | coded & synthesized | enc time | dec time |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 99.3% | 98.1% |
| Kendo | 0.0% | 0.1% | 97.4% | 93.6% |
| Newspapercc | 0.0% | 0.0% | 98.2% | 93.8% |
| GhostTownFly | 0.1% | 0.1% | 97.4% | 99.1% |
| PoznanHall2 | 0.0% | 0.0% | 98.9% | 103.1% |
| PoznanStreet | 0.0% | 0.0% | 95.2% | 93.8% |
| UndoDancer | 0.0% | 0.0% | 99.6% | 99.2% |
| 1024x768 | 0.0% | 0.0% | 98.3% | 95.1% |
| 1920x1088 | 0.0% | 0.0% | 97.7% | 98.8% |
| average | 0.0% | 0.0% | 98.0% | 97.2% |

# Conclusion

The proposed modification of SDC has been suggested. Coding results for both methods have been presented.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).