| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0097 |
| --- | --- |

| Title: | 3D-CE2.h related results on disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Ho-Cheon Wey, Du-Sik Park | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

For inter-view motion parameter prediction and residual prediction, a neighboring blocks based disparity vector derivation method was integrated on the current 3DV-HTM. The disparity vectors are derived from spatially adjacent blocks and temporally collocated blocks. If the disparity vectors are unavailable, a zero vector is employed. This proposal studies what candidate is more accurate for the prediction and gives the highest priority to the most accurate candidate. Based on the study, by using the temporal candidate with the highest priority, the proposed method achieves the bit rate reduction of -0.3% and -0.1% in dependent texture views as compared with an anchor. In addition, the proposal removes a disparity vector derivation process using the DV-MCP (JCT2-A0126) to reduce the required memory.

# Algorithm Description

(Method 1) The disparity vector, which is derived by the spatially neighboring blocks and the temporally collocated blocks as displayed in Fig. 1, is employed for inter-view motion and residual prediction. First, the disparity vector is derived from the spatial candidates. If the disparity is not derived, the temporal candidates are checked.

Fig. 1. Spatially neighboring blocks and temporally collocated blocks

However, it is observed that the temporal candidates are more accurate than the spatial candidates. Table 1 represents the performance when the spatial candidates are only used in the disparity vector derivation and Table 2 shows the performance when the temporal candidates are only employed, respectively.

Table 1. Performance when the spatial candidates are only used

Table 2. Performance when the temporal candidates are only used

As shown in Tables, the temporal candidates have higher influences on the coding performance than the spatial candidates Therefore, the proposed method searches the temporal candidates in the disparity vector derivation, and then if the disparity is not derived, the spatial candidates are investigated.

(Method 2) A disparity vector can be derived from not only a disparity compensated block but a motion compensated block in dependent views. If a motion vector in the motion compensated block is obtained by the inter-view motion vector prediction (DV-MCP), it means that the motion vector is predicted from the corresponding block in the inter-view reference picture where its location is pointed by the disparity vector. Therefore, when the disparity vector in the disparity compensated blocks is not found (the spatial and temporal candidates are not available), the current method uses the disparity vector derived from DV-MCP blocks coded by inter-view motion parameter prediction as shown in Fig. 2.

However, an additional memory is required to use the DV-MCP block. The DV-MCP block should store one bit flag and the disparity vector employed for the inter-view motion parameter prediction. In order to reduce the memory size, two proposals (JCT3V-B0045, JCT3V-B135) were adopted at the last meeting. In the proposed method, the DV-CMP block is not used to remove the additional memory. Hence, If the disparity vector is not available in spatial and temporal candidates, the proposed method directly employs the zero vector.

Fig. 2. Disparity vector derivation using DV-MCP block

# Test Results

Test results are summarized in Tables 3 to 5. Table 3 is about (a) changing the disparity vector derivation order. The result shows that the proposed method slightly improves the coding performance. Table 4 is about (b) not using the DV-MCP block. The result shows that the proposed method removes the memory required to the DV-MCP block with negligible loss. Table 5 is (c) the result when (a) and (b) methods are combined.

Table 3. (a) Changing disparity vector derivation order

Table 4. (b) Not using DV-MCP block in disparity vector derivation

Table 5. Combining (a) and (b)

# Conclusion

By integrating the proposed method on the current 3DV-HTM v5.0.1, it removes the additional memory required to the DV-MCP block without the coding performance loss. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3DV-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).