| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0098 |
| --- | --- |

| Title: | 3D-CE4.h related results on simplification of residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Ho-Cheon Wey, and Du-Sik Park (Samsung) Jicheng An, Kai Zhang, Jian-Liang Lin, and Shawmin Lei (MediaTek) | Email: | jinyoung79.lee@samsung.com {jicheng.an, kai.zhang, jl.lin, Shawmin.lei@mediatek.com} |
| Source: | Samsung Electronics Co. Ltd. and MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal introduces simplification of inter-view residual prediction. The residual prediction is only performed in dependent view when the corresponding block in base view is temporally inter-coded. Since temporal and inter-view predicted residual have low correlation, the temporally predicted residual of the current block generated by the temporal prediction is only re-predicted through the inter-view residual prediction. When both temporal and inter-view predictions are performed in bi-prediction case of B slice, the residual prediction is half enabled. However, the proposed method disables the residual prediction to avoid some multiplication operation, when the prediction types are different in the bi-prediction case. In addition, the proposed method removes an interpolation process in the residual prediction to reduce the complexity. The proposed method simplifies the residual prediction without the coding loss.

# Algorithm Description

Through inter-view residual prediction, residual of the current block in dependent view can be predicted by reference residual of its corresponding block in base view. The corresponding block is derived, based on disparities of neighboring blocks around the current block. It has been improved that the temporally predicted residual of the current block generated from the temporal prediction is only re-predicted when the corresponding block is temporally inter-coded (M24766). Therefore, the inter-view residual prediction is only performed in the temporal prediction of the current block in the current software.

When both the temporal prediction and the inter-view predictions are performed in bi-prediction case of B slice as shown in Fig. 1, the residual prediction is half enabled. The half enabling means that the reference residual is multiplied by 1/2, which is unnecessary calculation in the residual prediction. Also, we observe that the half enabling does not give any influence on the performance. Therefore, the proposed method disables the residual prediction when two different prediction types are used.

In addition, the proposed method removes an interpolation process in the residual prediction to reduce the complexity. If the disparity vector of the current prediction unit points to a sub-sample location in the reference view, the residual prediction signal is obtained by rounding the sub-sample location to the nearest integer location in the reference view.

Fig. 1. Bi-prediction that both temporal and inter-view predictions are performed

# Test Results

Test results are summarized in Tables 1 to 3. Table 1 is about (a) disabling the residual prediction when both the temporal and inter-view predictions are performed. Table 2 is about (b) removing the residual interpolation. Table 3 is (c) the result when (a) and (b) methods are combined. The results show that the proposed method simplifies the residual prediction without the coding loss.

Table 1. (a) Disabling the residual prediction when temporal and inter-view predictions are performed.

Table 2. (b) Removing residual interpolation

Table 3. Combining (a) and (b)

# Conclusion

By integrating the proposed method on the current 3DV-HTM v5.0.1, it simplifies the inter-view residual prediction without the coding performance loss. Since the modifications are simple, it is recommended to adopt the proposed method to the next 3DV-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).