| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0101r1 |
| --- | --- |

| Title: | AHG11: 3D Test Materials from NICT-3D data set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

In this document, a set of stereo or multiview sequences is proposed for usage as test sequences in the development of 3D video coding technologies. These sequences are distributed from NICT for the purpose of research and development activities.

# Introduction

NICT kindly provides many stereo and multiview sequences, NICT-3D, for the research and development of 3D video technologies and devices in 2010. We confirmed that these sequences can be used free of charge for non-commercial purposes. More detailed guideline is attached in this contribution.

# Test Sequences

NICT-3D includes four kinds of 3D sequences; 4K3D stereoscopic, 3 view + 1 depth, dense scanned multiview, and 3D CG. Since dense scanned multiview sequences are static pictures, the other three types of sequences are introduced.

## 4K3D stereoscopic

4K3D sequences were captured by using RED Camera Stereo Rig. There are three sequences in this categories; indoor, outdoor, and digest. The spatial resolution is 3840x2160, and each sequence is more than 3 minute length with many scene cuts. In order to evaluate many types of stereo settings, each scene cut has different camera settings, convergence vs. parallel, narrow baseline vs. wide baseline, static vs. dynamic camera motion, and so on.

Figure 1: 4K3D sequences: indoor (left) and outdoor (right)

## 3 view + 1 depth (3DTOF)

3DTOF sequences were captured by depth camera system named Axi-Vision [1]. There are 4 sequences in this category; living, office, dartsbar, and dance. Figure 2 shows the sample frame of each sequence and the characteristics of the typical scene cut are summarized in TABLE 1. The spatial resolution is 1920x1080 interlaced, and each sequence includes several scene cuts. Since depth maps are captured by a kind of TOF camera, there are a lot of noises. The camera parameters are not available, but checkerboard and color palette are captured for each cut in order to extract these parameters if necessary.

(a) living

(b) office

(c) dartsbar

(d) dance

Figure 2: 3D TOF sequences: center (left) and depth (right)

TABLE 1: Typical characteristics of the scenes

| Scene | Characteristics |
| --- | --- |
| living | Two people with small movements |
| office | Three people with large movements |
| dartsbar | Illumination changes, camera movements, reflections |
| dance | There people with fast motions and occlusions |

## 3D CG

3D CG sequences are provided in the MAYA 2010 Full CG data set. Therefore, any spatial resolution and any number of cameras can be rendered. Depth map and camera parameters can also be obtained. 6 data sets are provided. Figure 3 shows a sample frame of each sequence. The default camera settings contain camera motions including dynamic convergence angles and varying baseline distances although they can be changed on the rendering settings. There are many motion, specular, reflections, and so on. Default camera settings and typical scene characteristics are summarized in TABLE 2.

(a) Shark                                 (b) Micro World

(c) Robot                                  (d) Museum

(e) Wonder World                                (f) Park

Figure 3: 3D CG sequences

TABLE 2: Default camera settings and scene characteristics

| Sence | Camera arrangement | Dynamic changes on camera arrangement | Characteristics |
| --- | --- | --- | --- |
| Shark | Parallel | None | Fluctuating caustics, several moving objects, small marine snows and bubbles, underwater |
| Micro World | Convergence | Convergence angle, baseline distance | Many small moving objects, flight though, dynamic Zfar/Znear |
| Robot | Convergence | Convergence angle, zooming | Reflection, specular, scene change, flight though, dynamic Zfar/Znear |
| Museum | Convergence | Convergence angle, baseline distance, zooming | Many moving transparent objects (particles), specular, several scene cuts |
| Wonder World | Parallel | Baseline distance | Many moving objects, reflection, specular, transparent objects, object deformation |
| Park | Parallel | none | Many moving objects with occlusions, walk though |

# Conclusions

This contribution reported the publically available 3D test materials, NICT-3D. Many interesting characteristics can be observed in these sequences. We recommend including these sequences as 3DV test sequences.

# References

- Masahiro Kawakita, “Axi-Vision Camera: Real-time depth-mapping camera,” ISO/IEC JTC1/SC29/WG11 M14949, October 2007, Shenzhen, CN.