| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0101 |
| --- | --- |

| Title: | AHG11: 3D Test Materials from NTCT-3D data set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

In this document, a set of stereo or multiview sequences is proposed for usage as test sequences in the development of 3D video coding technologies. These sequences are distributed from NICT for the purpose of research and development activities.

# Introduction

NICT kindly provides many stereo and multiview sequences, NICT-3D, for the research and development of 3D video technologies and devices in 2010. We confirmed that these sequences can be used free of charge for non-commercial purposes. More detailed guideline is attached in this contribution.

# Test Sequences

NICT-3D includes four kinds of 3D sequences; 4K3D stereoscopic, 3 view + 1 depth, dense scanned multiview, and 3D CG. Since dense scanned multiview sequences are static pictures, the other three types of sequences are introduced.

## 4K3D stereoscopic

4K3D sequences were captured by using RED Camera Stereo Rig. There are three sequences in this categories; indoor, outdoor, and digest. The spatial resolution is 3840x2160, and each sequence is more than 3 minute length with many scene cuts. In order to evaluate many types of stereo settings, each scene cut has different camera settings, convergence vs. parallel, narrow baseline vs. wide baseline, static vs. dynamic camera motion, and so on.

Figure 1: 4K3D sequences: indoor (left) and outdoor (right)

## 3 view + 1 depth (3DTOF)

3DTOF sequences were captured by depth camera system named Axi-Vision [??]. There are 4 sequences in this category; living, office, dartsbar, and dance. The spatial resolution is 1920x1080 interlaced, and each sequence includes several scene cuts. Since depth maps are captured by a kind of TOF camera, there are a lot of noises. The camera parameters are not available, but checkerboard and color palette are captured for each cut in order to extract these parameters if necessary.

(a) living

(b) office

(c) dartsbar

(d) dance

Figure 2: 3D TOF sequences: center (left) and depth (right)

## 3D CG

3D CG sequences are provided in the MAYA 2010 Full CG data set. Therefore, any spatial resolution and any number of cameras can be rendered. Depth map and camera parameters can also be obtained. 6 data sets are provided. The default camera settings contain camera motions including dynamic convergence angles and varying baseline distances. There are many motion, specular, reflections, and so on.

(a) Shark                                 (b) Micro World

(c) Robot                                  (d) Museum

(e) Wonder World                                (f) Park

Figure 3: 3D CG sequences

# Conclusions

This contribution reported the publically available 3D test materials, NICT-3D. Many interesting characteristics can be observed in these sequences. We recommend to include these sequences as 3DV test sequences.

# References

- …