| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0106 |
| --- | --- |

| Title: | AHG7: Video parameter set extension design for MV-HEVC and 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Min Woo Park Jaewon Yoon Chanyul Kim Junghoon Park | Tel: Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Two video parameter set (VPS) designs are proposed for MV-HEVC and 3D-HEVC. The first design is to remove duplicated syntax signalling among views. A flag is proposed to indicate whether the same parameter sets (SPS, PPS) can be shared among view components of pictures by identical parameter set ids or not. In the current MV-HEVC HLS design, no special change of SPS or PPS for non-base views is present. So, the parameter sets of a base view can be shared by other non-base views. Even though the parameter sets are signalled independently without sharing among views, the common syntaxes of sequence parameter sets of view can be predicted between views, or signalled once in a VPS. To enable this SPS syntax prediction, two flags are proposed to indicate whether the common syntaxes of SPSs in multi-layers are signalled only in VPS once, or whether the syntaxes of a specific SPS is predicted from a directly referred SPS. The second design is a minor change for 3D-HEVC. Instead of fixing the view and depth layer_id orders, the view or depth is discriminated by using the scalability_mask (or scalability_flag) and layer_ids.

# Proposed VPS design for inter-view parameter set and slice header sharing or prediction

In order to enable the parameter set sharing or the syntax prediction among views, we proposed a modified VPS design for MV-HEVC. By the proposed VPS design and the indication of parameter set properties, the duplicated signalling of syntaxes in parameter sets can be minimized.

A flag vps_paramter_set_sharing_disable_flag is proposed to indicate whether the same parameter sets (SPS, PPS) can be shared among view components of pictures by identical parameter set ids or not. In the current MV-HEVC HLS design, no special change of SPS or PPS for non-base views is present. So, the parameter sets of a base view can be shared by other non-base views.

When vps_paramter_set_sharing_disable_flag is equal to 1, the layer_ids of all sequence parameter sets and picture parameter sets are equal to 0, and the sequence parameter sets and the picture parameter sets of a base view can be used by non-base views, of which layer_ids are not equal to 0.

When vps_paramter_set_sharing_disable_flag is equal to 0, the layer_ids of sequence parameter sets and picture parameter sets are equal to the layer_ids of their relevant view components. The sequence parameter sets and the picture parameter sets of a view cannot be used by other views.

In MV-HEVC, when the SPSs of each view are transmitted independently, some common syntaxes of sequence parameter sets (SPSs) are redundant between views. An efficient signalling method of duplicated syntaxes in multi-layered SPSs is that the overlapped syntaxes are signalled in a VPS or an SPS once, and other SPS are predicted from VPS or a reference SPS.

In this contribution, two flags, vps_sps_common_syntax_present_flag and vps_sps_interlayer_prediction_enable_flag, are proposed to indicate whether the common syntaxes of SPSs in multiple views are signalled only in VPS once, and whether the syntaxes of a specific SPS is predicted from a directly referred SPS, respectively.

When vps_sps_common_syntax_present_flag is equal to 1, the common syntaxes of sequence paremter sets of non-base views are signalled once in a video parameter set. The common SPS syntaxes of non-base views are prediced from the video paramter set.

When vps_sps_interlayer_prediction_enable_flag is equal to 1, the common syntaxes of the sequence parameter sets of non-base views can be predicted from the sequence parameter sets of the directly referred views.

Otherwise, the common syntaxes of SPS are signalled in each SPS independently without prediction or sharing.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i<NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i<= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_paramter_set_sharing_disable_flag | u(1) |
| vps_sps_common_syntax_present_flag | u(1) |
| if(!vps_sps_common_syntax_present_flag) |  |
| vps_sps_interlayer_prediction_enable_flag | u(1) |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j <num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| if(vps_sps_interlayer_prediction_enable_flag) { |  |
| interlayer_sps_prediction_flag[ i ] | u(1) |
| if(interlayer_sps_prediction_flag[ i ]) |  |
| sps_prediction_ref_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| If(!vps_sps_common_syntax_present_flag) { |  |
| // common syntaxes of sequence parameter sets |  |
| … |  |
| } |  |
| } |  |

vps_paramter_set_sharing_disable_flag equal to 0 specifies that the same sequence parameter sets and the same picture parameter sets can be shared among views. vps_paramter_set_sharing_disable_flag equal to 1 specifies that the same sequence parameter sets and the same picture parameter sets cannot be shared among views.

When vps_paramter_set_sharing_disable_flag is equal to 1, the layer_ids of all sequence parameter sets and picture parameter sets are equal to 0, and the sequence parameter sets and the picture parameter sets of a base view can be used by non-base views, of which layer_ids are not equal to 0.

When vps_paramter_set_sharing_disable_flag is equal to 0, the layer_ids of sequence parameter sets and picture parameter sets are equal to the layer_ids of their relevant view components. The sequence parameter sets and the picture parameter sets of a view cannot be used by other views.

# A minor change of VPS for 3D-HEVC

The second design is a minor change for 3D-HEVC. Instead of fixing the view and depth layer_id orders, the view or depth is discriminated by using the scalability_mask (or scalability_flag) and layer_ids. The current VPS design for 3D-HEVC assumes the fixed orders of views and depths, and does not consider the combination with other scalabilities. For a more clean and flexible VPS syntax , we propose an editorial change on the VPS design for 3D-HEVC, as follows. With the proposed VPS syntax, the flexible ordering between views and depths is enabled.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i<NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i<= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j <num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| if(scalability_mask&0x4) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if ( !dimension_id[ i ][ 3 ] ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if (dimension_id[ i ][ 3 ]) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j< num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

scalability_mask signals a pattern of 0 and 1 bits with each bit corresponding to one scalability dimension as indicated by the table below. A value of 1 for a particular scalability dimension indicates that this scalability dimension is present. A value of 0 for a particular scalability dimension indicates that this scalability dimension is not present. The values of NumScalabilityTypes is equal to the sum of number of bits in the scalability_mask having value of 1. Thus

Table – Mapping scalablity_mask to scalbility dimensions

| scalability_mask | Scalability dimension |
| --- | --- |
| 0 | Spatial |
| 1 | Quality |
| 2 | Multiview |
| 3 | depth |
| 4-15 | reserved |

NOTE- scalability_mask & 0x8 && !scalability_mask & 0x4 shall be 0.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0.

num_direct_ref_layers[ i ] specifies the number of layers the i-thlayer directly depends on. The inter-view subset reference picture set of the view components with layer_id equal to i contains num_direct_ref_layers[ i ] entries. When not present, the value of num_direct_ref_layers[ i ] is inferred to be equal to 0.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-thlayer directly depends on. The j-th entry of the inter-view subset reference picture set of the view components with layer_id equal to i is identified by ref_layer_id[ i ][ j ].

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).