| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0110 |
| --- | --- |

| Title: | 3D-CE4: Cross-check on advanced residual prediction for multiview coding(JCT3V-C0049) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Taesup Kim Sehoon Yea #221, Yangjae-dong, Seocho-gu, Seoul, 137-130, Korea | Tel: Email: | 82-2-6912-6477 taesup.kim@lge.com sehoon.yea@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Introduction

This document reports the cross check results on ‘3D-CE4: Advanced residual prediction for multiview coding (JCT3V-C0049)’ by Qualcomm[1]. Performance evaluation results reported by the proponent were verified.

# Test results

The tests were finished on a 64bit Cent OS Linux 5.6 cluster system, based on the common test conditions [2]. Table 1 and 2 show the test results of 3-view and 2-view cases. The BD-rate from our experimental results matches exactly those provided by Qualcomm.

Table 1. Rate-distortion results on 3-view coding with depth (anchor : HTM 5.0.1 CTC)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.8% | -3.4% | -1.3% | -0.9% | -1.0% |
| Kendo | 0.0% | -3.8% | -4.0% | -1.5% | -0.8% | -1.0% |
| Newspapercc | 0.0% | -3.0% | -2.2% | -1.0% | -0.6% | -0.7% |
| GhostTownFly | 0.0% | -1.7% | -1.7% | -0.6% | -0.4% | -0.5% |
| PoznanHall2 | 0.0% | -0.7% | -0.4% | -0.2% | -0.2% | -0.2% |
| PoznanStreet | 0.0% | -3.8% | -3.4% | -1.1% | -0.8% | -0.9% |
| UndoDancer | 0.0% | -1.8% | -1.7% | -0.5% | -0.4% | -0.4% |
| 1024x768 | 0.0% | -3.5% | -3.2% | -1.3% | -0.8% | -0.9% |
| 1920x1088 | 0.0% | -2.0% | -1.8% | -0.6% | -0.5% | -0.5% |
| average | 0.0% | -2.7% | -2.4% | -0.9% | -0.6% | -0.7% |

Table 2. Running time ratio (anchor : HTM 5.0.1 CTC)

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 103.9% | 91.0% | 105.8% |
| Kendo | 101.4% | 90.9% | 106.5% |
| Newspapercc | 106.4% | 89.0% | 97.0% |
| GhostTownFly | 102.3% | 92.6% | 105.9% |
| PoznanHall2 | 110.9% | 90.8% | 108.0% |
| PoznanStreet | 109.1% | 92.7% | 115.1% |
| UndoDancer | 107.0% | 93.5% | 113.7% |
| 1024x768 | 103.9% | 90.3% | 103.0% |
| 1920x1088 | 107.3% | 92.4% | 110.6% |
| average | 105.8% | 91.5% | 107.3% |

Table 3. Rate-distortion results on 3-view coding with depth (anchor : CE4 anchor)

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.9% | -5.6% | -2.2% | -1.7% | -1.8% |
| Kendo | 0.0% | -5.7% | -6.1% | -2.3% | -1.6% | -1.7% |
| Newspapercc | 0.0% | -4.5% | -3.5% | -1.5% | -1.1% | -1.2% |
| GhostTownFly | 0.0% | -2.2% | -2.1% | -0.7% | -0.5% | -0.6% |
| PoznanHall2 | 0.0% | -2.0% | -1.7% | -0.8% | -0.6% | -0.6% |
| PoznanStreet | 0.0% | -4.4% | -4.3% | -1.3% | -1.1% | -1.1% |
| UndoDancer | 0.0% | -2.7% | -2.7% | -0.8% | -0.6% | -0.6% |
| 1024x768 | 0.0% | -5.4% | -5.1% | -2.0% | -1.5% | -1.6% |
| 1920x1088 | 0.0% | -2.8% | -2.7% | -0.9% | -0.7% | -0.7% |
| average | 0.0% | -3.9% | -3.7% | -1.4% | -1.0% | -1.1% |

Table 4. Running time ratio (anchor : CE4 anchor)

|  | enc time | dec time | ren time |
| --- | --- | --- | --- |
| Balloons | 73.7% | 95.3% | 109.3% |
| Kendo | 71.8% | 90.5% | 106.7% |
| Newspapercc | 73.7% | 90.0% | 100.6% |
| GhostTownFly | 70.3% | 95.0% | 105.6% |
| PoznanHall2 | 72.5% | 92.4% | 110.2% |
| PoznanStreet | 74.0% | 93.4% | 117.9% |
| UndoDancer | 73.1% | 93.3% | 112.5% |
| 1024x768 | 73.1% | 91.9% | 105.5% |
| 1920x1088 | 72.5% | 93.5% | 111.5% |
| average | 72.7% | 92.8% | 108.8% |

# Conclusion

The cross check results on ‘3D-CE4: Advanced residual prediction for multiview coding (JCT3V-C0049)’ by Qualcomm were matched to those provided by the proponent.

# References

[1] L. Zhang, Y. Chen, X. Li, M. Karczewicz, “3D-CE4: Advanced residual prediction for multiview coding”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0049, Jan. 2013

[2] D. Rusanovskyy, K. Mueller, A. Vetro, " Common Test Conditions of 3DV Core Experiments", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, Oct. 2012