- The section G8.5.2.2.5.1 of Draft Text in JCT3V-C0152 should be modified as following description.

          - Depth picture retrieval process

- Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view identifier refViewIdx specifying a reference view,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit.

- variable blockW and blockH specifying the conversion precision of the corresponding depth samples.

- Outputs to this process are:

- a (nPSW)x(nPSH) array depthPicture samples of depth picture sample values.

- This process is invoked only if the depth reference in view refViewIdx is available.

Let refDepthSamples be the (nPSW)x(nPSH) array of reconstructed depth samples with ViewIdx equal to refViewIdx. . The luma location (xd0, yd0) of top-left luma sample of refDepthSamples is derived by

xd0 = Clip3( 0, pic_width_in_luma_samples – 1, xP + ( ( mvDisp[0] + 2 ) >> 2 ) )	(G-1)
yd0 = Clip3( 0, pic_height_in_luma_samples – 1, yP + ( ( mvDisp[1] + 2 ) >> 2 ))	(G-2)

The luma location (xd1, yd1) of bottom-right luma sample of refDepthSamples is derived by

xd1 = Clip3( 0, pic_width_in_luma_samples – 1, xP + ( ( nPSW + ( ( mvDisp[0] + 2 ) >> 2 ) )	(G-3)
yd1 = Clip3( 0, pic_height_in_luma_samples – 1, yP + ( ( nPSH + ( ( mvDisp[1] + 2 ) >> 2 ) )	(G-4)

- If vsp_precision is equal to 0, set depthPicture equal to refDepthSamples, and depthPicture is returned and the current process terminated.

- Otherwise, the following process applies.

for(j = yd0; j < yd1; j += blockH ) {
  for(i = xd0; i < xd1; i += blockW ) {
    depthMax = INT_MIN
    xd2 = Clip3(0, pic_width_in_luma_samples – 1, i + blockW – 1)
    yd2 = Clip3(0, pic_height_in_luma_samples – 1, j + blockH – 1)
    DepthSubSample[0] = refDepthSamples[ i, j ]
    DepthSubSample[1] = refDepthSamples[ i, yd2]
    DepthSubSample[2] = refDepthSamples[xd2, j ]
    DepthSubSample[3] = refDepthSamples[xd2, yd2]
    DepthSubSample[4] = refDepthSamples[ (i + xd2)>> 1, (j + yd2)>> 1]
    for (n = 0; n< 5; n++) { 
        if (depthMax < DepthSubSample[n])
            depthMax = DepthSubSample[n]
    }

- for(l = j-yd0; l < j-yd0 + blockH; l ++) {
      for(k = i-xd0; k < i-xd0 + blockW; k ++) {
        depthPicture[ k, l ] = depthMax
      }
    }
  }
}