| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0114 |
| --- | --- |

| Title: | 3D-CE1.a and CE2.a related: synthesized disparity vectors for BVSP and DMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Yichen Zhang, Lu Yu #38 Zheda Road Zhejiang Univ. Hangzhou, China | Tel: Email: | +86-571-87953107 zhaoyin@zju.edu.cn felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

In ATM6, BVSP and DVMP derive disparity vectors (from a dependent view to a reference view) based on reconstructed depth map of the dependent view, which results in the fact that BVSP and DVMP currently cannot work in the texture-first coding order for dependent views. In this proposal, a 4x4-block-based disparity vector field is synthesized from the depth map of a reference view to provide the disparity vectors required by BVSP and DMVP. More specifically, for each 4x4 texture block in the reference view, one depth value X is obtained as the maximum of the depth values associated with four corners of the block. The corresponding 4x4 block in the current-view texture is then located by the disparity vector D converted from X, which results in a 4x4-block-based disparity vector field. The synthesis process utilizes two low-complexity techniques, 1) one-loop warping and hole filling and 2) directional warping without z-buffering. For BVSP, the disparity vector for each 4x4 texture block in current view is derived as a corresponding synthesized disparity vector (i.e., BVSP uses 4x4-based disparity compensation). For DMVP, the disparity vector for an MxN texture block is derived as the maximum of the disparity vectors of 4x4 blocks at the four corners of the MxN texture block. With this technique, BVSP and DMVP can work in the texture-first coding order, which makes texture-first coding order now achieve -0.30% BD rate (in terms of synthesized PSNR vs. total rate) compared with ATM 6.0 CTC anchor (using depth-first coding order). The encoding and decoding time is 100% and 103% of the ATM 6.0 CTC anchor.

# Introduction

Depth-first coding order (DFCO) is utilized for dependent views in CTC anchor, and ATM supports flexible coding order as well. In ATM 6.0, BVSP and DVMP derive disparity vectors (from a dependent view to a reference view) based on reconstructed depth map of the dependent view. This results in the fact that Block-based VSP (BVSP) and Depth-based Motion Vector Prediction (DMVP) cannot work in the texture-first coding order for dependent views (TFCO), since the reconstructed depth map is not available prior to the texture coding.

It was reported in JCT3V-B0121 that with BVSP and DMVP turned off, ATM configured in TFCO (i.e., T0D0T1D1T2D2) suffered from about 18% coding performance loss. In view of this, we study the possibility of enabling BVSP and DMVP in TFCO, and figure out a method to make up the performance gap between TFCO and DFCO.

In JCT3V-B0103, NTT proposed a technique of using synthesized depth map to derive disparity vectors required by BVSP. Inspired by this proposal, we propose another disparity vector derivation method of much lower complexity to support both BVSP and DMVP in TFCO, which synthesizes a 4x4-block-based disparity vector (DV) field from a reference-view depth map. The synthesis of DV field uses two low-complexity techniques, 1) the one-loop warping and hole filling and 2) directional warping without z-buffering. More details are provided in Section 2. Compared with synthesis of a texture image, the synthesis of a DV field is of far less complexity.

With the synthesized DV field, BVSP uses 4x4-block-based disparity compensation (i.e., DVs for pixels in a 4x4 block are the same), and directly obtains a DV from the synthesized DV field for each 4x4 block. For DMVP, each MxN target block selects the maximum disparity vector of four corner 4x4 blocks covered by the MxN block, as illustrated in Fig. 1, which is similar to the approach of DV from maximum depth of four corners of the MxN block in ATM 6.0 (when depth is available).

This DV derivation approach could serve as a complementary method to derive DV for BVSP and DMVP when reconstructed depth is not available (in addition to current DV derivation), as illustrated in Fig. 2.

Fig. 1. Deriving disparity vector for an MxN target block by the maximum DV of 4x4 blocks covered by the MxN block for DMVP. If the target block is smaller than 8x8 (i.e., 8x4, 4x8 and 4x4), only 2 or 1 DV is accessed.

Fig. 2. DV derivation approaches for BVSP and DMVP in DFCO (original method in ATM 6.0) and TFCO (proposed method).

# Synthesis of disparity vector (DV) field

The reference-view image and current-view image are segmented into 4x4 blocks. Here, we first discuss the DV field synthesis with a reference view on the left of the current view, as illustrated in Fig. 3. For each block in the reference view, the maximum depth of four corners of a 4x4 block is set as the depth of the block. The maximum depth is converted to a DV (in quarter-pel precision), and a corresponding 4x4 block in the target view is located (e.g., DV1 block), whose DV is set as that of the reference block (with an opposite sign such that the DV is from the target view to the reference view). The DV derivation process can be considered as “warping” of a 4x4 block-based disparity vector field of the reference view to the target view, where only a disparity vector is copied for a 4x4 block.

The warping process is from left to right to avoid z-buffering (similar to that in VSRS-1D-fast), and follows the spirit of one-loop warping and hole filling. If two neighboring reference blocks correspond to two non-adjacent blocks in the current view with a disocclusion hole in between (e.g., DV2 and DV3 blocks), the DV of the hole block(s) is set as the background-side DV. After processing blocks in a line of the reference view, the last block with DV in the reference view is figured out (e.g., DV4 block), and the DV for each block at the right side of the last block is set to be that of the last block.

For a reference view at the right-side of the current view, the warping direction is from right to left to avoid z-buffering. In this case, the depth map of the reference view can be horizontally flipped, and then the above-mentioned “warping” process from left to right can be applied to derive a horizontally reversed DV field. Finally, horizontally flip the intermediate DV field.

Note that DV synthesis process of each block line is conducted independently, and thus synthesis of multiple lines of DVs can be run in parallel. On the other hand, considering a WxH texture image, the warping of block-based DVs should be only performed for WxH/16 times, and thus warping DV is of lower complexity than warping the depth map as proposed in JCT3V-B0103. Moreover, the depth-to-disparity conversion is not needed when the synthetic disparity vector field is available.

Fig. 3. Illustration of the generation of disparity vector field when the reference view is at the left side of the target view. Based on the maximum depth of four corners of a 4x4 block in the reference view, a corresponding 4x4 block in the target view is located (e.g., DV1 block), whose DV is set as that of the reference block. If two neighboring reference blocks correspond to two non-adjacent blocks (e.g., DV2 and DV3 blocks), the DV of the hole block(s) is set as the background-side DV. After processing blocks in a line of the reference view, the last block with DV in the reference view is figured out (e.g., DV4 block), and the DV for each block at the right side of the last block is set to be that of the last block.

# Experimental results

The coding results of BVSP and DMVP with proposed disparity vector derivation in TFCO compared with ATM 6.0 CTC anchor (in depth-first coding order with QC_GVSP_B0054 = 0) is provided in Table 1. It is shown that the texture coding performance is slightly worse (0.37% BD bitrate) than CTC anchor on average, but the depth coding performance is improved (-3.48% BD rate) partly due to applying IVMP at two dependent views (in DFCO, IVMP is only applied at the base view). Besides, with reconstructed texture samples being available, depth RDO can estimate the synthesis distortions more accurately. Therefore, owing to more efficient coding of depth data in TFCO, -0.30% BD bitrate is achieved in terms of synthesized PSNR vs. total rate. Among 7 test sequences, coding gains are observed in 5 sequences, and the two animation sequences (S03 Dancer, and S04 GTfly) with good depth map achieve the highest gain of -1.06% and -0.93%, respectively. The encoding and decoding time are 100% and 103% of the CTC anchor. Therefore, it is shown that the TFCO with proposed synthetic disparity performs as well as the DFCO, in terms of both coding efficiency and complexity.

The proposed method can also be applied in DFCO (replacing the original DV derivation in ATM 6.0), and the performance comparison with CTC anchor is listed in Table 2. Competitive coding results are also observed for texture coding (0.14% BD rate) and synthesis quality (0.30% BD rate).

It is also tested that without the proposed disparity derivation, the TFCO with DVMP and BVSP off suffers from a great coding loss of 19.20% BD-rate, as listed in Table 3. A summary of the above-mentioned coding performance (in terms of synthesis PSNR vs. total rate) is shown in Fig. 4.

The algorithm is also implemented on ATMv6.1, and similar coding performance is achieved, as shown in Table 4 to Table 6 in the Appendix, respectively.

Fig. 4 Summary of coding efficiency comparison in terms of synthesized PSNR vs. total bitrate (ATM 6.0)

Table 1. ATM 6.0 with BVSP and DMVP using proposed DV derivation in TFCO vs. ATM 6.0 CTC anchor (cross-checked)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.03 | 0.00 | -2.52 | 0.17 | -0.12 | 0.00 | -0.56 | 0.02 | 99.93 | 104.11 | / |
| S02 | -0.07 | 0.00 | -3.24 | 0.15 | -0.44 | 0.01 | -0.37 | 0.01 | 100.13 | 104.37 | / |
| S03 | -0.37 | 0.01 | -1.38 | 0.11 | -0.35 | 0.01 | -1.06 | 0.03 | 101.33 | 102.59 | / |
| S04 | -0.04 | 0.00 | -6.25 | 0.36 | -0.26 | 0.01 | -0.93 | 0.03 | 98.22 | 102.23 | / |
| S05 | 1.54 | -0.07 | -1.99 | 0.09 | 0.96 | -0.04 | 0.72 | -0.03 | 99.63 | 102.74 | / |
| S06 | 1.22 | -0.06 | -6.35 | 0.30 | 0.40 | -0.02 | 0.51 | -0.02 | 100.92 | 103.92 | / |
| S08 | 0.28 | -0.01 | -2.62 | 0.11 | -0.37 | 0.02 | -0.39 | 0.01 | 100.10 | 104.37 | / |
| Average | 0.37 | -0.02 | -3.48 | 0.19 | -0.03 | 0.00 | -0.30 | 0.01 | 100.04 | 103.48 | / |

Table 2. ATM 6.0 with BVSP and DMVP using proposed DV derivation in DFCO vs. ATM 6.0 CTC anchor (cross-checked)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.31 | 0.01 | 0.00 | 0.00 | -0.28 | 0.01 | -0.08 | 0.00 | 98.79 | 102.65 | / |
| S02 | -0.26 | 0.01 | 0.00 | 0.00 | -0.22 | 0.01 | 0.00 | 0.00 | 99.10 | 102.71 | / |
| S03 | -0.46 | 0.02 | 0.00 | 0.00 | -0.42 | 0.01 | -0.16 | 0.01 | 101.94 | 101.26 | / |
| S04 | -0.14 | 0.00 | 0.00 | 0.00 | -0.13 | 0.00 | 0.05 | 0.00 | 99.54 | 100.86 | / |
| S05 | 1.24 | -0.06 | 0.00 | 0.00 | 1.12 | -0.05 | 1.14 | -0.05 | 104.84 | 100.56 | / |
| S06 | 0.91 | -0.04 | 0.00 | 0.00 | 0.78 | -0.04 | 1.05 | -0.05 | 100.19 | 102.41 | / |
| S08 | -0.04 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 | 0.08 | 0.00 | 101.42 | 103.36 | / |
| Average | 0.14 | -0.01 | 0.00 | 0.00 | 0.11 | -0.01 | 0.30 | -0.01 | 100.83 | 101.97 | / |

Table 3. ATM 6.0 in TFCO with BVSP and DMVP off vs. ATM 6.0 CTC anchor (informative)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 40.75 | -1.09 | -2.72 | 0.18 | 36.76 | -1.01 | 31.50 | -0.89 | 97.66 | 94.20 | / |
| S02 | 12.61 | -0.35 | -6.07 | 0.28 | 10.96 | -0.31 | 9.11 | -0.27 | 97.68 | 95.21 | / |
| S03 | 22.22 | -0.70 | -4.90 | 0.40 | 20.41 | -0.66 | 15.60 | -0.46 | 99.27 | 92.55 | / |
| S04 | 28.60 | -0.92 | -11.16 | 0.67 | 26.40 | -0.86 | 20.92 | -0.65 | 96.09 | 89.74 | / |
| S05 | 27.79 | -1.19 | -2.59 | 0.12 | 22.57 | -0.94 | 20.93 | -0.80 | 100.76 | 94.77 | / |
| S06 | 33.52 | -1.48 | -6.68 | 0.32 | 28.82 | -1.26 | 25.53 | -1.04 | 100.77 | 95.14 | / |
| S08 | 15.09 | -0.61 | -2.99 | 0.13 | 12.09 | -0.48 | 10.82 | -0.37 | 99.27 | 97.53 | / |
| Average | 25.80 | -0.90 | -5.30 | 0.30 | 22.57 | -0.79 | 19.20 | -0.64 | 98.78 | 94.16 | / |

# Discussion and conclusion

The proposed method mitigates the coding efficiency gap between TFCO and DFCO, with slight complexity increase. Now, the high-efficiency TFCO brings some benefits for the 3D-AVC standard.

1) Allowing more flexible coding order (i.e., supporting TFCO in addition to DFCO) brings more room for encoding optimization (such as depth RDO) and may support more applications.

2) TFCO support unpaired MVD (some texture may not have depth) more efficiently. Unpaired MVD data were commonly agreed as important application scenarios in 2nd JCT-3V meeting. For unpaired MVD, it is clear that TFCO with proposed DV derivation achieves significant coding gain for the dependent-view texture, compared with DFCO (with DMVP and BVSP turned off).

3) TFCO leads to less decoding complexity if only N-view texture is needed at a decoder. When the dependent-view depth is not needed at a decoder, the decoder can skip decoding of the bits for the dependent-view depth in TFCO bitstream, while the decoder still need perform the decoding of dependent-view depth in DFCO bitstream prior to the texture decoding.

Therefore, we recommend adopting this alternative disparity vector derivation method, which improves TFCO performance significantly and make 3D-AVC standard suitable for more applications.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The algorithm is implemented on ATMv6.1 anchor (with QC_GVSP_B0054 = 1), and the performance of proposed DV derivation in TFCO and DFCO are tested, as listed in Table 4 and Table 5. Besides, the performance of ATM6.1 in TFCO with BVSP and DMVP off is provided in Table 6. Two extra simulations are done to test the coding performance of DMVP or BVSP individually in TFCO with proposed disparity vector derivation, as listed in Table 7 and Table 8.

A summary of the above-mentioned coding performance (in terms of synthesis PSNR vs. total rate) is shown in Fig. 5. Note that the anchor is ATMv6.1 with QC_GVSP_B0054 = 1.

Fig. 5 Summary of coding efficiency comparison in terms of synthesized PSNR vs. total bitrate (ATM 6.1)

Table 4. ATM 6.1 with BVSP and DMVP using proposed DV derivation in TFCO vs. ATM 6.1 anchor (cross-checked)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.27 | -0.01 | -2.70 | 0.18 | 0.09 | 0.00 | -0.41 | 0.01 | / | 104.16 | / |
| S02 | -0.04 | 0.00 | -3.29 | 0.15 | -0.42 | 0.01 | -0.32 | 0.01 | / | 103.81 | / |
| S03 | -0.33 | 0.01 | -1.71 | 0.14 | -0.34 | 0.01 | -1.01 | 0.03 | / | 102.81 | / |
| S04 | 0.02 | 0.00 | -6.34 | 0.37 | -0.21 | 0.01 | -0.90 | 0.03 | / | 102.53 | / |
| S05 | 1.64 | -0.08 | -1.94 | 0.09 | 1.06 | -0.05 | 0.75 | -0.03 | / | 102.14 | / |
| S06 | 1.23 | -0.06 | -6.32 | 0.30 | 0.42 | -0.02 | 0.50 | -0.02 | / | 103.88 | / |
| S08 | 0.29 | -0.01 | -2.77 | 0.12 | -0.34 | 0.01 | -0.33 | 0.01 | / | 105.86 | / |
| Average | 0.44 | -0.02 | -3.58 | 0.19 | 0.04 | 0.00 | -0.25 | 0.01 | / | 103.60 | / |

Table 5. ATM 6.1 with BVSP and DMVP using proposed DV derivation in DFCO vs. ATM 6.1 anchor (cross-checked)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.07 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | 0.08 | 0.00 | / | 102.62 | / |
| S02 | -0.23 | 0.01 | 0.00 | 0.00 | -0.20 | 0.01 | 0.04 | 0.00 | / | 101.94 | / |
| S03 | -0.42 | 0.01 | 0.00 | 0.00 | -0.38 | 0.01 | -0.13 | 0.00 | / | 101.26 | / |
| S04 | -0.08 | 0.00 | 0.00 | 0.00 | -0.07 | 0.00 | 0.09 | 0.00 | / | 98.67 | / |
| S05 | 1.34 | -0.06 | 0.00 | 0.00 | 1.21 | -0.05 | 1.19 | -0.05 | / | 101.50 | / |
| S06 | 0.92 | -0.05 | 0.00 | 0.00 | 0.79 | -0.04 | 1.04 | -0.05 | / | 102.26 | / |
| S08 | -0.04 | 0.00 | 0.00 | 0.00 | -0.03 | 0.00 | 0.10 | 0.00 | / | 102.84 | / |
| Average | 0.20 | -0.01 | 0.00 | 0.00 | 0.18 | -0.01 | 0.34 | -0.01 | / | 101.58 | / |

Table 6. ATM 6.1 in TFCO with BVSP and DMVP off vs. ATM 6.1 anchor (informative)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 40.80 | -1.09 | -2.72 | 0.18 | 36.80 | -1.01 | 31.52 | -0.89 | / | 94.04 | / |
| S02 | 12.74 | -0.35 | -6.07 | 0.28 | 11.07 | -0.31 | 9.22 | -0.27 | / | 94.82 | / |
| S03 | 22.39 | -0.70 | -4.90 | 0.40 | 20.57 | -0.67 | 15.72 | -0.46 | / | 92.19 | / |
| S04 | 28.97 | -0.93 | -11.16 | 0.67 | 26.75 | -0.87 | 21.22 | -0.66 | / | 89.54 | / |
| S05 | 27.62 | -1.18 | -2.59 | 0.12 | 22.38 | -0.93 | 20.92 | -0.80 | / | 95.17 | / |
| S06 | 33.43 | -1.48 | -6.67 | 0.32 | 28.75 | -1.26 | 25.45 | -1.04 | / | 94.55 | / |
| S08 | 14.98 | -0.60 | -2.99 | 0.13 | 11.99 | -0.48 | 10.72 | -0.36 | / | 97.81 | / |
| Average | 25.85 | -0.91 | -5.30 | 0.30 | 22.61 | -0.79 | 19.25 | -0.64 | / | 94.02 | / |

Table 7. ATM 6.1 with BVSP using proposed DV derivation in TFCO (DMVP off) vs. ATM 6.1 anchor (informative)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 19.01 | -0.56 | -0.85 | 0.07 | 17.38 | -0.52 | 13.13 | -0.41 | / | 104.59 | / |
| S02 | 3.23 | -0.09 | -1.64 | 0.08 | 2.77 | -0.08 | 2.14 | -0.06 | / | 103.42 | / |
| S03 | 5.99 | -0.20 | 1.56 | -0.12 | 5.73 | -0.19 | 3.64 | -0.11 | / | 102.53 | / |
| S04 | 5.46 | -0.19 | -4.41 | 0.25 | 5.02 | -0.18 | 3.31 | -0.11 | / | 101.23 | / |
| S05 | 18.36 | -0.82 | -0.72 | 0.03 | 15.64 | -0.67 | 12.69 | -0.51 | / | 101.40 | / |
| S06 | 21.08 | -0.95 | -3.87 | 0.18 | 18.37 | -0.82 | 15.04 | -0.63 | / | 102.66 | / |
| S08 | 10.81 | -0.44 | -1.48 | 0.07 | 8.87 | -0.36 | 7.44 | -0.26 | / | 102.45 | / |
| Average | 11.99 | -0.46 | -1.63 | 0.08 | 10.54 | -0.40 | 8.20 | -0.30 | / | 102.61 | / |

Table 8. ATM 6.1 with DMVP using proposed DV derivation in TFCO (BVSP off) vs. ATM 6.1 anchor (informative)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 1.47 | -0.05 | -3.41 | 0.22 | 1.08 | -0.03 | 0.61 | -0.02 | / | 102.46 | / |
| S02 | 3.74 | -0.11 | -6.43 | 0.30 | 2.75 | -0.08 | 2.23 | -0.07 | / | 101.58 | / |
| S03 | 3.09 | -0.10 | -7.84 | 0.65 | 2.50 | -0.09 | 1.30 | -0.04 | / | 99.19 | / |
| S04 | 6.02 | -0.21 | -12.81 | 0.78 | 5.09 | -0.18 | 3.19 | -0.11 | / | 98.34 | / |
| S05 | 3.24 | -0.15 | -3.17 | 0.15 | 1.86 | -0.08 | 2.18 | -0.09 | / | 102.86 | / |
| S06 | 2.35 | -0.11 | -7.92 | 0.38 | 1.16 | -0.05 | 1.55 | -0.07 | / | 102.33 | / |
| S08 | 0.79 | -0.03 | -3.43 | 0.15 | -0.12 | 0.00 | 0.02 | 0.00 | / | 103.97 | / |
| Average | 2.96 | -0.11 | -6.43 | 0.38 | 2.04 | -0.07 | 1.58 | -0.06 | / | 101.53 | / |