| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0115 |
| --- | --- |

| Title: | 3D-HEVC: Alignment of inter-view vector scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com ohji.nakagami@jp.sony.com Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to align the implementation of inter-view vector scaling (IVS) tool in HTM to 3D-HEVC test model description. In IVS, the predictive inter-view vector is added to the candidate list after the vector is scaled with the difference between view indices. The syntax view_order_index is used as view indices in HTM-5.1 and the syntax view_id is used as view indices in the initial version of 3D-HEVC test model description. Thus the different kind of view indices is used in IVS between HTM-5.1 and 3D-HEVC test model description. This inconsistency should be solved.

It is proposed that IVS uses view_id from the next version of HTM in order that the implementation of HTM consists with 3D-HEVC test model description. In addition, it is also proposed to transmit iv_vector_scaling_flag in SPS extension to avoid the incorrect inter-view vector scaling since view_id is no restrictions on values. 

The experiment is conducted to evaluate the current performance of IVS on HTM-5.1. The experimental results show 4.6% BD-rate gain in view2 and 0.7% BD-rate gain in total views in case of hierarchal inter-view prediction structure. It is recommended to adopt this proposal to the next version HTM and 3D-HEVC test model description.

# Introduction

Inter-view vector scaling (IVS) [1] was adopted at 99th Sanjose 3DV meeting and can scale the predictive inter-view vector in TMVP with the difference between view indices before the vector is added to the candidate lists. An example of IVS is shown in Figure 1.

Figure 1: Inter-view vector scaling in TMVP

The prediction structure is hierarchal inter-view prediction structure and both left view (V0) and right view (V2) can be referred when the center view (V1) is coded. The coding block refers to the reference picture of V0 and the co-located block refers to the picture of V2. The predictive vector from co-located block is scaled since the difference of view index (V1-V0) between coding block and coding reference block is not equal to that (V1-V2) between co-located block and this reference block.

The scaling function to inter-view vector is the same as that to MV scaling, but the scaling factors are different. In case of MV scaling [2], the scaling factor “tb” is the subtracted value between POC of coding block and coding reference block and “td” is the subtracted value between POC of co-located block and this reference block. In case of IVS, the scaling factors “tb” and “td” are calculated with subtracted value between view indices of each block instead of POC.

# Problem statement

It is not sufficient alignment about IVS between HTM-5.1 and 3D-HEVC test model description [2]. 

IVS uses view_order_index as view indices in HTM-5.1. The view_order_idx is transmitted in SPS extension and is assumed to be assigned in spatial order of views either from left to right or from right to left. In 3D-HEVC test model description, IVS uses view_id as view indices. The view_id is transmitted in VPS and no restrictions on values. Thus the different kind of view indices is used in IVS between HTM-5.1 and 3D-HEVC test model description.

# Proposal

It is proposed that IVS uses view_id as view indices from the next version of HTM in order that the implementation of HTM consists with 3D-HEVC 3D-HEVC test model description.

The coding efficiency of IVS depends on the accuracy of view indices. It is essential that the view indices are assigned in spatial order of views either from left to right or from right to left so that the inter-view vector is scaled correctly. However, the view_id is not assumed to represent the spatial ordered view indices since this index is no restrictions on values. Thus, the inter-view vector may not be scaled correctly.

Then, it is also proposed to transmit iv_vector_scaling_flag in SPS extension to avoid the incorrect inter-view vector scaling. The inter-view vector is scaled with view_id when this flag is equal to 1 and not scaled when this flag is equal to 0. It is supposed that this flag is set to 1 when view_id is assigned in spatial order of views and is set to 0 otherwise in encoder side.

# Experimental results

The experiment is conducted to evaluate the current performance of IVS on HTM-5.1. In this software, anchor data is created by changing “SONY_COLPIC_AVAILABILITY” macro, which includes IVS based on m23639, to 0 in order to emulate the coding efficiency with no IVS. The other macro settings are not changed. The proposed data is created with IVS in TMVP described in m23639.

The coding efficiency of IVS can not be evaluated in prediction structure of common test condition since this tool requires at least two inter-view reference pictures. Then, the prediction structure is changed to the right side in Figure 2. This prediction structure is hierarchal inter-view prediction structure which is the same prediction structure with MVC anchor of 3DV CfP [4]. IVS actually runs only in center view (V1) since this view can refer to both left view (V0) and right view (V2).

Figure 2: Tested prediction structure

Table 1 shows the view_id used in this experiment and view_id is assigned in spatial order of views either from left to right or from right to left and represents the approximately relative camera position. The same view order is used in this experiment.

Table 1: view_id of each test sequences

|  | view_id |  |  |
| --- | --- | --- | --- |
|  | V0 | V1 | V2 |
| Balloons | 0 | 1 | 2 |
| Kendo | 0 | 1 | 2 |
| Newspapercc | 0 | 1 | 2 |
| GhostTownFly | 2 | 1 | 0 |
| PoznanHall2 | 2 | 1 | 0 |
| PoznanStreet | 2 | 1 | 0 |
| UndoDancer | 0 | 1 | 2 |

The reference picture list in V1 is changed to Table 2. The inter-view reference picture of left view (V0) is mainly stored to LIST_0 and that of right view (V2) is mainly stored to LIST_1. The inserted position of inter-view reference picture is basically the same as common test condition.

Table 2: Changed reference picture list of V1

| LIST_0 | ref_idx |  |  |  |  | LIST_1 | ref_idx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POC | 0 | 1 | 2 | 3 |  | POC | 0 | 1 | 2 | 3 |
| 0 | V0 |  |  |  |  | 0 | V2 |  |  |  |
| 8 | 0 | V0 | V2 |  |  | 8 | 0 | V2 | V0 |  |
| 4 | 0 | V0 | 8 |  |  | 4 | 8 | V2 | 0 |  |
| 2 | 0 | V0 | 4 |  |  | 2 | 4 | V2 | 8 |  |
| 1 | 0 | V0 | 2 |  |  | 1 | 2 | V2 | 4 |  |
| 3 | 2 | 0 | V0 |  |  | 3 | 4 | 8 | V2 |  |
| 6 | 4 | 2 | V0 |  |  | 6 | 8 | 4 | V2 |  |
| 5 | 4 | 0 | V0 |  |  | 5 | 6 | 8 | V2 |  |
| 7 | 6 | 4 | V0 |  |  | 7 | 8 | 6 | V2 |  |
| 16 | 8 | V0 | 6 | 4 |  | 16 | 8 | V2 | 6 | 4 |
| 12 | 8 | V0 | 6 |  |  | 12 | 16 | V2 | 8 |  |
| 10 | 8 | V0 | 6 |  |  | 10 | 12 | V2 | 16 |  |
| 9 | 8 | V0 | 10 |  |  | 9 | 10 | V2 | 12 |  |
| 11 | 10 | 8 | V0 |  |  | 11 | 12 | 16 | V2 |  |
| 14 | 12 | 10 | V0 |  |  | 14 | 16 | 12 | V2 |  |
| 13 | 12 | 8 | V0 |  |  | 13 | 14 | 16 | V2 |  |
| 15 | 14 | 12 | V0 |  |  | 15 | 16 | 14 | V2 |  |

# Compression performance

The following results show the coding performance in comparison with anchor. There are 4.6% BD-rate gain in video2 and 0.7% BD-rate gain in the total of coding views and synthesized views. The coding efficiency of video1 is no difference between anchor and proposal.

Table 3 : BD-rate

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -3.4% | -0.6% | -0.6% | -0.5% |
| Kendo | 0.0% | 0.0% | -4.8% | -0.9% | -0.8% | -0.7% |
| Newspaper_CC | 0.0% | 0.0% | -3.7% | -0.7% | -0.6% | -0.6% |
| GT_Fly | 0.0% | 0.0% | -6.4% | -0.7% | -0.7% | -0.6% |
| Poznan_Hall2 | 0.0% | 0.0% | -3.3% | -0.7% | -0.6% | -0.6% |
| Poznan_Street | 0.0% | 0.0% | -5.9% | -0.8% | -0.8% | -0.7% |
| Undo_Dancer | 0.0% | 0.0% | -4.5% | -0.6% | -0.6% | -1.1% |
| 1024x768 | 0.0% | 0.0% | -3.9% | -0.7% | -0.7% | -0.6% |
| 1920x1088 | 0.0% | 0.0% | -5.0% | -0.7% | -0.7% | -0.7% |
| average | 0.0% | 0.0% | -4.6% | -0.7% | -0.7% | -0.7% |

The following results show the simulation time in comparison with anchor. The encoding and decoding time is no significant difference between the proposal and the anchor.

Table 4 : Encoding and decoding time

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 100.6% | 100.0% |
| Kendo | 101.5% | 100.0% |
| Newspapercc | 99.8% | 100.0% |
| GhostTownFly | 98.5% | 100.2% |
| PoznanHall2 | 101.4% | 100.2% |
| PoznanStreet | 99.5% | 100.1% |
| UndoDancer | 99.5% | 100.1% |
| 1024x768 | 100.6% | 100.0% |
| 1920x1088 | 99.7% | 100.1% |
| average | 100.1% | 100.1% |

# Conclusion

This contribution proposed to align the implementation of inter-view vector scaling (IVS) tool in HTM to 3D-HEVC test model description. It was proposed that IVS uses view_id from the next version of HTM in order that the implementation of HTM consists with the latest MV-HEVC draft. It was also proposed to transmit iv_vector_scaling_flag in SPS extnsion to avoid the incorrect inter-view vector scaling. The experimental results showed 4.6% BD-rate gain in view2 and 0.7% BD-rate gain in total views in case of hierarchal inter-view prediction structure. It is recommended to adopt this proposal to the next version HTM and 3D-HEVC test model description.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “3D-HEVC-CE3 : Results on Motion Parameter Prediction by Sony”, Doc. M23639, Sanjose, USA, Feb. 2012.
[2] ISO/IEC JTC1/SC29/WG11, “3D-HEVC Test Model 2”, Doc. JCT3V-B1005, Shanghai, CN, Oct. 2012.
[3] ISO/IEC JTC1/SC29/WG11, “High Efficient Video Coding (HEVC) text specification draft 9”, Doc. JCTVC-K1003, Shanghai, CN, Oct. 2012.
[4] ISO/IEC JTC1/SC29/WG11, “Call for Proposals on 3D Video Coding Technology”, Doc. N12036, CH, Mar. 2011.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed changes to JCT3V-B1005_d0

##### G.7.3.2.2	Sequence parameter set RBSP syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| iv_vector_scaling_flag | u(1) |
| } |  |

##### G.7.4.2.2	Sequence parameter set RBSP semantics

iv_vector_scaling_flag equal to 1 specifies that inter-view vector of current view may be scaled with view_id[i]. iv_vector_scaling_flag equal to 0 specifies that inter-view vector of current view is not scaled.

G.8.5.2.1.7	Derivation process for temporal luma motion vector prediction

The specifications in subclause 8.5.2.1.7 apply with the following additions at the end of the subclause.

RefViewId( picX, refIdx, LX ) returns the view_id value of reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

RefPicViewId( picX, refIdx, LX ) = ViewId( RefPicListX[ refIdx ] of the picture picX)  (G-xx)

wherein ViewId( pic ) returns the view_id of the picture pic.

The view_id value of a picture is the view_id of the view the picture blongs to and is defined in the video parameter set extension.

The function ViewRefPicType( picX, refIdx, LX ) is defined as follows. If the reference picture with index refIdx from reference picture list LX of the picture picX was an inter-view reference picture at the time when picX was the current picture, ViewRefPicType (picX, refIdx, LX) returns 1; otherwise ViewRefPicType( picX, refIdx, LX ) returns 0.

If availableFlagLXCol is equal to 1 and RefPicListX[ refIdxLX ] is a long-term reference picture, the following applies.

- If ViewId( colPic ) – RefViewId( colPic, refIdxCol, listCol ) is not equal to ViewId – ViewOrdeId( RefPicListX[ refIdxLX ] ) and both ViewRefPicType( currPic, refIdxLX, ListX) and ViewRefPicType( colPic, refIdxCol, listCol ) are equal to 1 and iv_vector_scaling_flag is equal to 1, mvLXCol is derived as scaled version of the motion vector mvCol as specified by equation 8-167, equation 8-168 and equation 8-169.

where td and tb are derived as

td = Clip3( −128, 127, ViewId( colPic ) − RefViewId( colPic, refIdxCol, listCol ) )      (G-xx)

tb = Clip3( −128, 127, ViewId( currPic ) − ViewId( RefPicListX [ refIdxLX ] )         (G-xx)