| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0116 |
| --- | --- |

| Title: | 3D-HEVC: Inter-view vector scaling for AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Sally Hattori Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com ohji.nakagami@jp.sony.com Shinobu.Hattori@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to extend inter-view vector scaling (IVS) to AMVP (IVS-AMVP). In IVS, the predictive inter-view vector is added to the candidate list after the vector is scaled with the difference between view indices. In the latest software and specification text, IVS is used in only TMVP (IVS-TMVP), but it is natural that IVS should be also used in AMVP.

The experiment is conducted to evaluate the performance of IVS-AMVP on HTM-5.1. In comparison with IVS-TMVP, both IVS-TMVP and IVS-AMVP can achieve 0.2% BD-rate gain in view2 and 0.0% BD-rate gain in total views in case of hierarchal inter-view prediction structure. The experimental results also show 4.8% BD-rate gain in view2 and 0.7% BD-rate gain in total views in comparison with no IVS.

It is recommended to adopt this proposal to the next version HTM and 3D-HEVC test model description.

# Introduction

Inter-view vector scaling (IVS) [1] was adopted at 99th Sanjose 3DV meeting and can scale the predictive inter-view vector in TMVP with the difference between view indices before the vector is added to the candidate lists. An example of IVS in TMVP is shown in Figure 1.

Figure 1: Inter-view vector scaling in TMVP

The prediction structure is hierarchal inter-view prediction structure and both left view (V0) and right view (V2) can be referred when the center view (V1) is coded. The coding block refers to the reference picture of V0 and the co-located block refers to the picture of V2. The predictive vector from co-located block is scaled since the difference of view index (V1-V0) between coding block and coding reference block is not equal to that (V1-V2) between co-located block and this reference block.

The scaling function to inter-view vector is the same as that to MV scaling, but the scaling factors are different. In case of MV scaling [2], the scaling factor “tb” is the subtracted value between POC of coding block and coding reference block and “td” is the subtracted value between POC of co-located block and this reference block. In case of IVS, the scaling factors “tb” and “td” are calculated with subtracted value between view indices of each block instead of POC.

# Proposal

It is proposed that IVS is extended to AMVP. As for MV scaling, it is possible that IVS is used in AMVP as well as TMVP. An example of IVS in AMVP and TMVP is shown in Figure 2.

Figure 2: Inter-view vector scaling in AMVP and TMVP

The coding block refers to the reference picture of V0 and the neighboring block refers to the picture of V2. The predictive vector from neighboring block is scaled since the difference of view index (V1-V0) between coding block and coding reference block is not equal to that (V1-V2) between neighboring block and this reference block. The scaling function is the same as TMVP.

# Experimental results

The experiment is conducted to evaluate the performance of IVS-AMVP on HTM-5.1. The performance of IVS-AMVP and IVS-TMVP is compared with IVS-TMVP and no IVS, respectively. IVS-TMVP is created based on the proposal in JCT3V-C0115[4]. The no IVS is created by changing “SONY_COLPIC_AVAILABILITY” macro to 0 in order to emulate the coding efficiency with no IVS. 

The coding efficiency of IVS can not be evaluated in prediction structure of common test condition since this tool requires at least two inter-view reference pictures. Then, the prediction structure is changed to the right side in Figure 2. This prediction structure is hierarchal inter-view prediction structure which is the same prediction structure with MVC anchor of 3DV CfP [5]. IVS actually runs only in center view (V1) since this view can refer to both left view (V0) and right view (V2).

Figure 3: Tested prediction structure

Table 1 shows the view_id used in this experiment and view_id is assigned in spatial order of views either from left to right or from right to left and represents the approximately relative camera position. The same view order is used in this experiment.

Table 1: view_id of each test sequences

|  | view_id |  |  |
| --- | --- | --- | --- |
|  | V0 | V1 | V2 |
| Balloons | 0 | 1 | 2 |
| Kendo | 0 | 1 | 2 |
| Newspapercc | 0 | 1 | 2 |
| GhostTownFly | 2 | 1 | 0 |
| PoznanHall2 | 2 | 1 | 0 |
| PoznanStreet | 2 | 1 | 0 |
| UndoDancer | 0 | 1 | 2 |

The reference picture list in V1 is changed to Table 2. The inter-view reference picture of left view (V0) is mainly stored to LIST_0 and that of right view (V2) is mainly stored to LIST_1. The inserted position of inter-view reference picture is basically the same as common test condition.

Table 2: Changed reference picture list of V1

| LIST_0 | ref_idx |  |  |  |  | LIST_1 | ref_idx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POC | 0 | 1 | 2 | 3 |  | POC | 0 | 1 | 2 | 3 |
| 0 | V0 |  |  |  |  | 0 | V2 |  |  |  |
| 8 | 0 | V0 | V2 |  |  | 8 | 0 | V2 | V0 |  |
| 4 | 0 | V0 | 8 |  |  | 4 | 8 | V2 | 0 |  |
| 2 | 0 | V0 | 4 |  |  | 2 | 4 | V2 | 8 |  |
| 1 | 0 | V0 | 2 |  |  | 1 | 2 | V2 | 4 |  |
| 3 | 2 | 0 | V0 |  |  | 3 | 4 | 8 | V2 |  |
| 6 | 4 | 2 | V0 |  |  | 6 | 8 | 4 | V2 |  |
| 5 | 4 | 0 | V0 |  |  | 5 | 6 | 8 | V2 |  |
| 7 | 6 | 4 | V0 |  |  | 7 | 8 | 6 | V2 |  |
| 16 | 8 | V0 | 6 | 4 |  | 16 | 8 | V2 | 6 | 4 |
| 12 | 8 | V0 | 6 |  |  | 12 | 16 | V2 | 8 |  |
| 10 | 8 | V0 | 6 |  |  | 10 | 12 | V2 | 16 |  |
| 9 | 8 | V0 | 10 |  |  | 9 | 10 | V2 | 12 |  |
| 11 | 10 | 8 | V0 |  |  | 11 | 12 | 16 | V2 |  |
| 14 | 12 | 10 | V0 |  |  | 14 | 16 | 12 | V2 |  |
| 13 | 12 | 8 | V0 |  |  | 13 | 14 | 16 | V2 |  |
| 15 | 14 | 12 | V0 |  |  | 15 | 16 | 14 | V2 |  |

# Compression performance

The following results show the coding performance in comparison with two kinds of anchor. It is shown that 0.2% BD-rate gain in video2 and 0.0% BD-rate gain in the total of coding views and synthesized views in comparison with IVS-TMVP. It is also shown that 4.8% BD-rate gain in video2 and 0.7% BD-rate gain in the total of coding views and synthesized views in comparison with no IVS. The coding efficiency of video1 is no difference between anchors and proposal.

Table 3: BD-rate in comparison with IVS-TMVP

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Kendo | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| GT_Fly | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.4% | -0.1% | -0.1% | 0.0% |
| Poznan_Street | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |
| Undo_Dancer | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 1920x1088 | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.1% |
| average | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |

Table 4: BD-rate in comparison with no IVS

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -3.4% | -0.6% | -0.6% | -0.6% |
| Kendo | 0.0% | 0.0% | -4.9% | -0.9% | -0.8% | -0.7% |
| Newspaper_CC | 0.0% | 0.0% | -3.6% | -0.7% | -0.6% | -0.6% |
| GT_Fly | 0.0% | 0.0% | -6.7% | -0.8% | -0.7% | -0.6% |
| Poznan_Hall2 | 0.0% | 0.0% | -3.7% | -0.8% | -0.7% | -0.5% |
| Poznan_Street | 0.0% | 0.0% | -6.2% | -0.9% | -0.8% | -0.7% |
| Undo_Dancer | 0.0% | 0.0% | -4.8% | -0.7% | -0.6% | -0.7% |
| 1024x768 | 0.0% | 0.0% | -4.0% | -0.7% | -0.7% | -0.7% |
| 1920x1088 | 0.0% | 0.0% | -5.3% | -0.8% | -0.7% | -0.6% |
| average | 0.0% | 0.0% | -4.8% | -0.7% | -0.7% | -0.7% |

The following results show the simulation time in comparison with anchors. The encoding and decoding time is no significant difference between the proposal and the anchors.

Table 5 : Encoding and decoding time in comparison with IVS-TMVP

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 99.4% | 99.2% |
| Kendo | 101.3% | 99.2% |
| Newspapercc | 99.4% | 99.1% |
| GhostTownFly | 100.5% | 102.5% |
| PoznanHall2 | 100.2% | 99.1% |
| PoznanStreet | 100.5% | 99.1% |
| UndoDancer | 100.6% | 99.2% |
| 1024x768 | 100.0% | 99.2% |
| 1920x1088 | 100.4% | 100.0% |
| average | 100.3% | 99.6% |

Table 6 : Encoding and decoding time in comparison with no IVS

|  | enc time | dec time |
| --- | --- | --- |
| Balloons | 100.0% | 99.2% |
| Kendo | 102.9% | 99.3% |
| Newspapercc | 99.2% | 99.1% |
| GhostTownFly | 99.1% | 102.6% |
| PoznanHall2 | 101.6% | 99.3% |
| PoznanStreet | 100.0% | 99.2% |
| UndoDancer | 100.1% | 99.4% |
| 1024x768 | 100.7% | 99.2% |
| 1920x1088 | 100.2% | 100.1% |
| average | 100.4% | 99.7% |

# Conclusion

This contribution proposed inter-view vector scaling (IVS) tool in AMVP. In comparison with IVS-TMVP, IVS-AMVP and IVS-TMVP can achieve 0.2% BD-rate gain in view2 and 0.0% BD-rate gain in total views. The experimental results also show 4.8% BD-rate gain in view2 and 0.7% BD-rate gain in total views in comparison with no IVS. It is recommended to adopt this proposal to the next version HTM and 3D-HEVC test model description.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “3D-HEVC-CE3 : Results on Motion Parameter Prediction by Sony”, Doc. M23639, Sanjose, USA, Feb. 2012.
[2] ISO/IEC JTC1/SC29/WG11, “3D-HEVC Test Model 2”, Doc. JCT3V-B1005, Shanghai, CN, Oct. 2012.
[3] ISO/IEC JTC1/SC29/WG11, “High Efficient Video Coding (HEVC) text specification draft 9”, Doc. JCTVC-K1003, Shanghai, CN, Oct. 2012.
[4] ISO/IEC JTC1/SC29/WG11, “3D-HEVC:Alignment of inter-view vector scaling”, Doc. JCT3V-C0115, CH, Jan. 2013.
[5] ISO/IEC JTC1/SC29/WG11, “Call for Proposals on 3D Video Coding Technology”, Doc. N12036, CH, Mar. 2011.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed changes to JCT3V-B1005_d0 with JCT3V-C0116

##### G.7.3.2.2	Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1),

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-4 – Spatial motion vector neighbours (informative)

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

Let CurrViewId be view_id value of a current picture.The function ViewId( picX ) is specified as follows:

ViewId( picX ) = view_id of the picture picX 						(8-xxx)

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- The sample location (xA0, yA0) is set equal to (xP − 1, yP + nPbH) and the sample location (xA1, yA1) is set equal to (xA0, yA0 − 1).

- The availability flag availableFlagLXA is set equal to 0 and the both components of mvLXA are set equal to 0.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xA0, yA0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA0.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xA1, yA1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA1.

- When availableA0 or availableA1 is equal to TRUE, the variable isScaledFlagLX is set equal to 1.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ), the following applies repeatedly until availableFlagLXA is equal to 1:

- When availableAk is equal to TRUE , PredMode[ xAk][ yAk ] is not equal to MODE_INTRA and availableFlagLXA is equal to 0, the following applies.

- If, PredFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the following assignments are made.

mvLXA = MvLX[ xAk ][ yAk ]                         			(8-1)
refIdxA = RefIdxLX[ xAk ][ yAk ]                      			(8-2)

- Otherwise, if PredFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xAk ][ yAk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXA is set equal to 1 and the following assignments are made.

mvLXA = MvLY[ xAk ][ yAk ]			                             (8-3)
refIdxA = to RefIdxLY[ xAk ][ yAk ]                 	     			(8-4)

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ), the following applies repeatedly until availableFlagLXA is equal to 1:

- When availableAk is equal to TRUE , PredMode[ xAk][ yAk ] is not equal to MODE_INTRA and availableFlagLXA is equal to 0, the following applies.

- If PredFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

mvLXA = MvLX[ xAk ][ yAk ]			                         	(8-5)
refIdxA = RefIdxLX[ xAk ][ yAk ]                       			(8-6)
refPicListA = RefPicListX                             			(8-7)

- Otherwise, if PredFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

mvLXA = MvLY[ xAk ][ yAk ]                         			(8-8)
refIdxA = to RefIdxLY[ xAk ][ yAk ]                    			(8-9)
refPicListA = RefPicListY                            			(8-10)

- When availableFlagLXA is equal to 1, and both refPicListA[ refIdxA ] and RefPicListX[ refIdxLX ] are short-term reference pictures, or ViewId(refPicListA[ refIdxA ] ) is not equal to ViewId(refPicListX[ refIdxX ] ) and iv_vector_scaling_flag is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td                                           (8-11)
distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )                         (8-12)
mvLXA = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLXA ) * 		( (Abs( distScaleFactor * mvLXA ) + 127 ) >> 8 ) )                       (8-13)

where td and tb are derived as

      - If RefPicListX[ refIdxLX ] is short-term reference picture,

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( refPicListA[ refIdxA ] ) )     	(8-14)
tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )   	(8-15)

      - Otherwise,

td = Clip3( −128, 127, CurrViewId  – ViewId ( refPicListA[ refIdxA ] ) )	        (8-xxx)
tb = Clip3( −128, 127, CurrViewId  –  ViewId ( RefPicListX[ refIdxLX ] )	        (8-xxx)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPbW, xB1 = xB0− 1, xB2 = xP − 1 and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-16)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-17)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-18)

- The availability flag availableFlagLXB is set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPbW, xB1 = xB0 − 1, and xB2 =  xP − 1, the following applies repeatedly until availableFlagLXB is equal to 1:

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xBk, yBk ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies.

- If PredFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the following assignments are made.

mvLXB = MvLX[ xBk ][ yBk ]							(8-19)
refIdxB = RefIdxLX[ xBk ][ yBk ]							(8-20)

- Otherwise, if PredFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xBk ][ yBk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ), availableFlagLXB is set equal to 1 and the following assignments are made.

mvLXB = MvLY[ xBk ][ yBk ]							(8-21)
refIdxB = RefIdxLY[ xBk ][ yBk ]							(8-22)

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

mvLXA = mvLXB									(8-23)
refIdxA = refIdxLXB									(8-24)

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPbW, xB1 = xB0 − 1, and xB2 = xP − 1, the following applies repeatedly until availableFlagLXB is equal to 1:

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xBk, yBk ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies.

- If PredFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made.

mvLXB = MvLX[ xBk ][ yBk ]							(8-25)
refIdxB = RefIdxLX[ xBk ][ yBk ]							(8-26)
refPicListB = RefPicListX							(8-27)

- Otherwise, if PredFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made.

mvLXB = MvLY[ xBk ][ yBk ]							(8-28)
refIdxB = RefIdxLY[ xBk ][ yBk ]							(8-29)
refPicListB = RefPicListY							(8-30)

- When availableFlagLXB is equal to 1 and PicOrderCnt( refPicListB[ refIdxB ] ) is not equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and both refPicListB[ refIdxB ] and RefPicListX[ refIdxLX ] are short-term reference pictures, or ViewId(refPicListB[ refIdxB ] ) is not equal to ViewId(refPicListX[ refIdxX ] ) and iv_vector_scaling_flag is equal to 1,  mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td						(8-31)
distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )				(8-32)
mvLXB =Clip3( −32768, 32767, Sign2( distScaleFactor * mvLXB ) * 
				　　　( (Abs( distScaleFactor * mvLXB ) + 127 ) >> 8 ) )			(8-33)

where td and tb are derived as

      - If RefPicListX[ refIdxLX ] is short-term reference picture,

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( refPicListB[ refIdxB ] ) )	(8-34)
tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(8-35)

      - Otherwise,

td = Clip3( −128, 127, CurrViewId  – ViewId ( refPicListB[ refIdxB ] ) )		(8-xxx)
tb = Clip3( −128, 127, CurrViewId  –  ViewId ( RefPicListX[ refIdxLX ] ) )		(8-xxx)