| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, Switzerland, 17–23 Jan. 2013 | Document: JCT3V-C0117 |
| --- | --- |

| Title: | 3D-CE2.h related: Motion Aware Temporal Disparity Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaewon Sung, Moonmo Koo, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-434-5259 {jw.sung, moonmo.koo, sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a modified disparity vector derivation algorithm that changes the location of temporal neighbour blocks according to the motion vector obtained from spatial neighbour blocks. In the disparity derivation process of the HTM 5.1, the locations of temporal neighbour blocks are always fixed although there might be motions between the current and temporal reference pictures. When one of the spatial neighbour blocks is referencing the same temporal reference picture for disparity vector derivation, the locations of the temporal neighbour blocks are changed by the amount of motion vector. The proposed algorithm reportedly showed coding gains about -0.2%, -0.1%, -0.1% BD-bitrate savings for V1, V2, video only.

# Introduction

In the HTM 5.0.1, the derived disparity vector is obtained from spatial and temporal neighbour blocks. The positions of temporal neighbour blocks are fixed as the center (CT) of current coding block and bottom right (BR) neighbour blocks in temporal reference pictures. However, due to the motions that happen between current picture and temporal reference pictures for temporal disparity derivation, looking at the fixed temporal neighbour block positions cannot be the best choice.

In this contribution, the positions of the temporal neighbour blocks are estimated by the temporal motion vector of spatial neighbour blocks of current coding block. If a spatial neighbour block refers the same temporal reference picture that is used for temporal disparity vector derivation, then the positions of the temporal neighbour blocks are modified by the motion vector of the spatial neighbour block.

# Proposed algorithm

Figure 1 shows the positions of spatial and temporal neighbour blocks used for disparity vector derivation in the HTM 5.0.1. The disparity vector derivation process works as follows [1],[2].

- Step1) The spatial neighbour blocks are scanned to check if any of them is coded by disparity compensated prediction. In that case, the disparity vector used for the disparity compensated prediction is returned as the derived disparity vector and the disparity derivation process terminates.

- Step2) The temporal neighbour blocks in the temporal reference pictures are visited to check if any of them is coded by disparity compensated prediction. If the disparity compensated block is not found in the collocated temporal reference picture, then another temporal reference picture with low temporal level id is used. For each temporal reference picture, the temporal neighbour blocks are visited in the order of CT, BR for V1 and BR, CT for V2. If a disparity compensated prediction block is found then the disparity vector used for the disparity compensated prediction is returned as the derived disparity vector and the disparity derivation process terminates.

- Step3) The spatial neighbour blocks are scanned to check if any of them is coded by motion compensated prediction where the motion vector is inter-view predicted (DV-MCP block). In that case, the disparity vector used for the inter-view motion vector prediction is returned as the derived disparity vector and the disparity derivation process terminates.

Figure 1 Positions of the spatial and temporal neighbour blocks for a current PU

The Figure 2 shows the way that current temporal disparity vector derivation process works where no matter how the temporal reference picture is far from the current picture a fixed position is always used. The Figure 3 shows the proposed idea where the position of the BR temporal neighbour block is adaptively determined by the estimated motion vector mvNb obtained from a spatial neighbour block.

| Figure 2 | Figure 3 |
| --- | --- |

The proposed algorithm modified the positions of temporal neighbour blocks as follows.

- Modified step 1): During step1, if a spatial neighbour block is coded by motion compensated prediction then the motion vector and the POC of the reference picture are saved to a temporal memory as (mvNbX, pocNbX), where X is being replaced by A0, A1, B0, B1, B2.

- Modified step2): The Step 2) is processed after the following two steps: 2-1 and 2-2. For each temporal reference picture whose POC is curPoc, the following applies.

- Step 2-1) Motion vector estimation: the mvNbX are scanned in the following order: A1, B1, B0, A0, B2. In this step the mvNbX are scanned until a valid mvNbX is found and saved to mvNb. A mvNbX is valid if the following conditions are all satisfied.

  - The mvNbX is a temporal motion vector

  - The pocNbX is equal to curPoc; If current temporal reference picture is the last temporal reference picture among the available temporal reference pictures, then the (mvNbX, pocNbX) is scaled so that the scaled mvNbX corresponds to curPoc. Therefore, when the temporal reference picture is the last temporal reference picture, this condition is always satisfied.

  - The horizontal and vertical components of mvNbX are large enough to change the current position of a temporal neighbour block to another 4x4 partition. i.e., they should be larger than 2 pixels.

- Step 2-2) Modification of temporal neighbour block positions:

  - If a valid mvNb is found, then the mvNb is added to the position of the partition that indicating the position of temporal neighbour blocks.

  - This step is applied to both CT and BR block positions to change them to CT' and BR'.

# Experiment results

The proposed algorithm is implemented on top of the HTM5.0.1. For experiment, common test condition for coding 3 view texture and depth was used [3].

Table 1 shows the coding performance of the proposed algorithm with respect to HTM5.0.1. The proposed algorithm showed -0.2%, -0.1%, and -0.1% BD rate savings for V1, V2, and video only cases, respectively. The proposed algorithm did not increase the encoding and decoding time.

Table 1

|  | V0 | V1 | V2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 101.2% | 95.5% | 99.6% |
| Kendo | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 100.9% | 95.0% | 97.3% |
| Newspapercc | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 101.3% | 100.8% | 102.7% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 97.7% | 110.5% |
| PoznanHall2 | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% | 0.1% | 101.0% | 92.4% | 107.0% |
| PoznanStreet | 0.0% | -0.3% | 0.0% | 0.0% | -0.1% | -0.1% | 101.1% | 95.1% | 101.3% |
| UndoDancer | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 100.4% | 102.4% | 99.5% |
| 1024x768 | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 101.1% | 97.1% | 99.9% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 96.8% | 104.5% |
| average | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 100.9% | 96.9% | 102.5% |

# Conclusion

This contribution proposed a modified temporal disparity derivation process that adaptively changes the position of temporal neighbour blocks with an motion vector estimated by the motion vectors of spatial neighbour blocks.

Experiment results reportedly showed coding gains without increasing encoding and decoding complexity where the BD bitrate savings were -0.2%, -0.1%, and -0.1% for V1, V2 and video only cases, respectively. Therefore, we recommend adopting the proposed algorithm.

# References

- J. Sung, M. Koo, S. Yea, "3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding," JCT3V-A0126, Jul. 2012.

- J. Kang, Y. Chen, L. Zhang, M. Karczewicz, "3D-CE5.h related: Improvements for disparity vector derivation," JCT3V-B0047, Oct. 2012.

[3] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”,  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, October 2012, Shanghai.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).