| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, Switzerland, 17–23 Jan. 2013 | Document: JCT3V-C0117r1 |
| --- | --- |

| Title: | 3D-CE2.h related: Motion Aware Temporal Disparity Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaewon Sung, Moonmo Koo, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-434-5259 {jw.sung, moonmo.koo, sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a modified temporal disparity vector derivation algorithm that changes the location of temporal neighbour blocks adaptively by an estimated motion vector. In the disparity derivation process of the HTM 5.1, the locations of temporal neighbour blocks are always fixed although there might be motions between the current and temporal reference pictures selected for temporal DV derivation. The motion vector from current picture to the candidate picture is estimated from the temporal motion vectors of spatial neighbor blocks and the locations of the temporal neighbour blocks are changed by the amount of motion vector. The proposed algorithm reportedly showed coding gains about -0.1%, -0.1%, -0.1% BD bitrate savings for V1, V2, video only.

# Introduction

In the HTM 5.0.1, the derived disparity vector is obtained from spatial and temporal neighbour blocks. The positions of temporal neighbour blocks are fixed as the center (CT) of current coding block and bottom right (BR) neighbour blocks (LT when BR is located outside of picture) in two candidate pictures. However, due to the motions between current picture and the candidate picture for temporal disparity derivation, looking at the temporal neighbour blocks at fixed positions cannot be the best choice.

In this contribution, the positions of the temporal neighbour blocks are estimated by the temporal motion vector of spatial neighbour blocks of current coding block. If a spatial neighbour block refers the candidate picture, then the positions of the temporal neighbour blocks are modified by the estimated motion vector.

# Proposed algorithm

Figure 1 shows the positions of spatial and temporal neighbour blocks used for disparity vector derivation in the HTM 5.0.1. The disparity vector derivation process works as follows [1],[2].

- Step1) The spatial neighbour blocks are scanned to check if any of them is coded by disparity compensated prediction. In that case, the disparity vector used for the disparity compensated prediction is returned as the derived disparity vector and the disparity derivation process terminates.

- Step2) The temporal neighbour blocks in the candidate pictures are visited to check if any of them is coded by disparity compensated prediction. If the disparity compensated block is not found in the first candidate picture, then another candidate picture is used. For each candidate picture, the temporal neighbour blocks are visited in the order of CT, BR for V1 and BR, CT for V2. If a disparity compensated prediction block is found then the disparity vector used for the disparity compensated prediction is returned as the derived disparity vector and the disparity derivation process terminates.

- Step3) The spatial neighbour blocks are scanned to check if any of them is coded by motion compensated prediction where the motion vector is inter-view predicted (DV-MCP block). In that case, the disparity vector used for the inter-view motion vector prediction is returned as the derived disparity vector and the disparity derivation process terminates.

Figure 1 Positions of the spatial and temporal neighbour blocks for a current PU

The Figure 2 shows the way that current temporal disparity vector derivation process works where no matter how the candidate picture is far from the current picture, a fixed position is always used. The Figure 3 shows the proposed idea where the position of the BR temporal neighbour block is adaptively determined by the estimated motion vector mvEst obtained from a spatial neighbour block.

| Figure 2 | Figure 3 |
| --- | --- |

The proposed algorithm modified the positions of temporal neighbour blocks as follows.

- Modified step 1): During step1, when a spatial neighbour block has temporal motion vector, the motion vector and the POC value of the reference picture that the motion vector is referencing are saved to a temporal array mvCand[] and pocCand[]. Because the spatial neighbor blocks are scanned in order of A0, A1, B0, B1, B2, the motion vectors and POC values are saved in the same order.

- Modified step2): The Step 2) is followed after the following two steps: 2-1 and 2-2. For each candidate picture whose POC value is pocCandPic, the following applies.

- Step 2-1) Motion vector estimation:

  - The mvCand[] and pocCand[] are scanned sequentially to find an estimated motion vector mvEst. When i-th POC value, pocCand[i], is equal to pocCandPic, then corresponding mvCand[i] is the valid one and set to mvEst.

  - If current candidate picture is the last (2nd) candidate picture and an estimated motion vector mvEst is not found, then the motion vector of the first candidate, mvCand[0] is scaled so that the scaled mvCand[0] corresponds to pocCandPic and set to mvEst.

- Step 2-2) Modification of temporal neighbour block positions:

  - If the mvEst is found, then it is added to the center coordinates of the CT and BR partitions. Otherwise, the position of the CT and BR are not changed.

# Experiment results

The proposed algorithm is implemented on top of the HTM5.0.1. For experiment, common test condition for coding 3 view texture and depth was used [3].

Table 1 shows the coding performance of the proposed algorithm with respect to HTM5.0.1. The proposed algorithm showed -0.1%, -0.1%, and -0.1% BD rate savings for V1, V2, and video only cases, respectively. The proposed algorithm did not increase the encoding and decoding time.

Table 1

|  | V0 | V1 | V2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.1% | -0.1% | 0.0% | -0.1% | 100.8% | 97.2% | 115.5% |
| Kendo | 0.0% | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 100.5% | 98.0% | 111.9% |
| Newspapercc | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 100.4% | 100.9% | 109.7% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 98.3% | 106.8% |
| PoznanHall2 | 0.0% | 0.4% | 0.2% | 0.1% | 0.1% | 0.1% | 100.5% | 91.4% | 116.2% |
| PoznanStreet | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.1% | 100.3% | 94.3% | 114.1% |
| UndoDancer | 0.0% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 100.0% | 101.8% | 102.2% |
| 1024x768 | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 100.5% | 98.7% | 112.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 96.4% | 109.7% |
| average | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 100.4% | 97.4% | 110.8% |

# Conclusion

This contribution proposed a modified temporal disparity derivation process that adaptively changes the position of temporal neighbour blocks with an motion vector estimated by the motion vectors of spatial neighbour blocks.

Experiment results reportedly showed coding gains without increasing encoding and decoding complexity where the BD bitrate savings were -0.1%, -0.1%, and -0.1% for V1, V2 and video only cases, respectively. Therefore, we recommend adopting the proposed algorithm.

# References

- J. Sung, M. Koo, S. Yea, "3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding," JCT3V-A0126, Jul. 2012.

- J. Kang, Y. Chen, L. Zhang, M. Karczewicz, "3D-CE5.h related: Improvements for disparity vector derivation," JCT3V-B0047, Oct. 2012.

[3] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”,  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, October 2012, Shanghai.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text

- G.8.5.2.1.13 Derivation process for a disparity vector

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- Outputs of this process are:

- a view order index refViewIdx specifying a reference view.

- a flag availableDV specifying whether the disparity vector is available,

- a disparity vector mvDisp.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

A variable, numCandMv, specifying the number of collected temporal motion vectors is set equal to 0.

The variable checkParallelMergeFlag is derived as follows.

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xC ][ yC ] is equal to MODE_SKIP

  - PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The motion vector mvDisp and the availability flag availableDV are derived in the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set equal to 1.

  - For each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ), the following applies.

- When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

        - xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(G-1)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(G-2)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(G-3)

- The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the partition index partIdx, and the luma location ( xN, yN ) as inputs and the output is assigned to the prediction block availability flag availableN.

- When all of the following conditions are true, availableA1 is set equal to 0.

      - N is equal to A1

      - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N

      - partIdx is equal to 1

- When all of the following conditions are true, availableB1 is set equal to 0.

      - N is equal to B1

      - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD

      - partIdx is equal to 1

- When all of the following conditions are true, availableN is set equal to 0.

      - checkParallelMergeFlag is equal to 1

      - (xP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to
(xN >> (log2_parallel_merge_level_minus2 + 2) )

      - (yP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2) ).

- The flag availableIvpMvSearchFlagN is set equal to availableN.

- When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

      - N is equal to B0 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

      - N is equal to B1 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

      - N is equal to B2 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

- The flags availableFlagIvpMvN0 and availableFlagIvpMvN1 are set equal to 0.

- For each X from 0 to 1, the following applies, when availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

          - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(G-4)
mvDisp = MvLXN[ xN ][ yN ]		(G-5)
availableDV = 1		(G-6)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies.

        - When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(G-7)
availableFlagIvpMvLXN = 1		(G-8)

        - PredMode[ xN ][ yN ] is equal to MODE_INTER or MODE_SKIP, the following applies:

            - mvCand[ numCandMv ] = MvLX[ xN ][ yN ]

            - pocCand[ numCandMv ] = POC( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )

            - numCandMv ++

  - The derivation process for a disparity vector from temporal neighbour block as specified in  is invoked with a luma location ( xP, yP ), variables nPSW and nPSH, candidate motion vectors mvCand and the POC values, pocCand, of the reference picture that the candidate motion vectors are referring, and the number of candidate motion vectors numCandMv as inputs, and flag availableDV, and disparity vector mvDisp outputs

  - For each N being A1, B1, B0, A0, and B2, the following applies.

    - For each X from 0 to 1, the following applies.

      - When availableFlagIvpMvLXN is equal to 1, the following applies:

          - mvDisp = ivpMvDispLXN		(G-9)
availableDV = 1		(G-10)

- G.8.5.2.1.16 Derivation process for a disparity vector from temporal neighbour blocks

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH

- candidate motion vectors mvCand

- POC values pocCand

- a variable numCandMv specifying the number of candidate motion vectors

Outputs of this process are

- the disparity vector mvDisp,

- the availability flag availableFlag.

The luma location ( xPRb , yPRb) specifying the position of the right bottom luma prediction block of the current luma prediction block and the flag availableRb specifying the availability of the right bottom luma prediction block are derived by the following ordered steps:

  - The vertical component of the position of the right bottom luma prediction block is derived as follows.

      - yPRb = yP + nPbH		(G-11)

  - The horizontal component of the position of the right bottom luma prediction block is derived as follows.

      - xPRb = xP + nPbW		(G-12)

  - When xPRb is less than pic_width_in_luma_samples and yPRb is less than pic_height_in_luma_samples, availableRb is set equal to 1. Otherwise, availableRb is set equal to 0.

The luma location ( xCtr , yCtr) specifying the centre position of the current luma prediction block is derived by

  - xPCtr = xP + ( nPbW >> 1 )		(G-13)

  - yPCtr = yP + ( nPbH >> 1 )		(G-14)

The flag availableFlag is set equal to 0, and both components of mvDisp are set equal to 0.

For i from 0 to NumDdvCandPics, the following ordered steps apply and the whole decoding process of this sub-clause terminates once availableFlag is set to 1.

  - When i is equal to NumDdvCandPic-1 lastTempRefPicFlag is set equal to 1, otherwise, lastTempRefPicFlag is set equal to 0.

  - When numCandMv is larger than 0, the derivation process for a modified position of temporal neighbour prediction block as specified in G.8.5.2.1.20 is invoked with a luma location, (xPRb,yPRb), candidate picture DdvCandPicsList[ i ], candidate motion vectors mvCand, POC numbers pocCand, a variable numCandMv, and a flag lastTempRefPicFlag as inputs, and a modified position of bottom right neighbour prediction block (xPRb,yPRb) as output

  - When numCandMv is larger than 0, the derivation process for a modified position of temporal neighbour prediction block as specified in G.8.5.2.1.20 is invoked with a luma location (xPCtr,yPCtr), candidate picture DdvCandPicsList[ i ], candidate motion vectors mvCand, POC numbers pocCand, a variable numCandMv, and a flag lastTempRefPicFlag as inputs, and a modified position of bottom right neighbour prediction block (xPCtr,yPCtr) as output

  - When ViewIdx of the current picture is greater than 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag and the disparity vector mvDisp as outputs.

  - The derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location (xPCtr, yPCtr) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

  - When ViewIdx of the current picture is equal to 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

- G.8.5.2.1.20 Derivation process for a modified position of temporal neighbour prediction blocks

Inputs to this process are

- a luma location ( xPTnb, yPTnb ) specifying the position of the temporal neighbor luma prediction block of the current luma prediction block relative to the top-left luma sample of the current picture,

- a temporal reference picture tempRefPic specifying current temporal reference picture for disparity vector derivation

- motion vectors mvCand

- POC numbers pocCand

- a variable specifying the number of candidate motion vectors

- a flag lastTempRefFlag specifying that current temporal reference picture is the last temporal reference picture for temporal disparity vector derivation

Outputs of this process are

- the modified luma location (xPTnb,yPTnb)

The luma location (xPTnb, yPTnb) is modified as follows.

The flag availableMvEstFlag is set equal to 0.

For i from 0 to numCand-1, the followings applies.

- When pocCand[i] is equal to PicOrderCnt( tempRefPic ) the following applies.

  - mvEst = mvCand[i]

  - availableMvEstFlag = 1

When availableMvEstFlag is equal to 0 and lastTempRefFlag is equal to 1 the following applies.

  - availableMvEstFlag = 1

  - mvEst is set to scaled mvCand[0] as specified below.

- tx = ( 16384 + ( Abs( td ) >>1 ) ) / td

- distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )

- mvEst =  Clip3( −32768, 32767, Sign2( distScaleFactor * mvCand[0] ) * 		
	        ( (Abs( distScaleFactor * mvCand[0] ) + 127 ) >> 8 ) )

- where td and tb are derived as

- td = Clip3( −128, 127, PicOrderCntVal – pocCand[0] )

- tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( tempRefPic )

- If availableMvEstFlag is equal to 1, the following applies.

- Modified location of temporal luma neighbor block (xPTnb2,yPTnb2) is computed as follows.

    - xPTnb2 = xPTnb+2+((mvEst[0]+2)>>2)

    - yPTnb2 = yPTnb+2+((mvEst[1]+2)>>2)

- When xPTnb2 is larger or equal to 0 and xPTnb2 is smaller than PicWidthInSamplesL and yPTnb2 is larger or equal to 0 and yPTnb2 is smaller than PicHeightInSamplesL, the following applies.

    - xPTnb = xPTnb2

    - yPTnb = yPTnb2

- Otherwise (availableMvEstFlag is equal to 0), (xPTnb,yPTnb) is not changed.