| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, Switzerland, 17–23 Jan. 2013 | Document: JCT3V-C0118r1 |
| --- | --- |

| Title: | 3D-CE5.h related: Inter-view prediction using image deformation characteristics between multi-view images |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaewon Sung, Sehoon Yea 221, Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Tel: Email: | +82-10-434-5259 {jw.sung, sehoon.yea}.lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes an inter-view prediction method that considers image deformations in multi-view videos by different view point. The plane objects such as walls and floors look horizontally scaled or sheared in other views. However, current HTM 5.0 uses traditional block-based translational motion model for inter-view prediction where the deformations should be signalled by residual data. In this contribution, an inter-view prediction method that searches horizontal scaling and shearing parameters with the disparity vector and transmits to decoder. The disparity compensation process of the decoder makes a horizontally scaled and sheared prediction image using the decoded parameters. Proposed method reportedly showed -0.1%, -0.2%, and -0.1% for V1, V2 and video only cases, respectively. In case of for Undo_Dancer test sequence, the largest bitrate savings -1.6%, -1.2%, -0.4% for V1, V2, and video only cases, respectively.

# Introduction

In the multi-view images, the objects in one view look different in other views due to different view positions. The deformations are affected by baseline distance, different focal length, convergence angle, and so on. However, when the multi-view images are assumed to be rectified and captured by identical cameras, then the deformations happen along the same horizontal scan lines. In this contribution, two types of deformations, horizontal scaling and shearing, are considered under the assumptions. In the following explanation, the scaling and shearing implies horizontal scaling and horizontal shearing.

Figure 1 shows the scaling effect where a plane like a wall is projected to two parallel aligned cameras C1 and C2. The projection of the plane to C1 and C2 results different width. Figure 2 shows the shearing effect where a plane is projected to two parallel aligned cameras C1 and C2. In this example, the plane looks rectangular in C1 but looks sheared parallelogram in C2.

These kinds of deformations always happen between multi-view images. However, the HTM 5.0.1 uses traditional block-based translational motion model for inter-view prediction and the deformations should be signalled by residual data or approximated by splitting the coding blocks to small fragments.

In this contribution, an inter-view prediction method that searches scaling and shearing parameters together with the disparity vector and transmits to decoder. The decoder makes a scaled and sheared prediction image in the disparity compensation process using the decoded parameters.

| Figure 1 | Figure 2 |
| --- | --- |

# Proposed algorithm

## Deformation model

As explained the introduction section, the y components does not affected by the scaling and shearing deformations. Therefore, deformations can be represented by the following equation:

Equation 1

x'y'=αβ01xy+dxdy=αx+βy+dxy+dy.

In the equation α, β represent scaling and shearing parameters, (dx,dy) the disparity vector, (x,y) the coordinate of a pixel in the current coding block, (x',y') the corresponding pixel coordinate in the inter-view reference picture.

## Parameter coding

The scaling and shearing parameters are quantized to three levels: 0, 1, and 2. The three values represent approximately 0.94, 1.00, 1.06 for scaling, and -0.06, 0.00, 0.06 for shearing, respectively. Therefore, the combination of scaling parameter index 1 and shearing parameter index 1 is equal to not applying the scaling and shearing.

In the motion estimation process, the RD costs of 9 combinations of the scaling and shearing parameters {SC0,SC1,SC2} x {SH0,SH1,SH2} are measured and one combination with minimum RD cost is selected. The selected scaling and shearing parameter indices are encoded for AMVP blocks coded by inter-view prediction at P_SLICE, i.e., at the dependent view of random access unit.

To improve the coding efficiency of the scaling and shearing parameter index, merge_left_flag and merge_up_flags are used. In case of the scaling, if the merge_left_flag or merge_up_flag is equal to 1, then the scaling parameter index of the merging neighbor block is used. When both flags are equal to 0, the scaling parameter index of the current coding block is coded. When a neighbor block is not the AMVP block coded by inter-view prediction, the scaling parameter index is derived to 1.

## Implementation

The proposed algorithm is implemented to the HTM5.0.1. When both of the scaling and shearing parameter indices are not equal to 1, the motion estimation and compensation functions implementing the Equation 1 is called where bilinear interpolation is used. The scaling and shearing is not applied to the coding blocks whose depth is larger than 2.

# Experiment results

For experiment, common test condition for coding 3 view texture and depth was used [1]. Table 1 shows the coding performance of the proposed algorithm with respect to HTM5.0.1. BD rate changed by the proposed algorithm were -0.1%, -0.2%, and -0.1% for V1, V2, and video only cases, respectively. The change of encoding and decoding times were small. In case of Undo_Dancer sequence the BD rate gains were -1.6%, -1.2%, and -0.4% for V1, V2, and video only cases, respectively.

Table 1 Coding performance

|  | V0 | V1 | V2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 1.0% | 0.7% | 0.3% | 0.3% | 0.3% | 103.1% | 100.7% | 106.3% |
| Kendo | 0.0% | 0.3% | 0.4% | 0.1% | 0.2% | 0.1% | 102.8% | 95.6% | 110.1% |
| Newspapercc | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 103.0% | 98.6% | 100.9% |
| GhostTownFly | 0.0% | 0.5% | 0.3% | 0.1% | 0.1% | 0.1% | 102.2% | 101.1% | 102.9% |
| PoznanHall2 | 0.0% | -0.1% | -0.3% | -0.1% | 0.0% | -0.1% | 103.5% | 96.9% | 106.1% |
| PoznanStreet | 0.0% | -0.8% | -0.8% | -0.5% | -0.3% | -0.3% | 102.9% | 98.2% | 105.2% |
| UndoDancer | 0.0% | -1.6% | -1.2% | -0.4% | -0.5% | -0.5% | 102.3% | 99.8% | 106.0% |
| 1024x768 | 0.0% | 0.4% | 0.3% | 0.1% | 0.1% | 0.1% | 103.0% | 98.2% | 105.7% |
| 1920x1088 | 0.0% | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 102.7% | 99.0% | 105.0% |
| average | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 102.8% | 98.7% | 105.3% |

# Conclusion

This contribution proposed an inter-view prediction algorithm that considers the horizontal scaling and shearing effects that always happens in the multi-view images. When an AMVP block is coded by inter-view prediction at P-SLICE, the horizontal scaling and shearing parameters are estimated together with the disparity vector and transmitted to decoder. The decoder uses the decoded scaling and shearing parameters to make a scaling and shearing effect compensated prediction image.

Experiment results reportedly showed coding gains without increasing encoding and decoding complexity for some test sequences, such as Poznan_Street and Newspaper. The BD bitrate savings for the Undo_Dancer sequence were -1.6%, -1.2%, and -0.4% for V1, V2 and video only cases, respectively.

Therefore, we recommend further investigation for possible improvements of the proposed algorithm in the CE.

# References

[1] Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments”,  Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1100, October 2012, Shanghai.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).