| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0121 |
| --- | --- |

| Title: | MVC+D: Clarification on Depth acquisition information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang Nanjing R&D Center, 101 Software Avenue, Yuhuatai District, Nanjing, P.R.China 200012 | Tel: Email: | +86 25 56620730 yx.yangxiaofeng@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

Camera parameters are coded in Depth acquisition information SEI message for view synthesis prediction (VSP). However the semantics of some camera related flag are not so clear. This contribution proposes to clarify the semantics of these flags. The proposed solution saves some bits in most cases by only adding minor descriptions to the current specification.

Revision 1: added option2 as indicated by the BoG, and option3 as an alternative

# Introduction

Focal length and principal point are two camera parameters coded in Depth acquisition information SEI message for view synthesis prediction (VSP), they both have horizontal value and vertical value components. The latest specification uses separate flags (focal_length_x_flag, focal_length_y_flag, principal_point_x_flag, principal_point_y_flag) to specify the presence of the values of both directions. If focal length is present, both focal_length_x_flag and focal_length_y_flag shall be 0, otherwise both shall be 1. The current specification allows one of focal_length_x_flag and focal_length_y_flag set to 0 and the other set 1, which is unreasonable. This problem exists with principal point too.

In addition, horizontal focal length equals to vertical focal length when the CCD and the picture have the same aspect ratio, which is quite common. In this case, only one directional value shall be transmitted, with a flag to signal the other value is the same.

For principal point, if the value is not transmitted, it can be inferred to be the default value – the half of the picture width or height. This is also the most common case.

This contribution proposes to use one flag setting to 1 and the other to 0 to signal the case of horizontal value equals to vertical value for focal length. And the absence of the parameter to indicate it equals to the default value.

# Proposal

## changed syntax and semantics

the yellow highlighted texts are the added descriptions

2.1.1 Option1

I.13.2.3 Depth acquisition information SEI message semantics

......

focal_length_x_flag equal to 0 specifies that the syntax elements specifying the horizontal focal length are not present in the syntax structure, and if focal_length_y_flag equal to 1 indicates the horizontal focal length equals to the vertical focal length; focal_length_x_flag equal to 1 specifies that the syntax elements specifying the horizontal focal length are present in the syntax structure.

focal_length_y_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are not present in the syntax structure, and if focal_length_x_flag equal to 1 indicates the vertical focal length equals to the horizontal focal length; focal_length_y_flag equal to 1 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure.

principal_point_x_flag equal to 0 specifies horizontal principal point equals to half of the image width in pixel. principal_point_x_flag equal to 1 specifies that the syntax elements specifying the horizontal principal point are present in the syntax structure.

principal_point_y_flag equal to 0 specifies the vertical principal point equals to half of the image height in pixel. principal_point_y_flag equal to 1 specifies that the syntax elements specifying the vertical principal point are present in the syntax structure.

……

2.1.2 Option2 (for focal length only)

I.13.1.3 Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ) { |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| } else { |  |  |
| numViews = 1 |  |  |
| num_frames | 5 | ue(v) |
| } |  |  |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_flag | 5 | u(1) |
| focal_length_infer_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_flag ) { |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( !focal_length_infer_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| } |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if( rotation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, YawSign, YawExp, YawMantissa, YawManLen ) |  |  |
| } |  |  |
| } |  |  |

I.13.2.3 Depth acquisition information SEI message semantics

......

focal_length_flag equal to 0 specifies that the syntax elements specifying the focal length are not present in the syntax structure, focal_length_x_flag equal to 1 specifies that the syntax elements specifying the focal length are present in the syntax structure.

focal_length_infer_flag equal to 0 specifies that the syntax elements specifying the vertical focal length are present in the syntax structure, equal to 0 specifies that the vertical focal length can be inferred to equal to that of the horizontal focal length.

……

2.1.3 Option3 (for principal point only)

I.13.1.3 Depth acquisition information SEI message syntax

| depth_acquisition_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| all_views_equal_flag | 5 | u(1) |
| if( all_views_equal_flag = = 0 ) { |  |  |
| num_views_minus1 | 5 | ue(v) |
| numViews = num_views_minus1 + 1 |  |  |
| } else { |  |  |
| numViews = 1 |  |  |
| num_frames | 5 | ue(v) |
| } |  |  |
| z_near_flag | 5 | u(1) |
| z_far_flag | 5 | u(1) |
| d_min_flag | 5 | u(1) |
| d_max_flag | 5 | u(1) |
| focal_length_x_flag | 5 | u(1) |
| focal_length_y_flag | 5 | u(1) |
| principal_point_x_flag | 5 | u(1) |
| principal_point_y_flag | 5 | u(1) |
| principal_point_infer_flag | 5 | u(1) |
| translation_flag | 5 | u(1) |
| rotation_flag | 5 | u(1) |
| for( i = 0; i < numViews; i++ ) { |  |  |
| if( z_near_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 0, 7, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| if( d_min_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |  |
| if( d_max_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 0, 7, DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |  |
| if( focal_length_x_flag ) |  |  |
| depth_acquisition_sei_element( i, num_frames, 1, 6, FocalLengthXSign, FocalLengthXExp, FocalLengthXMantissa, FocalLengthXManLen ) |  |  |
| if( focal_length_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, FocalLengthYSign, FocalLengthYExp, FocalLengthYMantissa, FocalLengthYManLen ) |  |  |
| if( principal_point_x_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointXSign, PrincipalPointXExp, PrincipalPointXMantissa, PrincipalPointXManLen ) |  |  |
| if( principal_point_y_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, PrincipalPointYSign, PrincipalPointYExp, PrincipalPointYMantissa, PrincipalPointYManLen ) |  |  |
| if( translation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, TXSign, TXExp, TXMantissa, TXManLen ) |  |  |
| if( rotation_flag ) |  |  |
| depth_acquisition_element( i, num_frames, 1, 6, YawSign, YawExp, YawMantissa, YawManLen ) |  |  |
| } |  |  |
| } |  |  |

I.13.2.3 Depth acquisition information SEI message semantics

......

principal_point_flag equal to 1 specifies the horizontal principal point equals to half of the image height in pixel if principal_point_x_flag equal to 0, and the vertical principal point equals to half of the image height in pixel if principal_point_y_flag equal to 0.

……

## Benefits

- Clearify the unclear settings

- Add useful signalings to the current specification

- Minor change to the current text.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).