| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0124 |
| --- | --- |

| Title: | MVC deblocking for Adaptive Luminance Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyung Yong Kim, Ho Sung Hwang, Gwang Hoon Park, Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-31-201-3680 {kimky, strenght, ghpark} @khu.ac.kr |
| Source: | Kyung Hee University |  |  |

_____________________________

# Abstract

In the last meeting, Adaptive Luminance Compensation (ALC) had been adopted [1] for the 3DV-ATM. The ALC can far improve the coding efficiency of 3DV-ATM, even though it is applied to the video sequences that had been already luminance-compensated. However, the block-based LC mechanism may generate the subjectively-visible blocking artifacts, especially on the 3D video sequences that have not been luminance-compensated (via preprocessing before the video codec applies) as previously reported on the MVC standardization (for example, MVC video sequences).

To remove the blocking artifacts caused by luminance change, a deblocking mechanism for the ALC is also required, especially for the video sequences that are not preprocessed for luminance compensation. In this contribution, we now remind and introduce the deblocking filter for the ALC that had been adopted for the JMVM standardization (called as MVC deblocking). MVC deblocking [3] had been proposed and adopted in the JMVM [2] to improve the subjective picture quality by eliminating blocking artifacts caused by block-based LC. The MVC deblocking [3] and also the simplified MVC deblocking [4] can be simply integrated on the 3DV-ATM 6.0, and then are also tested whether it can reduce the blocking artifacts caused by block-based LC that are subjectively shown to users, especially on the video sequences that are not preprocessed for LC.

Simulation results for the objective picture quality show that the simplified MVC deblocking brings 0.04% BD-rate reduction for texture views with 100% encoding time, and 101% decoding time for the MVC video sequences (those video sequences are not preprocessed for LC). And also the simplified MVC deblocking slightly increased 0.03% BD-rate for texture views and 0.02% BD-rate for synthesized views with 100% encoding time, and 102% decoding time for the 3DV video sequences (most video sequences are preprocessed for LC).

Simulation results for the subjective picture quality show that the simplified MVC deblocking can far reduce the subjectively-visible blocking artifacts caused by block-based LC, especially on the video sequences that had not been compensated on the luminance, while maintaining very similar or slightly-better picture qualities on the 3DV video sequences that had been preprocessed for luminance compensation. These means that the simplified MVC deblocking for ALC can improve the subjectively-visible blocking artifacts while maintaining almost same objective picture quality, especially for the video sequences that are not preprocessed for the luminance compensation.

# Introduction

At present, the JCT-3V has working on the standardization on 3DV-ATM, based on many reference video sequences that have been compensated on the illumination (or luminance) via some sort of the LC (or IC) preprocessors. This means, the preprocessing mechanism for luminance compensation (or rectification, etc) may be necessary before the 3DV-ATM coding standard applied for the 3D video sequences.

In the last meeting, Adaptive Luminance Compensation (ALC) had been adopted [1] for the 3DV-ATM. The ALC can far improve the coding efficiency of 3DV-ATM, even though it is applied to the video sequences that had been already luminance-compensated.

However, the block-based LC mechanism may generate the subjectively-visible blocking artifacts, especially on the 3D video sequences that have not been luminance-compensated (via preprocessing before the video codec applies) as previously reported on the MVC standardization (for example, MVC video sequences).

We now remind and also introduce the deblocking filter for the ALC that had been adopted for the JMVM standardization (called as MVC deblocking).

MVC deblocking had been proposed and adopted in the JMVM [2] to improve the subjective picture quality by eliminating the blocking artifacts caused by block-based LC.

The MVC deblocking [3] adopted in the JMVM was as follows; The boundary filtering strength (bS) is set as 1 or 3 in case the LC is applied to both comparing blocks and also their LC values are not same, or LC is applied to only one comparing block (the other block is not coded by LC mode), even though the current boundary condition for deblocking satisfies that the filtering is not necessary (bS = 0).

In this contribution, the simplified MVC deblocking [4] has been also tested, and its mechanism is as follows; The boundary filtering strength (bS) is set as 1 in case the LC is applied to both comparing blocks and also their LC values are not same, or LC is applied to only one comparing block (the other block is not coded by LC mode), even though the current boundary condition for deblocking satisfies that the filtering is not necessary (bS = 0).

The MVC deblocking [3] and also the simplified MVC deblocking [4] can be simply integrated on the 3DV-ATM 6.0, and then are also tested whether it can reduce the blocking artifacts caused by block-based LC that are subjectively shown to users, especially on the video sequences that are not preprocessed for LC.

Figure 1 shows the difference between current and proposed 3DV-ATM architecture.

Fig. 1. Difference between current and proposed 3DV-ATM architecture

# Phenomenon: Subjectively-visible blocking artifacts caused by block-based LC method

Many current 3DV video sequences used in the 3DV-ATM/HTM standardization are preprocessed for the rectification, and luminance or chrominance compensations.

Therefore, we made an investigation whether the block-based LC method causes the subjectively-visible blocking artifacts or not on the MVC video sequences that had not been preprocessed for the LC (according to JVT-U211 (Common test conditions for multi-view video coding) [5]).

Figure 2 shows the subjectively-visible blocking artifacts caused by ALC on the MVC test sequence (Race1 & AkkoKayo). We can easily see the blocking artifacts in the block-based LC blocks.

| (a) Race1 VGA 5th view, 1st frame (QP=36) | (b) AkkoKayo VGA 5th view, 96th frame (QP=36) |
| --- | --- |

Fig. 2. Subjectively-visible blocking artifacts caused by block-based LC

Normally in the INTER mode of H.264/AVC (or also 3DV-ATM), deblocking filter selects the boundary filtering strength (bS) as 0 when two comparing blocks have similar motions (DIFF < 1) and the same reference picture. However, in case the IC (or LC) is applied, two comparing blocks as shown Fig. 3 can have different IC values if the IC is applied to both blocks or even applied to only one block, whereas the boundary conditions for bS =0 are satisfied. For these cases, mean values between two comparing blocks can be different, and thereby the blocking artifact may be subjectively appeared [4].

Fig. 3. IC (or LC) block boundaries

Therefore, a simple modification of the current H.264/AVC deblocking filter is required to effectively eliminate the blocking artifacts caused by block-based LC.

Following examples as shown in Fig. 4 and Fig.5 are extracted from the MVC contribution paper of [4]. They illustratively show the subjectively-visible phenomenon caused by block-based IC (or LC) methods.

The relationship between IC and the bS strength of deblocking filter had been analyzed in Fig. 4 [4]. In Fig. 4, left image shows the boundary filtering strengths (bS) between IC block boundaries (bS=0: Red, bS=1: Blue, bS=2: Green, bS=3: Magenta, bS=4: Cyan) in the Ballroom QVGA 3rd view, 191st frame (QP=37) and the right image shows the magnified image of the white-dashed rectangle of the left image indicated by the white arrow. One can easily see that the blocking artifacts mainly appear at the IC boundary blocks and their bS = 0 (red boundary lines).

Figure 5 also shows the series of blocking artifacts caused by illumination compensation that are clearly shown in Exit QVGA sequence 3rd view, 12th frame (QP=37) [4]. In Fig. 5, left image shows the boundary filtering strengths (bS) between IC block boundaries (bS=0: Red, bS=1: Blue, bS=2: Green, bS=3: Magenta, bS=4: Cyan) in the Exit QVGA sequence 3rd view, 12th frame (QP=37) and the right image shows the magnified image of the white-dashed rectangle of the left image indicated by the white arrow.

One can also see that the blocking artifacts mainly appear at the boundaries between IC block and non-IC block. If both comparing blocks are the IC blocks and also have the same or very similar IC values, no blocking artifacts may be appears.

Fig. 4. Left image shows the boundary filtering strengths (bS) between IC block boundaries (bS=0: Red, bS=1: Blue, bS=2: Green, bS=3: Magenta, bS=4: Cyan) in the Ballroom QVGA 3rd view, 191st frame (QP=37) and right image shows the magnified image of the white-dashed rectangle of the left image indicated by the white arrow [4].

Fig. 5. Left image shows the boundary filtering strengths (bS) between IC block boundaries (bS=0: Red, bS=1: Blue, bS=2: Green, bS=3: Magenta, bS=4: Cyan ) in Exit QVGA sequence 3rd view, 12th frame (QP=37), and right image shows the magnified image of the white-dashed rectangle of the left image indicated by the white arrow [4].

# MVC deblocking for block-based LC method

## (Case#1) MVC deblocking [3] that were adopted in the JMVM

To remove the subjectively-visible blocking artifacts caused by block-based LC as previously discussed and decided at the MVC standardization [6], we remind and also introduce the MVC deblocking [3] for ALC in this contribution.

In the MVC deblocking (adopted in the JMVM) modified for the block-based ALC in 3DV-ATM, the boundary filtering strength (bS) is set as K1 (=1) or K2 (=3) according to the thresholds T (=5) based on the difference of the ALC weighting value (ValueALC) in case the ALC is applied to both comparing blocks and also their ALC values are not same, or ALC is applied to only one comparing block (the other block is not coded by ALC mode), even though the current boundary condition for deblocking satisfies that the filtering is not necessary (bS = 0).

In case the boundary conditions for deblocking filter satisfies bS = 0 IF - ALC is applied to both comparing blocks & their ALC values are not same OR - ALC is applied to only ONE block (the other block is not coded by ALC mode) IF ¦ValueALCp-ValueALCq¦ >= T (=5) SET bS = K2 (=3: strong deblocking) ELSE SET bS = K1 (=1: weak deblocking) OTHERWISE SET bS = 0

Fig. 6. MVC deblocking filter for ALC mode (Case#1)

## (Case#2) Simplified MVC deblocking [4]

In this contribution, the simplified MVC deblocking [4] that had proposed at MVC standardization is also reminded and tested as follows.

In the simplified MVC deblocking modified for the block-based ALC in 3DV-ATM, the boundary filtering strength (bS) is set as 1 in case the block-based ALC is applied to both comparing blocks and also their ALC weighting values are not same, or ALC is applied to only one comparing block (the other block is not coded by ALC mode), even though the current boundary condition for deblocking filter satisfies that the filtering is not necessary (bS = 0 where two comparing blocks have similar motions (DIFF < 1) and motion compensation is carried out based on same reference frame).

In case the boundary conditions for deblocking filter satisfies bS = 0 SET bS = 1 if (1) ALC is applied to both comparing blocks & their ALC values are not same, OR if (2) ALC is applied to only ONE block (the other block is not coded by ALC mode). OTHERWISE bS = 0

Fig. 7. Simplified MVC deblocking for ALC mode (Case#2)

# Test Conditions

## Conditions for the MVC Sequence

The Table 1. shows the input and output view to be used for experiments.

Table 1. MVC test sequence

| Test Sequence | Image Property | Frames to be encoded | 3-view input |
| --- | --- | --- | --- |
| Ballroom | 640x480, 25fps | 250 | 1-3-5 |
| Exit | 640x480, 25fps | 250 | 1-3-5 |
| Uli | 1024x768, 25fps | 250 | 1-3-5 |
| Race1 | 640x480, 30fps | 300 | 1-3-5 |
| Flamenco2 | 640x480, 30fps | 300 | 1-0-3 |
| Breakdancers | 1024x768, 15fps | 100 | 1-3-5 |
| Rena | 640x480, 30fps | 300 | 1-3-5 |
| Akko&Kayo | 640x480, 30fps | 300 | 5-7-9 |

There are no depth maps for MVC sequence. So, the default depth map is used. The default depth map is generated by setting all value to ‘128’. For independent coding between texture and depth, the coding order configuration was set as “T0T1T2D0D1D2”. The other configuration settings are same as the CTC settings [7].

## Conditions for the 3DV Sequence

Common test conditions specified in JCT3V-B1100 [7] is used for the evaluation.

# Simulation Results

The following Table 2 shows the summary of the simulation results of the MVC deblocking applied for the ALC of 3DV-ATM using two different kinds of the 3D video sequences.

(1) MVC video sequences: Sequences are not luminance-compensated (No-preprocessed).

- The amount of bits for depth map was excluded from the total amount of bits.

(2) 3DV video sequences: Many sequences are preprocessed for the luminance compensation.

Table 2. Simulation results of the MVC deblocking for ALC

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | Enc. time | Dec. time |
| Test-1: MVC (MVC deblocking [3]) | -0.02 % | 0.00 dB | N/A | N/A | -0.02 % | 0.00 dB | N/A | N/A | 100 % | 102 % |
| Test-2: MVC (Simplified MVC deblocking [4]) | -0.04 % | 0.00 dB | N/A | N/A | -0.04 % | 0.00 dB | N/A | N/A | 100 % | 101 % |
| Test-3: 3DV (MVC deblocking [3]) | 0.09 % | 0.00 dB | 0.00 % | 0.00 dB | 0.09 % | 0.00 dB | 0.06 % | 0.00 dB | 100 % | 103 % |
| Test-4: 3DV (Simplified MVC deblocking [4]) | 0.03 % | 0.00 dB | 0.00 % | 0.00 dB | 0.03 % | 0.00 dB | 0.02 % | 0.00 dB | 100 % | 100 % |

- MVC: MVC video sequences

- 3DV: 3DV video sequences

- Case#1: MVC deblocking [3] for ALC

- Case#2: Simplified MVC deblocking [4] for ALC

The following tables show the simulation results of the test cases.

Table 3. Simulation results of Test-1: MVC video sequences (Case#1: MVC deblocking)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| AkkoKayo | 0.13 | -0.01 | N/A | N/A | 0.13 | -0.01 | N/A | N/A | 100% | 101% | N/A |
| ballroom | 0.02 | 0.00 | N/A | N/A | 0.02 | 0.00 | N/A | N/A | 100% | 101% | N/A |
| breakdancers | -0.01 | 0.00 | N/A | N/A | -0.01 | 0.00 | N/A | N/A | 100% | 103% | N/A |
| Exit | -0.05 | 0.00 | N/A | N/A | -0.05 | 0.00 | N/A | N/A | 99% | 101% | N/A |
| Flamenco2 | -0.18 | 0.01 | N/A | N/A | -0.18 | 0.01 | N/A | N/A | 100% | 101% | N/A |
| Race1 | -0.07 | 0.00 | N/A | N/A | -0.07 | 0.00 | N/A | N/A | 100% | 101% | N/A |
| Rena | 0.02 | 0.00 | N/A | N/A | 0.02 | 0.00 | N/A | N/A | 100% | 101% | N/A |
| Uli | 0.00 | 0.00 | N/A | N/A | 0.00 | 0.00 | N/A | N/A | 100% | 102% | N/A |
| Average | -0.02 | 0.00 | N/A | N/A | -0.02 | 0.00 | N/A | N/A | 100% | 102% | N/A |

Table 4. Simulation results of Test-2: MVC video sequences (Case#2: Simplified MVC deblocking)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| AkkoKayo | 0.02 | 0.00 | N/A | N/A | 0.02 | 0.00 | N/A | N/A | 100% | 101% | N/A |
| ballroom | -0.01 | 0.00 | N/A | N/A | -0.01 | 0.00 | N/A | N/A | 100% | 100% | N/A |
| breakdancers | -0.02 | 0.00 | N/A | N/A | -0.02 | 0.00 | N/A | N/A | 100% | 102% | N/A |
| Exit | -0.02 | 0.00 | N/A | N/A | -0.02 | 0.00 | N/A | N/A | 99% | 101% | N/A |
| Flamenco2 | -0.14 | 0.01 | N/A | N/A | -0.14 | 0.01 | N/A | N/A | 100% | 101% | N/A |
| Race1 | -0.14 | 0.01 | N/A | N/A | -0.14 | 0.01 | N/A | N/A | 99% | 101% | N/A |
| Rena | -0.01 | 0.00 | N/A | N/A | -0.01 | 0.00 | N/A | N/A | 99% | 101% | N/A |
| Uli | -0.01 | 0.00 | N/A | N/A | -0.01 | 0.00 | N/A | N/A | 99% | 102% | N/A |
| Average | -0.04 | 0.00 | N/A | N/A | -0.04 | 0.00 | N/A | N/A | 100% | 101% | N/A |

Table 5. Simulation results of Test-3: 3DV video sequences (Case#1: MVC deblocking)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.09 | 0.00 | 0.00 | 0.00 | 0.08 | 0.00 | 0.06 | 0.00 | 100% | 103% | 100% |
| S02 | 0.32 | -0.01 | 0.00 | 0.00 | 0.32 | -0.01 | 0.20 | -0.01 | 100% | 103% | 100% |
| S03 | 0.05 | 0.00 | 0.00 | 0.00 | 0.05 | 0.00 | 0.03 | 0.00 | 100% | 103% | 100% |
| S04 | 0.08 | 0.00 | 0.00 | 0.00 | 0.08 | 0.00 | 0.06 | 0.00 | 100% | 103% | 100% |
| S05 | -0.06 | 0.00 | 0.00 | 0.00 | -0.06 | 0.00 | -0.06 | 0.00 | 100% | 103% | 100% |
| S06 | -0.02 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | 0.00 | 0.00 | 100% | 103% | 100% |
| S08 | 0.15 | -0.01 | 0.00 | 0.00 | 0.15 | -0.01 | 0.09 | 0.00 | 101% | 103% | 100% |
| Average | 0.09 | 0.00 | 0.00 | 0.00 | 0.09 | 0.00 | 0.06 | 0.00 | 100% | 103% | 100% |

Table 6. Simulation results of Test-4: 3DV video sequences (Case#2: Simplified MVC deblocking)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.04 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.01 | 0.00 | 100% | 102% | 100% |
| S02 | 0.18 | -0.01 | 0.00 | 0.00 | 0.18 | 0.00 | 0.12 | 0.00 | 100% | 103% | 100% |
| S03 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.01 | 0.00 | 100% | 102% | 100% |
| S04 | 0.04 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.04 | 0.00 | 100% | 102% | 100% |
| S05 | -0.10 | 0.00 | 0.00 | 0.00 | -0.09 | 0.00 | -0.09 | 0.00 | 99% | 102% | 100% |
| S06 | -0.05 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.02 | 0.00 | 99% | 101% | 100% |
| S08 | 0.07 | 0.00 | 0.00 | 0.00 | 0.07 | 0.00 | 0.05 | 0.00 | 100% | 102% | 100% |
| Average | 0.03 | 0.00 | 0.00 | 0.00 | 0.03 | 0.00 | 0.02 | 0.00 | 100% | 102% | 100% |

# Subjective Comparisons

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 8. Race1 VGA 5th view, 1st frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 9. AkkoKayo VGA 5th view, 96th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 10. Flamenco2 VGA 3rd view, 120th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 11. Rena VGA 1st view, 8th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 12. AkkoKayo VGA 9th view, 240th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 13. ballroom VGA 5th view, 176th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 14. Exit VGA 5th view, 120th frame (QP=36)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 15. AkkoKayo VGA 5th view, 40th frame (QP=41)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 16. Race1 VGA 1th view, 8th frame (QP=41)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 17. Rena VGA 1st view, 56th frame (QP=41)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 18. AkkoKayo VGA 9th view, 56th frame (QP=31)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 19. ballroom VGA 5th view, 56th frame (QP=31)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 20. Exit VGA 5th view, 88th frame (QP=31)

| (a) anchor | (b) Simplified MVC deblocking (Case#2) |  |
| --- | --- | --- |

Fig. 21. Race1 VGA 5th view, 32nd frame (QP=31)

# Conclusion

In this contribution, MVC deblocking methods for the ALC of 3DV-ATM are reminded and introduced. MVC deblocking had previously been proposed and adopted for the JMVM in the MVC standardization. Simulation results show that the simplified MVC deblocking for ALC can improve the subjectively-visible blocking artifacts caused by block-based LC methodology, while maintaining almost same objective picture quality, especially for the video sequences that are not preprocessed for the luminance compensation. We strongly recommend to adopt the simplified MVC deblocking for the ALC of 3DV-ATM WD.

# Reference

[1]	M. Mishurovskiy, A. Fartukov, I. Kovliga and J. Lee, “CE2.A results on inter-view coding with adaptive luminance compensation,” The 2nd JCT-3V meeting, JCT3V-B0031, Shanghai, China, October 2012.

[2] “Joint Multiview Video Model (JMVM) 8.0,” JVT-AA207, Geneva, Switzerland, April 2008.

[3] W. S. Shim, M. W. Park, G. H. Park, D. Y. Suh, H. S. Song, Y. H. Moon, J. B. Choi, “CE5 results- joint prop for MVC deblocking,” JVT-W024, San Jose, USA, April 2007.

[4] G.H. Park, M.W. Park, D.Y. Suh, K. Kim, “MVC deblocking for illumination compensation,” JVT-V033, Marrakech, Morocco, Jan. 2007.

[5] Y. Su, A. Vetro and A. Smolic, “Common Test Conditions for Multiview Video Coding,” JVT-U211, Hangzhou, China, October 2006.

[6] T. Wiegand, G.J. Sullivan, J.R. Ohm, A. Luthra, “Meeting report, Draft 6,” JVT-W200, San Jose, USA, April 2007.

[7]	D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” The 2nd JCT-3V Meeting, JCT3V-B1100, Shanghai, CN, Oct 2012.

# Patent rights declaration(s)

Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).