| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0126 |
| --- | --- |

| Title: | 3D-CE5.h: Cross-check report of JCT3V-C0047 on improved temporal merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yongbing Lin | Tel: Email: | +86 10 62612372 yblin@huawei.com |
| Source: | HiSilicon |  |  |

_____________________________

# Abstract

This document is a crosscheck report of the proposal JCT3V-C0047. In this proposal, temporal motion vector in merge mode exploits an additional target reference index in order to avoid an unavailable case of TMVP candidate due to the mismatch of reference picture type (short or long term) in MV scaling. The target reference index is searched form current reference picture list in a way that the reference picture with the target reference index for current PU has same reference picture type as the reference picture for collocated PU. The software provided by the proponents appears to match the description of the method detailed in JCT3V-C0047. Tests have been run and the results match those provided by the proponents.

# Introduction

To address the issue of unavailable temporal merge candidate caused by the mismatched reference picture type (short or long term) during MV scaling, JCT3V-C0047 [1] proposes to exploit an additional target reference index, instead of a specified reference index.

# Source code verification

The proponents provided with the source code to be tested. It is based on HTM 5.0.1 and the new or modified source code is activated by setting the macro “QC_TMVP_IDX_MOD_B0046” and “QC_TMVP_MRG_IDX”. The macro “QC_TMVP_IDX_MOD_B0046” is inherited from HTM5.1 for alignment with the updated HEVC specification, where the target reference index of temporal merging candidate is set to 0. The other macro “QC_TMVP_MRG_IDX” is related the proposal.

Simulations were tested with the following settings.

## Case #1: with modified HTM5.0.1 anchor

Anchor setting:    QC_TMVP_IDX_MOD_B0046  1, QC_TMVP_MRG_IDX  0

Proposal setting:  QC_TMVP_IDX_MOD_B0046  1, QC_TMVP_MRG_IDX  1

## Case #2: with HTM5.0.1 anchor

Anchor setting:   QC_TMVP_IDX_MOD_B0046  0, QC_TMVP_MRG_IDX  0

Proposal setting: QC_TMVP_IDX_MOD_B0046  0, QC_TMVP_MRG_IDX  1

# Results

The source codes have been successfully compiled using Microsoft VS 8.0 for Windows. All tests are run based on the common tests conditions [2]. The test results of the proposed method according to above test cases are shown in Table 1 and 2. Detailed results can be found in the attached excel sheets. It is confirmed that the obtained bit rate gains exactly match those provided by the proponent.

Table 1: Coding performance of test case #1 with respect to the modified HTM5.0.1 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.0% | -0.8% | -0.4% | -0.3% | -0.2% | 99.8% | 100.1% | 104.0% |
| Kendo | 0.0% | -1.5% | -1.2% | -0.5% | -0.5% | -0.5% | 100.5% | 102.4% | 104.2% |
| Newspaper_CC | 0.0% | -0.4% | -0.4% | -0.2% | -0.1% | -0.2% | 99.4% | 100.2% | 102.2% |
| GT_Fly | 0.0% | -1.5% | -1.3% | -0.4% | -0.4% | -0.4% | 99.5% | 99.8% | 99.2% |
| Poznan_Hall2 | 0.0% | -0.9% | -0.8% | -0.4% | -0.3% | -0.3% | 100.0% | 99.1% | 98.5% |
| Poznan_Street | 0.0% | -0.7% | -0.9% | -0.3% | -0.2% | -0.2% | 99.9% | 100.0% | 95.5% |
| Undo_Dancer | 0.0% | -0.5% | -0.4% | -0.2% | -0.2% | -0.1% | 99.7% | 100.1% | 101.1% |
| 1024x768 | 0.0% | -1.0% | -0.8% | -0.3% | -0.3% | -0.3% | 99.9% | 100.9% | 103.5% |
| 1920x1088 | 0.0% | -0.9% | -0.8% | -0.3% | -0.3% | -0.3% | 99.7% | 99.7% | 98.5% |
| average | 0.0% | -0.9% | -0.8% | -0.3% | -0.3% | -0.3% | 99.8% | 100.2% | 100.6% |

Table 2: Coding performance of test case #2 with respect to HTM5.0.1 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 99.7% | 99.1% | 98.3% |
| Kendo | 0.0% | -0.6% | -0.6% | -0.3% | -0.2% | -0.2% | 99.6% | 101.3% | 97.6% |
| Newspaper_CC | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 100.6% | 100.3% | 97.1% |
| GT_Fly | 0.0% | -0.2% | -0.4% | -0.1% | -0.1% | -0.2% | 100.0% | 98.0% | 100.6% |
| Poznan_Hall2 | 0.0% | -0.3% | 0.3% | 0.0% | 0.0% | 0.0% | 100.1% | 103.6% | 102.2% |
| Poznan_Street | 0.0% | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 100.0% | 101.4% | 97.4% |
| Undo_Dancer | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | 100.2% | 101.3% | 103.8% |
| 1024x768 | 0.0% | -0.4% | -0.4% | -0.2% | -0.2% | -0.1% | 100.0% | 100.2% | 97.6% |
| 1920x1088 | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 100.1% | 101.0% | 101.0% |
| average | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 100.0% | 100.7% | 99.5% |

# References

- L. Zhang, Y. Chen, M. Karczewicz, "3D-CE5.h: Improved temporal motion vector prediction for merge," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0047, 3nd Meeting: Geneva, CH, 17–23 Jan. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.