| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0129 |
| --- | --- |

| Title: | AHG13: DV constraint for inter-view data access |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Since the common test condition (CTC) sequences are all rectified and have no vertical shift between views, in the current HTM there is a constraint forcing the vertical component of the derived disparity vector (DV) used for the inter-view residual prediction to zero. In JCT3V-B0087, it was proposed to apply the same constraint on the DV used for the inter-view motion parameter prediction for rectified multi-view videos. In this proposal, in order to accommodate both rectified and non-rectified multi-view videos, it is proposed to add one flag in video parameter set (VPS) to enable or disable the DV constraint for both inter-view residual prediction and inter-view motion parameter prediction.

# Introduction

In HTM-5.1 [1], to share previously encoded motion information and residual of reference views, inter-view motion parameter prediction and inter-view residual prediction are employed. To derive reference motion parameters and reference residual signal for a current block in a dependent view, a disparity vector (DV) for the current block is firstly derived, and then a reference block in the coded picture of the reference view is located by adding the DV to the location of current block.

Since the common test condition (CTC) sequences [2] are rectified and there is no vertical shift between views, the vertical component of the derived DV for inter-view residual prediction is forced to be zero. As shown in the left part of Figure 1, the DV used to locate the reference residual block can be nonzero only in the horizontal direction. Note that, in the current HTM, there is no such constraint on the DV used to locate the reference motion block for inter-view motion parameter prediction, as shown in the right part of Figure 1. In JCT3V-B0087 [3], we proposed to apply the same DV constraint on inter-view motion parameter prediction for CTC sequences, and the results reportedly showed no coding efficiency loss in the common test condition.

Figure 1.  Inter-view data access for the inter-view residual prediction (left) and the inter-view motion parameter prediction (right).

# Proposal

Although the CTC sequences are all rectified, in real applications, input multi-view videos might not be always rectified. In this contribution, we propose to add one flag in the video parameter set (VPS) to control the DV constraint. When the DV constraint is enabled, the vertical component of each derived DV for inter-view residual or motion parameter prediction is forced to be zero; on the other hand, when the DV constraint is disabled, the vertical component of each derived DV for inter-view residual or motion parameter prediction could be non-zero. Table 1 summarizes the asymmetric DV constraint in current HTM and the proposed scheme with DV constraint enabled/disabled.

Table 1.  Conditions of DV constraint for HTM-5.1 and the proposed scheme

|  |  | Inter-view residual prediction | Inter-view motion parameter prediction |
| --- | --- | --- | --- |
| HTM-5.1 |  | On | Off |
| Proposed scheme | Enable DV constraint | On | On |
|  | Disable DV constraint | Off | Off |

# Experimental results

Additional testing sequences are used to investigate the impact of the DV constraint on the non-rectified multi-view videos. Experiments were conducted according to the common test condition settings, and the software is based on HTM-5.1. Compared to HTM-5.1, the coding performance of the proposed scheme is given in Table 2. Compared to current asymmetric DV constraint in HTM-5.1, disabling the DV constraint for the sequences with vertical shift between views could bring 0.7% and 0.3% BD-rate reductions for MVC sequences and shifted CTC sequences.

The comparison results of the proposed scheme with the DV constraint enabled over the proposed scheme with the DV constraint disabled are summarized in Table 3. As shown in the table, based on the proposed scheme, always enabling the DV constraint for the sequences without vertical shift between views introduce almost no coding loss for CTC sequences and only 0.1% loss for MVC ones while the memory access bandwidth for inter-view data access is reduced. For more detail, the simulation results for different testing sequences are given in the attached excel files.

Table 2.  Summary of experimental results of the proposed scheme over HTM-5.1

| Testing Sequences |  | Enable DV Constraint |  |  | Disable DV Constraint |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | video 1 | video 2 | video PSNR / total bitrate | video 1 | video 2 | video PSNR / total bitrate |
| Without vertical shifts between views | CTC_NoShift | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 0.1% |
|  | MVC_NoShift | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| With vertical shifts between views | MVC_Shift | 6.1% | 6.0% | 2.9% | -1.6% | -1.6% | -0.7% |
|  | CTC_Shift16 | 1.6% | 4.6% | 1.0% | -0.8% | -1.0% | -0.3% |
|  | CTC_Shift32 | 3.8% | 8.2% | 1.9% | -0.9% | -1.2% | -0.3% |
|  | CTC_Shift64 | 7.7% | 11.9% | 3.0% | -0.9% | -1.3% | -0.3% |

Table 3.  Comparison between enabling DV constraint and disabling DV constraint

| Testing Sequences |  | video 1 | video 2 | video PSNR / total bitrate |
| --- | --- | --- | --- | --- |
| Without vertical shifts between views | CTC_NoShift | -0.2% | -0.3% | -0.1% |
|  | MVC_NoShift | 0.1% | 0.0% | 0.1% |
| With vertical shifts between views | MVC_Shift | 7.9% | 7.3% | 3.5% |
|  | CTC_Shift16 | 2.4% | 5.6% | 1.4% |
|  | CTC_Shift32 | 4.7% | 9.5% | 2.2% |
|  | CTC_Shift64 | 8.7% | 13.4% | 3.4% |

# Conclusion

- In this proposal, in order to accommodate both rectified and non-rectified multi-view videos, it is proposed to add one flag in video parameter set (VPS) to enable or disable the DV constraint used for both inter-view residual prediction and inter-view motion parameter prediction.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, October, 2012.

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “3D-CE5.h related: Constrained DV for inter-view data access,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0087, October, 2012.

- Video and Test, Updated Call for Proposals on Multi-view Video Coding, ISO/IEC MPEG, N7567, October, 2005.