| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0130 |
| --- | --- |

| Title: | 3D-CE1.a: Inter-view Skip/Direct mode with sub-partition scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Ling Wu, Yu-Lin Chang, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiling.wu, yulin.chang, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an inter-view Skip/Direct mode is reported. In AVC-based 3D video coding, View Synthesis Prediction (VSP) is a tool to use the reference from the synthetic frame to improve coding efficiency. However, it is costly in terms of number of computations and amount of required storage to perform per-pixel warping. The inter-view Skip/Direct mode is proposed to eliminate full frame view synthesis for reducing the computational complexity, memory bandwidth, and buffer. When disabling the VSP frame generation but with the proposed inter-view Skip/Direct mode active, the experimental results reportedly show an average of 5% decoding time saving with 0.19% and 0.14% BD-rate reduction for coded view and synthesized pictures over the total bit-rate, respectively.

# Introduction

In 3D video coding, View Synthesis Prediction (VSP) provides an inter-view reference from the synthetic frames. A forward mapping VSP was original proposed to provide a synthetic reference as the following method: provided that the texture and depth pair of a first view is coded and decoded, a second view can be predicted by warping the first view to the second view position [1]. Also, a VSP skip/direct mode [2] and a context-based adaptive skip flag positioning method [3] were proposed to use a skip_type flag to adaptively select a synthetic reference or a non-synthetic reference according to the skip status of neighbouring blocks. In 3D-ATM version 5.0, B-VSP [4] is implemented to replace the original forward mapping VSP. A backward mapping view synthesis scheme is used by B-VSP: provided that the texture of a first view and the depth of a second view are coded and decoded, the texture of the second view can be predicted by warping the texture of the first view to the second view position through the converted disparity vector from the depth of the second view.

However, in these implementations, VSP is still quite expensive due to the warping process of every pixel to generate a synthetic picture. In the MPEG 100th meeting, some contributions were proposed to reduce the complexity, such as [4], [6], and [7], which were integrated in 3D-ATM v4.0. In [4], the subsampling was applied only when the pixels in the warped frame are located at sub-pixel positions. The decoding time was reduced to 80% with minor texture coding loss in 3D-ATM v0.3. In [6], a method of low complexity block-based VSP implementation was proposed. The decoding time was reduced to 70% with 30% encoding time increase and minor texture coding loss. In [7], VSP was directly applied on the lower resolution of texture and depth. The decoding time was reduced to 80% with minor texture coding loss in 3DV-ATM v0.3. In the MPEG 101st meeting, Block-based VSP (B-VSP) with backward warping was proposed to reduce the complexity of VSP. The upsampling of the depth was disabled in the proposal and the process of B-VSP with backward warping was conceptually equivalent to 2x2 interview prediction, which is searching a reference from another view with the block size 2x2. In 3D-ATM version 6.1, the B-VSP is further modified to support different block size.

The proposed scheme uses the existing 4x4 block motion compensation without sending VSP reference index for the VSP skip mode. It achieves higher decoding time saving and some total coding gain in 3D-ATM v6.1.

The rest of this contribution is organized as follows. The proposed method is described in Section 2, and the simulation results are shown in Section 3. Finally, Section 4 gives the conclusions.

# Algorithm description

In this contribution, an inter-view Skip mode without full frame view synthesis is proposed. Macroblocks with the inter-view Skip mode provides a good inter-view reference without warping the base view to the independent views. In the inter-view Skip mode, the macroblock is partitioned into blocks, and their corresponding blocks are obtained in the other view as the reference without warping the other view to the current view pixel by pixel.

Figure 1 shows the details of the inter-view Skip mode. Note that View 0 is coded or decoded before View 1 and the texture (Texture 1) is coded after the depth (Depth 1) in View 1. When coding or decoding a macroblock in Texture 1, the macroblock is first partitioned into 16 4x4 blocks. For each 4x4 block, the corresponding block in Texture 0 is obtained from the disparity converted from the depth value. In 3D-ATM v5.0, the depth value of the block is the maximum depth value among the samples in the collocated block in Depth 1. The inter-view reference of the current block in Texture 1 is obtained by collecting 16 4x4 corresponding blocks in Texture 0 without warping Texture 0 to View 1. One flag is needed to indicate the inter-view Skip mode, which is the same as the VSP Skip/Direct mode in [2].

Figure 1 Proposed inter-view Skip and Direct mode

# Test results

The proposed method is implemented on the test model for AVC-based 3D video coding (3D-ATM v6.1), and all the simulations are done following the common test conditions [8]. Since the proposed scheme does not affect HP condidtions, the tests are only carried out for EHP conditions. Four tests are carried out as shown in Table 1[10]. The anchor is ATM 6.1 with VSP block size 2x2, and the VSP picture is inserted in the reference picture list. Test 1 changes the block size for VSP to 4x4 and uses max among four corners while retrieving the depth values. Test 2 also changes the block size for VSP to 4x4 but uses the average value within a depth macroblock while retrieving the depth values. Test 5 is similar to test 1, but the VSP picture is not inserted in the reference picture list. Test 7 is similar to test 2, but the VSP picture is not inserted in the reference picture list. Note that Test 5 and Test 7 use the proposed method in which the EnableVSP is disabled to remove the VSP picture from the reference list and the existing motion compensation module is reused to obtain the reference. Table 2~5 show the BD-rate results of these tests, using 3D-ATM v6.1 CTC as the anchor. Compared with 3DV-ATM v6.1, the proposed method in test 5 reduces about 5% decoding time with small coding gain, which are 0.19% and 0.14% for coded and synthesized view in BD rates, respectively. Note that the decoding time saving mainly comes from disabling the generation of VSP synthetic frame.

Table 1: Combinations of the tools under investigation.

|  | Block size | Disparity vector derivation | VSP picture in the reference picture list | MV coding | Warping direction | Proponent |
| --- | --- | --- | --- | --- | --- | --- |
| ATM 6.1 | 2x2 | N/A | Insert | Skipped | Backward | Anchor |
| Test 1 | 4x4 | Max among 4 corners | Insert | Skipped | Backward | MediaTek |
| Test 2 | 4x4 | Average | Insert | Skipped | Backward | MediaTek |
| Test 3 | 2x2 | N/A | Remove | N/A | Backward | Qualcomm |
| Test 4 | 2x2 | N/A | Insert | Coded | Backward | NTT/MERL |
| Test 5 (1+3) | 4x4 | Max among 4 corners | Remove | N/A | Backward | MediaTek |
| Test 6 (1+4) | 4x4 | Max among 4 corners | Insert | Coded | Backward |  |
| Test 7 (2+3) | 4x4 | Average | Remove | N/A | Backward | MediaTek |
| Test 8 (2+4) | 4x4 | Other methods | Insert | Coded | Backward |  |
| Test 9 | 2x2 | N/A | Insert | Skipped | Forward | Zhejiang Univ. |
| Test 10 | 4x4 | Max among 4 corners | Insert | Skipped | Forward | Zhejiang Univ. |

Table 2: The simulation results of Test 1

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.04 | 0.00 | 0.00 | 0.00 | -0.03 | 0.00 | -0.01 | 0.00 | 109.03 | 96.64 | 104.58 |
| S02 | -0.11 | 0.00 | 0.00 | 0.00 | -0.09 | 0.00 | -0.10 | 0.00 | 99.70 | 86.79 | 97.41 |
| S03 | -0.14 | 0.00 | 0.01 | 0.00 | -0.13 | 0.00 | -0.10 | 0.00 | 105.93 | 110.56 | 94.37 |
| S04 | -0.31 | 0.01 | 0.00 | 0.00 | -0.29 | 0.01 | -0.25 | 0.01 | 113.63 | 99.76 | 102.90 |
| S05 | 0.14 | -0.01 | 0.00 | 0.00 | 0.16 | -0.01 | 0.01 | 0.00 | 108.14 | 99.08 | 100.85 |
| S06 | 0.07 | 0.00 | 0.01 | 0.00 | 0.06 | 0.00 | 0.06 | 0.00 | 108.83 | 92.25 | 97.04 |
| S08 | 0.10 | 0.00 | 0.00 | 0.00 | 0.10 | 0.00 | 0.07 | 0.00 | 101.91 | 97.31 | 97.83 |
| Average | -0.04 | 0.00 | 0.00 | 0.00 | -0.03 | 0.00 | -0.05 | 0.00 | 106.74 | 97.48 | 99.28 |

Table 3: The simulation results of Test 2

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.13 | 0.00 | 0.00 | 0.00 | -0.12 | 0.00 | -0.05 | 0.00 | 95.67 | 91.89 | 93.11 |
| S02 | 0.02 | 0.00 | 0.00 | 0.00 | 0.02 | 0.00 | 0.04 | 0.00 | 100.11 | 103.29 | 100.62 |
| S03 | 0.11 | 0.00 | 0.00 | 0.00 | 0.11 | 0.00 | 0.12 | 0.00 | 108.98 | 90.86 | 122.40 |
| S04 | -0.02 | 0.00 | 0.00 | 0.00 | -0.02 | 0.00 | -0.02 | 0.00 | 98.92 | 98.88 | 99.64 |
| S05 | 0.13 | -0.01 | 0.00 | 0.00 | 0.10 | 0.00 | 0.14 | -0.01 | 99.99 | 93.94 | 105.42 |
| S06 | 0.16 | -0.01 | 0.00 | 0.00 | 0.13 | -0.01 | 0.16 | -0.01 | 101.71 | 101.17 | 105.65 |
| S08 | 0.09 | 0.00 | 0.00 | 0.00 | 0.07 | 0.00 | 0.10 | 0.00 | 95.74 | 97.82 | 100.22 |
| Average | 0.05 | 0.00 | 0.00 | 0.00 | 0.04 | 0.00 | 0.07 | 0.00 | 100.16 | 96.84 | 103.87 |

Table 4: The simulation results of Test 5

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | -0.15 | 0.00 | 0.00 | 0.00 | -0.15 | 0.00 | -0.05 | 0.00 | 85.86 | 73.56 | 100.00 |
| S02 | -0.36 | 0.01 | 0.00 | 0.00 | -0.33 | 0.01 | -0.28 | 0.01 | 99.17 | 105.73 | 97.24 |
| S03 | -0.23 | 0.01 | 0.00 | 0.00 | -0.21 | 0.01 | -0.17 | 0.01 | 99.50 | 100.87 | 95.03 |
| S04 | -0.51 | 0.02 | 0.00 | 0.00 | -0.48 | 0.02 | -0.44 | 0.02 | 106.31 | 97.19 | 96.70 |
| S05 | -0.14 | 0.01 | 0.00 | 0.00 | -0.12 | 0.01 | -0.05 | 0.00 | 92.52 | 104.36 | 98.88 |
| S06 | 0.00 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | 0.08 | 0.00 | 98.69 | 97.92 | 101.84 |
| S08 | -0.06 | 0.00 | 0.00 | 0.00 | -0.05 | 0.00 | -0.04 | 0.00 | 98.98 | 87.12 | 101.99 |
| Average | -0.21 | 0.01 | 0.00 | 0.00 | -0.19 | 0.01 | -0.14 | 0.00 | 97.29 | 95.25 | 98.81 |

Table 5: The simulation results of Test 7

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 0.22 | -0.01 | 0.00 | 0.00 | 0.18 | 0.00 | 0.32 | -0.01 | 99.89 | 96.10 | 95.78 |
| S02 | 0.43 | -0.01 | 0.00 | 0.00 | 0.38 | -0.01 | 0.42 | -0.01 | 101.43 | 98.90 | 103.33 |
| S03 | 1.27 | -0.04 | 0.00 | 0.00 | 1.18 | -0.04 | 1.16 | -0.04 | 102.71 | 100.44 | 102.72 |
| S04 | 0.27 | -0.01 | 0.00 | 0.00 | 0.25 | -0.01 | 0.26 | -0.01 | 99.54 | 109.80 | 100.56 |
| S05 | 0.51 | -0.02 | 0.00 | 0.00 | 0.32 | -0.01 | 0.69 | -0.03 | 97.62 | 100.40 | 103.94 |
| S06 | 0.73 | -0.04 | 0.00 | 0.00 | 0.58 | -0.03 | 0.91 | -0.04 | 94.77 | 90.50 | 99.36 |
| S08 | 0.23 | -0.01 | 0.00 | 0.00 | 0.17 | -0.01 | 0.31 | -0.01 | 93.83 | 96.45 | 104.16 |
| Average | 0.52 | -0.02 | 0.00 | 0.00 | 0.44 | -0.02 | 0.58 | -0.02 | 98.54 | 98.94 | 101.41 |

# Conclusion

An inter-view Skip mode is proposed to avoid the high complexity and huge memory bandwidth caused by full frame view synthesis. The inter-view Skip mode is tested on 3DV-ATM v6.1 and applied on the texture coding without generation of VSP synthetic frame. When disabling the generation of VSP synthetic frame and modifying the VSP Skip/Direct mode to the proposed inter-view Skip mode, the results show that it achieves an average of 5% decoding time saving with small BD-rate gain 0.19% for coded view and 0.14% for synthesized pictures over the total bit-rate.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Tian, D. Graziosi, Y. Wang, N. Cheung, A. Vetro, “Mitsubishi Response to MPEG Call for Proposal on 3D Video Coding Technology”, ISO/IEC JTC1/SC29/WG11 document M22663, Geneva, Switzerland, November 2011.

- ISO/IEC JTC1/SC29/WG11 “3D-AVC-CE1 results on VSP skip/direct mode by Mitsubishi,” M23915, Feb. 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a results on a context-based adaptive skip flag positioning method by Samsung,” M24819, May 2012.

- Wenyi Su et al, “3DV-CE1.a: Block-based View Synthesis Prediction for 3DV-ATM, ”, JCT3V-A0107, Stockholm, Sweden, July 2012

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: Simplification of in-loop view synthesis with fractional-pel disparity,” M24796, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: 3DV-CE1.a-related: Low complexity block-based View Synthesis Prediction,” M24915, May 2012.

- ISO/IEC JTC1/SC29/WG11 “3D-CE1.a related: View synthesis based on asymmetric texture and depth resolutions,” M24941, May 2012.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, ”Common Test Conditions of 3DV Core Experiments”, ISO/IEC JTC1/SC29/WG11 document m27299/JCT3V-B1100, Shanghai, China, Oct., 2012.

- Heiko Schwarz, Dmytro Rusanovskyy, ” Common Test Conditions for AVC and HEVC-based 3DV”, ISO/IEC JTC1/SC29/WG11 document N12560, San Jose, USA, February 2012.

- Shinya Shimizu, Fabian Jäger, ” Description of Core Experiment 1 (CE1) on View Synthesis Prediction”, ISO/IEC JTC1/SC29/WG11 document JCT3V-B1101, Shanghai, China, Oct., 2012.