| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0132 |
| --- | --- |

| Title: | CE1.h related: Depth-oriented Disparity Vector Predictor (DoDVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Chi-Ling Wu, Yu-Pao Tsai, and Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, chiling.wu, yupao.tsai, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In current HTM, the neighboring block disparity vector (NBDV) mode is used to replace the original predicted depth map (PDM) for inter-view motion prediction. In this contribution, a depth oriented disparity vector predictor (DoDVP) is proposed to enhance the accuracy of the estimated disparity vector by utilizing the coded depth map in the same view while flexible coding order is enabled. By referring to the coded depth information, the inter-view information can be predicted more accurately. The experimental results reportedly show that 1.7% and 2.3% BD-Rate gains are achieved for video 1 and video 2, 0.7% BD-Rate gains is achieved for coded views over video bitrate while applying the DoDVP in the AMVP and the merge mode.

# Introduction

In 3D video coding, the coding order of texture and depth is always an issue because of the redundancy between texture and depth. In the 1st JCT-3V meeting, the coding order of texture and depth is defined as coding depth before texture in the dependent views for AVC-based 3D video coding (3D-AVC), and coding texture before depth in the dependent views for HEVC-based 3D video coding (3D-HEVC). Flexible coding order [1] was proposed to change the coding order for 3D-HEVC. The coding efficiency for texture can be improved by referring to the depth information.

The depth information helps to improve the coding efficiency by many different methods. Depth-based motion vector prediction (DMVP) in 3D-AVC uses the coded depth to improve the accuracy of the motion vector prediction. The depth map helps to identify interview candidate for motion parameter prediction. View Synthesis Prediction (VSP) is a popular topic which finds the inter-view reference from the frames warped from other views. In view synthesis prediction proposed by MERL [2], provided that the texture and depth pair of a first view is coded/ decoded, a second view can be predicted by warping the first view to the second view position. The depth map helps the texture to be warped to correct position. Coding tools such as interview motion parameter prediction [3] further utilize the depth information to improve the prediction of the current block. In the response of call for proposal of 3DVC from HHI [3], a predicted depth map (PDM) algorithm was proposed to provide depth information for current texture image. The predicted depth map is warped from the depth map in the coded view or made of coded disparity and motion vectors. The current block can be coded by the motion parameters derived from the predicted depth map which is called inter-view motion prediction. A neighboring block disparity vector (NBDV) is proposed in 3D-HEVC [4]. NBDV uses the disparity vector (DV) from interview prediction in the neighboring block to improve the motion vector prediction. Although NBDV replaces the use of PDM in the 1st JCT-3V meeting, depth information is still proved to be useful in 3D-AVC. As the depth information can be used to improve the coding efficiency, the method to retrieve depth information for the current block becomes important. In this contribution, a method to improve the accuracy of the NBDV by utilizing the coded depth information of the current view is proposed.

# Algorithm Description

In this contribution, a method to replace the NBDV by extracting a more accurate disparity vector from the depth map of the current view is proposed. While the flexible coding order is enabled and the depth is coded before texture, the depth information of the current view is available. A depth block from coded depth map of the same access unit in the current view is first retrieved and used as a collocated depth of the current block. While coding the texture in view 1 and view 2, the depth maps in view 1 and view 2 are already available. So the coding of texture in view 1 and view 2 can be benefited from the depth map in their same view. An estimated disparity vector can be extracted from the depth shown in Fig. 1. The overall flow is as following:

- Check if the depth map in the current view is coded and available

- Use the collocated depth in the current view for current block (coding unit) as depth reference.

- Extract a disparity vector for inter-view motion prediction from the maximum value in the depth reference retrieved in the previous step.

In the example of Fig. 1, the coded depth map in view 1 is denoted as Coded D1. The texture to be coded is T1. The current block (CB) is able to retrieve a depth block at the same position in the coded D1.

- The estimated disparity vector retrieval method

The main idea to replace the NBDV is to use the depth reference to extract a more accurate disparity vector for prediction. In current implementation, the new disparity vector is converted from the maximum disparity in the collocated depth block. The depth value is converted from depth according to the view position and camera parameters. The new disparity vector is called “depth oriented disparity vector predictor” (DoDVP). The overhead of obtaining the DoDVP is the access of the depth reference buffer, which can be further simplified by similar methods proposed to get interview candidate in 3D-AVC.

While the DoDVP is obtained, the disparity vector can be used by several coding tools. The DoDVP is used in the inter-view motion prediction in the AMVP mode and the merge mode in this contribution.

Improvement on the AMVP mode

For including the inter-view motion parameter prediction, the AMVP mode has been extended in a way that an inter-view motion vector predictor is added to the candidate list. It is inserted at the first position of the list. To determine the inter-view motion vector prediction, disparity vector and a reference block in a reference view should be derived. The DoDVP is the derived disparity vector in replacement of NBDV. If the reference index for the current block refers to an inter-view reference picture, the inter-view motion vector predictor is set equal to the corresponding disparity vector. If the current reference index refers to a temporal reference picture and the reference block uses a motion hypothesis that refers to the same access unit as the current reference index, the motion vector that is associated with this motion hypothesis is used as inter-view motion vector predictor. In all other cases, the inter-view motion vector predictor is marked as invalid and is not included in the list of motion vector predictor candidates.

Improvement on the Merge mode

In the merge mode of HEVC, basically the same motion parameters (prediction direction, reference pictures, and motion vectors) as for a neighboring block are used. If a block is coded in the merge mode, a candidate list of motion parameters is derived, which includes the motion parameters of spatially neighboring blocks as well as motion parameters that are calculated based on the motion parameters of the co-located block in a temporal reference picture. The chosen motion parameters are signaled by transmitting an index into the candidate list. Similarly as for the AMVP mode, the candidate list of motion parameters is extended by a motion parameter set that is obtained using inter-view motion prediction, as described in the following.

First the reference block in the base view is determined using the disparity vector derived as described above, which is the DoDVP. If the reference block is coded using MCP, it is tested for each prediction direction of the current block, whether an inter-view motion vector predictor for MCP can be derived. If a picture is included in reference picture lists of the current slice with a picture order count (POC) equal to the POC of one of the reference pictures, the motion parameters of the corresponding block are used as inter-view motion vector candidate and the MCP motion hypothesis of the current block is marked as available.

If none of the MCP motion hypotheses of the current block is marked as available, it is tested for each DCP motion hypothesis of the current block, whether an inter-view motion vector candidate for DCP can be derived. If the reference block is located in an inter-view reference picture of the current slice the inter-view motion vector candidate is set equal to the disparity vector and the DCP motion hypothesis is marked as available. If none of the potential motion hypotheses are marked as available, the inter-view candidate cannot be selected.

Possible improvement on the inter-view residual prediction

The basic principle of the inter-view residual prediction is illustrated in Fig. 2. Similarly as for the inter-view motion prediction, the inter-view residual prediction is based on an estimated disparity vector for the current picture. There is a possible solution to enhance the NBDV with the proposed DoDVP in the inter-view residual prediction. Based on the DoDVP, a disparity vector may be determined for a current block and the residual block in the reference view that is referenced by the disparity vector is used for predicting the residual of the current block.

- Basic concept for the inter-view residual prediction.

# Experiment Results

The proposed methods are integrated into HTM 5.1. Two tests are conducted. Test 1 is the experiment of using HTM 5.1 with flexible coding order T0D0D1D2T1T2. The results in Table 1 show that there’s no difference on texture coding. The motion parameter inheritance and DMM mode are not available in this coding order, so there are some losses in the results of synthesized view. This might be recovered by new tools which rebuild the dependency between depth and texture.

Test 2 is the experiment of using the DoDVP with the AMVP mode and the merge mode. Table 2 shows the coding results. In the test, 1.7% and 2.3% BD-rate saving can be observed for video1 and video2, and 0.7% BD-Rate saving can be observed for coded views over video bitrate.

Table 1 Experiment results of HTM 5.1 with flexible coding order enabled

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.9% | 1.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 2.6% | 2.9% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 1.2% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 2.1% | 2.2% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 2.5% | 2.7% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 1.7% | 1.8% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 2.0% | 2.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 1.3% | 1.8% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 2.1% | 2.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 1.7% | 2.0% |

Table 2 Experiment results of the proposed DoDVP on AMVP and merge mode

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.0% | -2.1% | -0.6% | 0.4% | 0.8% |
| Kendo | 0.0% | -1.8% | -2.5% | -0.9% | 1.9% | 2.2% |
| Newspapercc | 0.0% | -1.0% | -1.2% | -0.4% | 0.0% | 0.8% |
| GhostTownFly | 0.0% | -4.0% | -4.2% | -1.2% | 1.2% | 1.2% |
| PoznanHall2 | 0.0% | -0.3% | -2.2% | -0.6% | 2.2% | 2.3% |
| PoznanStreet | 0.0% | -1.3% | -1.6% | -0.5% | 1.3% | 1.4% |
| UndoDancer | 0.0% | -2.6% | -2.5% | -0.9% | 1.4% | 1.3% |
| 1024x768 | 0.0% | -1.3% | -1.9% | -0.7% | 0.8% | 1.3% |
| 1920x1088 | 0.0% | -2.0% | -2.6% | -0.8% | 1.5% | 1.5% |
| average | 0.0% | -1.7% | -2.3% | -0.7% | 1.2% | 1.4% |

# Conclusion

In this contribution, a method to obtain a more accurate estimated disparity vector is proposed to improve inter-view motion prediction. It utilizes the coded depth of the same access unit in the same view to get a reference depth block. A more accurate estimated disparity vector DoDVP can be retrieved from the depth block to use in the AMVP and the merge mode. The experiments show that 1.7% and 2.3% BD-rate gain on video 1 and video 2, 0.7% BD-rate gain on coded views over video bitrate can be achieved by applying the DoDVP in the AMVP and the merge mode.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- Lan Deyan, Oguz Bici, Dmytro Rusanovskyy, Miska M. Hannuksela, “Flexible Order of Texture and Depth Map Coding for 3DV-HEVC development, “ ISO/IEC JTC1/SC29/WG11 document M23724, San Jose, February, 2012.

- D. Tian, D. Graziosi, Y. Wang, N. Cheung, A. Vetro, “Mitsubishi Response to MPEG Call for Proposal on 3D Video Coding Technology”, ISO/IEC JTC1/SC29/WG11 document M22663, Geneva, Switzerland, November 2011.

- Heiko Schwarz et al, “Description of 3D Video Technology Proposal by Fraunhofer HHI(HEVC compatible; configuration A)” , ISO/IEC JTC1/SC29/WG11 document M22570, Geneva, Switzerland, November 2011.

- Li Zhang et al, “CE5.h: Disparity vector generation results”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-JCT3V-A0097, July 2012, Stockholm.

- Gerhard Tech et al, “3D-HEVC Test Model 1”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-A1005, July 2012, Stockholm.