# Modification to WD

This document describes the modification to working draft.

      - General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …. | u(v) |
| enable_pred_by_depth_flag | u(1) |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- G 8.5.2.1.13	Derivation process for a disparity vector

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- Outputs of this process are:

- a view order index refViewIdx specifying a reference view.

- a flag availableDV specifying whether the disparity vector is available,

- a disparity vector mvDisp.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows.

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

  - PredMode[ xC ][ yC ] is equal to MODE_SKIP

  - PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

If enable_pred_by_depth_flag = 0, the motion vector mvDisp and the availability flag availableDV are derived in the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set equal to 1.

  - For each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ), the following applies.

- When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

        - xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(G-1)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(G-2)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(G-3)

- The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the partition index partIdx, and the luma location ( xN, yN ) as inputs and the output is assigned to the prediction block availability flag availableN.

- When all of the following conditions are true, availableA1 is set equal to 0.

      - N is equal to A1

      - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N

      - partIdx is equal to 1

- When all of the following conditions are true, availableB1 is set equal to 0.

      - N is equal to B1

      - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD

      - partIdx is equal to 1

- When all of the following conditions are true, availableN is set equal to 0.

      - checkParallelMergeFlag is equal to 1

      - (xP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to
(xN >> (log2_parallel_merge_level_minus2 + 2) )

      - (yP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2) ).

- The flag availableIvpMvSearchFlagN is set equal to availableN.

- When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

      - N is equal to B0 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

      - N is equal to B1 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

      - N is equal to B2 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

- The flags availableFlagIvpMvN0 and availableFlagIvpMvN1 are set equal to 0.

- For each X from 0 to 1, the following applies, when availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

          - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(G-4)
mvDisp = MvLXN[ xN ][ yN ]		(G-5)
availableDV = 1		(G-6)

      - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies.

        - When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

            - ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(G-7)
availableFlagIvpMvLXN = 1		(G-8)

  - The derivation process for a disparity vector from temporal neighbour block as specified in Error: Reference source not found is invoked with a luma location ( xP, yP ), variables nPSW and nPSH as inputs, and flag availableDV, and disparity vector mvDisp outputs

  - For each N being A1, B1, B0, A0, and B2, the following applies.

    - For each X from 0 to 1, the following applies.

      - When availableFlagIvpMvLXN is equal to 1, the following applies:

          - mvDisp = ivpMvDispLXN		(G-9)
availableDV = 1		(G-10)

  - [Ed. (GT): In contrast to the previous version where refViewIdx was assumed to be equal to 0. Here refViewIdx is derived together with the motion vectors from neighbouring blocks. Although the whole inter-view motion vector prediction has still the problem as described in the editor comment in subclause Error: Reference source not found and needs further discussion, this approach is more closely aligned to what happens in software. However, for IvpMv refViewIdx and the temporal candidate refViewIdx is not derived here. ]

- Else if enable_pred_by_depth_flag is equal to 1 and depth of the current view is coded, the derivation process as specified in as specified in subclause G.8.5.2.1.18 is invoked with the luma location ( xP, yP ), the reference view index refViewIdx which possesses the same POC and view ID with the current view, and the variables nPSW, nPSH, as the inputs and a flag availableDoDV and a disparity vector mvDoDisp as the outputs. If availableDoDV is equal to 1, the following apply

mvDisp = mvDoDisp		(G-125)
availableDV = 1		(G-126)

- G 8.5.2.1.18	Derivation process for a depth oriented disparity vector predictor

The process is only invoked if enable_pred_by_depth_flag is equal to 1 and depth first coding order.

The specifications in subclause 8.5.2.1.18 apply with the following additional operation at the end of the process.

Inputs to this process are:

–	a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

–	a view identifier refViewIdx specifying a reference view.

–	variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Output of this process are:

- a flag availableDoDV specifying whether the depth oriented disparity vector is available,

- a depth oriented disparity vector mvDoDisp.

If the depth reference in view refViewIdx is available, let refDepthSamples be the (nPSW)x(nPSH) array of reconstructed depth samples with ViewIdx equal to refViewIdx. The luma location (xd0, yd0) of top-left luma sample of refDepthSamples is derived by

xd0 = Clip3( 0, PicWidthInSamplesD – 1, xP )	(G-)
yd0 = Clip3( 0, PicHeightInSamplesD – 1, yP )	(G-)

The luma location (xd1, yd1) of bottom-right luma sample of refDepthSamples is derived by

xd1 = Clip3( 0, PicWidthInSamplesD – 1, xP + ( ( nPSW )	(G-)
yd1 = Clip3( 0, PicHeightInSamplesD – 1, yP + ( ( nPSH )	(G-)

If the depth reference in view refViewIdx is available, the following applies.

depth = INT_MIN

for(j=yd0; j<yd1; j+=1) {

for(i=xd0; i<xd1; i+=1) {

0	if(refDepthSamples[ i, j ] > depth )

depth = refDepthSamples[ i, j ]

}

}

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewIdx )

mvDoDisp[ 0 ] = Disparity( NdrInverse[depth], ZNear[ dps_id, index ], ZFar[dps_id, index ],
  		FocalLengthX[dps_id, index ], AbsTX[ index ] – AbsTX[ refIndex ] )

mvDoDisp[ 1 ] = 0

availableDoDV = 1