G.8.4.1.0.6  Derivation process for temporal luma motion vector prediction

……

Derivation process for a disparity vector from temporal neighbour blocks

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH

Outputs of this process are

- the disparity vector mvDisp,

- the availability flag availableFlag.

The luma location ( xPRb , yPRb) specifying the position of the right bottom luma prediction block of the current luma prediction block and the flag availableRb specifying the availability of the right bottom luma prediction block are derived by the following ordered steps:

  - The vertical component of the position of the right bottom luma prediction block is derived as follows.

      - yPRb = yP + nPbH		(G-1)

  - The horizontal component of the position of the right bottom luma prediction block is derived as follows.

      - xPRb = xP + nPbW		(G-2)

  - When xPRb is less than pic_width_in_luma_samples and yPRb is less than pic_height_in_luma_samples, availableRb is set equal to 1. Otherwise, availableRb is set equal to 0.

[Ed.(GT): This is not reflected in the software. There availableRb is always true, even when (xPRb, yPRb) is outside the LCU or the frame. When (xPRb, yPRb) is outside the frame ( xP, yP ) is used instead.]

[Ed. (CY): Based on the previous comment, this subclause is modified to reflect the current software. Checking whether (xPRb, yPRb) is outside the LCU is currently not implemented but may be desirable.]

The luma location ( xCtr , yCtr) specifying the centre position of the current luma prediction block is derived by

  - xPCtr = xP + ( nPbW >> 1 )		(G-3)

  - yPCtr = yP + ( nPbH >> 1 )		(G-4)

The flag availableFlag is set equal to 0, and both components of mvDisp are set equal to 0.

For i from 0 to NumDdvCandPics, the following ordered steps apply and the whole decoding process of this sub-clause terminates once availableFlag is set to 1.

  - When ViewIdx of the current picture is greater than 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag and the disparity vector mvDisp as outputs.

  - The derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location (xPCtr, yPCtr) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

  - When ViewIdx of the current picture is equal to 1 and availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause Error: Reference source not found is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

……