| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0135 |
| --- | --- |

| Title: | 3D-CE2.h related: Restriction on the temporal blocks for memory bandwidth reduction in DV derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1,3,Yi-Wen Chen2, Jian-Liang Lin2, Shawmin Lei2, Siwei Ma4, Debin Zhao3, Wen Gao4 1MediaTek Beijing, North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 4School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {na.zhang, yiwen.chen, jl.lin, shawmin.lei}@mediatek.com {swma, dbzhao, wgao} @jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology, Peking University |  |  |

_____________________________

# Abstract

In HTM-5.0.1, when searching temporal neighboring blocks to derive the disparity vector (DV), it utilizes different searching orders for different dependent views. Besides, different from the derivation of the temporal motion vector predictor (TMVP), DV derivation could access the temporal motion data residing in lower coding tree unit (CTU) row and add an additional temporal search position at top left corner 4x4 block when the prediction unit (PU) is at the image boundary. The three aforementioned procedures increase the complexity and the overhead of memory access for DV derivation. This contribution proposes to simplify the DV derivation by unifying the searching order for all dependent views and aligning the usage of temporal search positions of DV derivation and TMVP derivation. Experimental results reportedly show that the proposed simplification has no overall coding performance loss compared to HTM 5.0.1, while the DV derivation process is simplified and the memory access is also reduced.

# Introduction

In HTM-5.0.1 [1], a disparity vector (DV) is derived to specify the location of a corresponding block in an inter-view reference picture for the inter-view motion prediction and inter-view residual prediction. The DV is derived from spatial and temporal neighboring blocks. The possible searched temporal neighboring blocks are depicted in Figure 1.

Figure 1.  Temporal candidates for DV derivation in HTM-5.0.1

In HTM-5.0.1, when deriving DV from the temporal neighboring blocks in a temporal collocated picture, it utilizes different searching orders for different dependent views. For view 1, the searching order CTBR is utilized; for those views with view ID larger than 1, the searching order BRCT is utilized. It also uses TL (the top left temporal block) as an additional checking position when the BR temporal block is outside the image boundary. Moreover, it allows to access the BR temporal block which resides in the lower coding tree unit (CTU) row. However, in the derivation of temporal motion vector predictor (TMVP), the BR temporal block residing at the lower CTU row is regarded as unavailable to make temporal data fetch more efficient.

In this contribution, we propose restrictions and unification on the temporal blocks for memory bandwidth reduction in DV derivation.

# Proposed schemes

In this contribution, we propose three simplifications to the current design:

Simplification1: Unify the searching order of the temporal neighboring blocks for all dependent views when deriving DV. We propose to use the searching order BRCT for all dependent views.

Simplification2: Remove the TL temporal block so that the derivation of DV could be aligned with the derivation of TMVP.

Simplification3: Prohibit the access of the BR temporal block residing at the lower CTU row to align with the derivation of TMVP as shown in Figure 2.

Figure 2.  BR temporal block at the lower CTU row is not used in the proposed scheme.

# Experimental results

The proposed three simplifications have been integrated into HTM 5.0.1 [1], and all tests were conducted under common test conditions [2]. Negligible BD-rate changes are observed for overall coded and synthesized results. The detailed experimental results of the combined results, simplification1, simplification2, and simplification3 are given in Table1, Table2, Table3, and Table4, respectively.

Table 1.  Experimental results of the combination

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.2% | 0.6% | 0.1% | 0.1% | 0.1% | 98.5% | 100.7% | 97.3% |
| Kendo | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 97.6% | 99.2% | 95.0% |
| Newspapercc | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 98.4% | 103.6% | 98.3% |
| GhostTownFly | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 99.2% | 100.5% | 99.8% |
| PoznanHall2 | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 99.1% | 100.7% | 97.7% |
| PoznanStreet | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 99.3% | 94.4% | 100.0% |
| UndoDancer | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 98.2% | 98.0% | 99.9% |
| 1024x768 | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 98.2% | 101.2% | 96.8% |
| 1920x1088 | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.0% | 98.4% | 99.3% |
| average | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 98.6% | 99.5% | 98.3% |

Table 2.  Experimental results of simplification 1

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.1% | 99.8% | 98.7% |
| Kendo | -0.3% | 0.0% | -0.1% | 0.0% | 0.0% | 98.1% | 98.6% | 95.8% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.9% | 99.6% | 99.4% |
| GhostTownFly | 0.5% | 0.0% | 0.1% | 0.1% | 0.1% | 99.6% | 101.4% | 99.5% |
| PoznanHall2 | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 98.7% | 99.6% | 98.3% |
| PoznanStreet | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 98.7% | 100.9% |
| UndoDancer | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 98.3% | 99.3% | 99.5% |
| 1024x768 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 99.3% | 98.0% |
| 1920x1088 | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.1% | 99.8% | 99.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.9% | 99.6% | 98.9% |

Table 3.  Experimental results of simplification 2

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.7% | 101.7% | 97.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.3% | 97.5% | 96.5% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.9% | 100.8% | 99.1% |
| GhostTownFly | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 98.8% | 97.9% | 98.4% |
| PoznanHall2 | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 98.9% | 95.1% | 97.2% |
| PoznanStreet | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 97.2% | 100.5% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.3% | 95.9% | 99.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.6% | 100.0% | 97.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.0% | 96.5% | 98.9% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.8% | 98.0% | 98.4% |

Table 4.  Experimental results of simplification 3

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | 0.5% | 0.1% | 0.1% | 0.1% | 98.8% | 97.9% | 97.8% |
| Kendo | 0.2% | 0.5% | 0.1% | 0.1% | 0.1% | 98.2% | 99.4% | 96.4% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.5% | 99.0% | 99.1% |
| GhostTownFly | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 98.8% | 100.3% | 99.2% |
| PoznanHall2 | 0.1% | 0.3% | 0.1% | 0.1% | 0.1% | 99.1% | 100.7% | 99.0% |
| PoznanStreet | -0.3% | 0.3% | 0.0% | 0.0% | 0.0% | 99.2% | 100.2% | 100.9% |
| UndoDancer | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 97.9% | 98.6% | 100.6% |
| 1024x768 | 0.1% | 0.4% | 0.1% | 0.1% | 0.1% | 98.5% | 98.8% | 97.8% |
| 1920x1088 | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 98.7% | 100.0% | 99.9% |
| average | 0.0% | 0.3% | 0.1% | 0.0% | 0.0% | 98.6% | 99.4% | 99.0% |

# Conclusion

This contribution proposes to unify the searching order for all dependent views and align the temporal blocks used in the DV derivation and the TMVP derivation. With the proposed simplification, the DV derivation process is simplified and the memory access is also reduced. Experimental results show that the proposed simplification introduces no BD rate change for overall coding performance.

# Patent rights declaration (s)

MediaTek Inc., Harbin Institute of Technology and Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM 5.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1110, October, 2012.