mb_ivmp_flag equal to 1 indicates that the mb_type and motion information of the current macroblock are not present and are derived from the one or four co-located macroblocks of the texture view component (depending on reduced_resolution_flag is 0 or 1, respectively) in the same view component pair. mb_ivmp_flag equal to 0 indicates that the mb_type and motion information are present. The motion information includes reference indices and motion vectors. When not present, this flag is inferred to be equal to 0.

mb_ivmp_flag shall be equal to 0 if one of the following conditions is satisfied:

–	if reduced_resolution_flag is equal to 0, the co-located macroblock in the texture view component is intra coded, or view synthesis prediction is included for any partition in current macroblock. The co-located macroblock in the texture view component is specified in subclause Error: Reference source not found.

–	if reduced_resolution_flag is equal to 1 and if any of the four co-located macroblocks in the texture view component is intra coded, or its mb_type equal to equal to P_8x8, P_8x8ref0, or B_8x8, or view synthesis prediction is included. The relationsip between current macroblock in the depth view component and its four co-located macroblocks in the texture view component is specified in subclause Error: Reference source not found.

J.8.3.3.3 Derivation process for motion information from one inside view co-located macroblock

This process is invoked when reduced_resolution_flag is equal to 0.

The reference indices, luma motion vectors of the co-located macroblock denoted by ColMb are assigned to the current macroblock.

Denote the reference indices of each reference picture list X of one 8x8 partition mbPartIdx by refIdxLXCol[ mbPartIdx ] (X being 0..1).

- If Macroblock type of ColMb specifies an intra coded type or view synthesis prediction type, motion vectors are set as zero motion vectors, refIdxL0[ mbPartIdx ] is set to 0 and refIdxL1[ mbPartIdx ] is set to -1.  PredMode[ mbPartIdx ] is set to Pred_L0. mb_type[ mbPartIdx ] is set to P_L0_8x8. If current slice is a B slice mb_type[ mbPartIdx ] is set to B_L0_8x8.

–	Otherwise, if both of the following conditions are false, macroblock type and sub-macroblock types of ColMb are inherited by current macroblock.

–	Macroblock type of ColMb is B_8x8 and at least one of the sub-macroblock types is B_Direct_8x8,

–	Macroblock type of ColMb is P_Skip, B_skip or B_Direct_16x16.

–	Otherwise (at least one of the above conditions is true)

–	If the current slice is a P slice, mb_type is set to P_8x8 and sub_mb_type[ mbPartIdx ] is set to P_L0_8x8

–	Otherwise (the current slice is a B slice), mb_type is set to B_8x8, and

–	If both refIdxLXCol[ mbPartIdx ] (X being 0..1) are not equal to -1, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8;

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_LX_8x8, when refIdxLXCol[ mbPartIdx ] is not equal to -1.

J.8.3.3.4 Derivation process for motion information from inside view co-located macroblocks

This process is invoked when reduced_resolution_flag is equal to 1.

mb_type is set to P_8x8 if the current slice is a P slice or B_8x8 if the current slice is a B slice.

Denote the macroblock with address ColMbAddr[ mbPartIdx ] by ColMb and its macroblock type by mbtypeCol.

–	If mbtypeCol is equal to P_Skip, P_L0_16x16, B_LX_16x16, B_Bi_16x16 or B_Skip or B_Direct_16x16 for B slices (X being 0 or 1), the following ordered steps are specified:

- Reference indices are inherited by the 8x8 partition mbPartIdx. Luma motion vectors of ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause Error: Reference source not found.

- SubPredMode[ mbPartIdx ] is derived as follows:

–	If both of list 0 and list 1 reference indices of ColMb are not equal to -1, BiPred is assigned to SubPredMode[ mbPartIdx ].

–	Otherwise (list X reference index of ColMb is not equal to -1), Pred_LX is assigned to SubPredMode[ mbPartIdx ].

- sub_mb_type[ mbPartIdx ] is derived as follows:

–	If the prediction mode is Pred_LX, sub_mb_type[mbPartIdx] is set to P_L0_8x8 for P slices and B_LX_8x8 for B slices, respectively.

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8 for B slices.

–	Otherwise, if mbtypeCol is equal to P_8x8, P_8x8ref0, B_8x8 for B slices or P_L0_L0_MxN, P_L0_L0_MxN, B_X0_X1_MxN for B slices (with X0, X1 being replaced by L0, L1, or Bi, and MxN being replaced by 16x8 or 8x16), let refIdxLXCol[ i ] (i being 0..3) denotes the reference index of four 8x8 partitions of ColMb. The index i of the top-left and bottom-right 8x8 partition is 0 and 3, respectively, as depicted in Figure 6-11. The following ordered steps are specified:

- Reference indices, luma motion vectors of the 8x8 partition with index mbPartIdx are derived as follows:

–	If both of refIdxLXCol[ 0 ] and refIdxLXCol[ 3 ] are -1, refIdxLX[ mbPartIdx ] is set to -1.

–	Otherwise, if refIdxLXCol[ 0 ] is equal to refIdxLXCol[ 3 ], refIdxLXCol[ 0 ] is assigned to refIdxLX[mbPartIdx]. The sub-macroblock partition size for 8x8 partition mbPartIdx is set to 8x4, 4x8 or 4x8 correspoding to the partition size of ColMb equal to 16x8, 8x16 or 8x8, respectively. The luma motion vectors of the two 8x8 partitions are inherited by the two 8x4 or 4x8 partitions of the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, if refIdxLXCol[ i ] (i being 0 or 3) is unequal to -1 and refIdxLXCol[ j ] (j being 3 or 0) is equal to -1, refIdxLXCol[ i ] is assigned to refIdxLX[mbPartIdx] and the luma motion vectors of 8x8 partition with index i in ColMb are inherited by the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, refIdxLXCol[ i ] is assigned to refIdxLX[ mbPartIdx ] with i equal to mbPartIdx and the luma motion vectors of the 8x8 partition with index i in ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause Error: Reference source not found.

- Sub-macroblock partition size MxN is derived as follows:

–	If the sub-macroblock partition size is set to 8x4 or 4x8 in the above step, MxN is set to 8x4 or 4x8.

–	Otherwise, MxN is set to 8x8.

- SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ] of the 8x8 partition mbPartIdx are derived as follows:

–	If current slice is a P slice, Pred_L0 and P_L0_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, if refIdxLX[ mbPartIdx ] (X being 0 or 1) is equal to -1 and refIdxLY[ mbPartIdx ] (Y being 1 or 0) is unequal to -1, Pred_LY and B_LY_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, BiPred and B_Bi_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

- Otherwise if mbtypeCol specifies an intra coded type or view synthesis prediction type, motion vectors are set as zero motion vectors, refIdxL0[ mbPartIdx ] is set to 0 and refIdxL1[ mbPartIdx ] is set to -1.  SubPredMode[ mbPartIdx ] is set to Pred_L0. sub_mb_type[ mbPartIdx ] is set to P_L0_8x8. If current slice is a B slice sub_mb_type[ mbPartIdx ] is set to B_L0_8x8.