| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_additional_layer_operation_points | u(8) |
| num_additional_profile_level_sets_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| reserved_zero_4bits_num_types[ i ] | u(4) |
| reserved_zero_4bits_type[ i ] | u(4) |
| view_id[ i ] | u(8) |
| if ( i>0 ) |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ] && i; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| for( i = 0; i <= num_additional_profile_level_sets_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= num_additional_layer_operation_points; i++ ) { |  |
| op_point( i ) |  |
| if (num_additional_profile_level_sets) |  |
| profile_level_idx[ i ] |  |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if ( (i ! = 0) && !( i % 2 ) ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if ( i % 2 ) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

When merge_idx is equal to 0 and use_mvi_flag is equal to 1, for use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = x0..x0 + nPbW − 1, y = y0..y0 + nPbH − 1:

  - MotionInhFlag[ x ][ y ] = 1		(G-1)

      - Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1

- chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

Let the variable currPic and ListX be the current picture and RefPicListX (with X being 0 or 1) of the current picture, respectively.

The function LongTermRefPic( picX, refIdx, LX ), with X being either 0 or 1, is defined as follows. If the picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) is equal to 1; otherwise LongTermRefPic( picX, refIdx, LX ) is equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as prefFlagL0 and prefFlagL1, the following applies.

- If MotionInhFlag[ xP ][ yP ] is equal to 1, following ordered steps apply.

    - The derivation process for a luma motion vector for motion parameter inheritance mode as specified in subclause G.1.1.1.1.2 is invoked with the luma location ( xP, yP ), and the variables nPbW, nPbH as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1.

    - The variable ivpMvFlagLX and the variable ivpMvDispLX are derived as

        - ivpMvFlagLX = 0		(G-2)
ivpMvDispLX[ 0 ] = 0		(G-3)
ivpMvDispLX[ 1 ] = 0		(G-4)

- Otherwise, IIf PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1.

- Otherwise, if PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma location ( xC, yC ), luma location ( xP, yP ), variables nCS, nPbW and nPbH and the partition index partIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX, ivpMvFlagLX, ivpMvDispLX , and in Pred_LX and in the syntax elements ref_idx_lX and MvdLX, the following applies.

    - The variables refIdxLX and predFlagLX are derived as follows.

      - If inter_pred_idc[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

          - refIdxLX = ref_idx_lX[ xP ][ yP ]		(G-5)
predFlagLX = 1		(G-6)

      - Otherwise, the variables refIdxLX and predFlagLX are specified by

          - refIdxLX = −1		(G-7)
predFlagLX = 0		(G-8)

    - The variable mvdLX is derived as follows.

        - mvdLX[ 0 ] = MvdLX[ xP ][ yP ][ 0 ]		(G-9)
mvdLX[ 1 ] = MvdLX[ xP ][ yP ][ 1 ]		(G-10)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), variables nPbW and nPbH, refIdxLX, and the partition index partIdx as the inputs and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

        - uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216	(G-11)
mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(G-12)
uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216	(G-13)
mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(G-14)

    - The variable ivpMvFlagLX and the variable ivpMvDispLX are derived as

        - ivpMvFlagLX = 0		(G-15)
ivpMvDispLX [ 0 ] = 0		(G-16)
ivpMvDispLX [ 1 ] = 0		(G-17)

NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xB..xB + nPbW − 1, y = yB..yB + nPbH− 1(with X being either 0 or 1):

  - IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(G-18)
IvpMvDispLX[ x ][ y ] = ivpMvDispLX		(G-19)

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the disparity vectors ivpMvDispL0 and ivpMvDispL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1 are derived as specified by the following ordered steps:

  - The derivation process for spatial merge ing candidates from neighboring prediction unit partitions as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

      - availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(G-20) 
predFlagLXCol = availableFlagLXCol		(G-21)

  - Depending on multi_view_mv_pred_flag, the following applies.

  - If multi_view_mv_pred_flag is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

  - Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause  is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the output is assigned to the view order index of the reference view refViewIdx, the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC (with X being 0 or 1, respectively), and the disparity vector mvDisp.

  - Depending on DepthFlag, the following applies.

–	If DepthFlag is equal to 0, the variable availableFlagTexture is set equal to 0.

–	Otherwise (DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause 8.5.2.x.x is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH as the inputs and the outputs are the flag availableFlagTexture, the prediction utilization flags predFlagL0Texture and predFlagL1Texture, the reference indices refIdxL0Texture and refIdxL1Texture, and the motion vectors mvL0Texture and mvL1Texture.

  - The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC and the variable numMergeCand is increased by 1.

  - When availableFlagTexture is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to Texture and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1 and one or more of the following conditions is true,

        - availableFlagIvMC  = = 0 && availableFlagTexture==0,

        - availableFlagIvMC  = = 1 && predFlagLXIvMC ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagIvMC  = = 1 && mvLXIvMC ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagIvMC  = = 1 && refIdxLXIvMC ! = refIdxLXA1 (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && predFlagLXTexture ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && mvLXTexture ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && refIdxLXTexture ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1 and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, and one or more of the following conditions is true ,

        - availableFlagIvMC = = 0&& availableFlagTexture==0,

        - availableFlagIvMC  = = 1 && predFlagLXIvMC ! = predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagIvMC  = = 1 && mvLXIvMC ! = mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagIvMC  = = 1 && refIdxLXIvMC ! = refIdxLXB1 (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && predFlagLXTexture ! = predFlagLXB1, (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && mvLXTexture ! = mvLXB1 (with X being replaced by 0 and 1),

        - availableFlagTexture = = 1 && refIdxLXTexture ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1 and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

        - availableFlagA1 = = 0,

        - predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

        - mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

        - refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

        - availableFlagB1 = = 0,

        - predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

        - mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

        - refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0 and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to B2 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

  - The variable numMergeCand and numOrigMergeCand are is set equal to numMergeCand to the number of merging candidates in the mergeCandList.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

  - The variable MergeIdx is derived as follows.

  - If use_mvi_flag is equal to 0, MergeIdx is set equal to merge_idx[ xP][ yP ].

  - Otherwise (use_mvi_flag is equal to 1), MergeIdx is set equal to merge_idx[ xP][ yP ] −  1.

  - The following assignments are made with N being the candidate at position MergeIdx merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ MergeIdx merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(G-22)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(G-23)

      - refIdxLX = refIdxLXN		(G-24)

      - predFlagLX = predFlagLXN		(G-25)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

      - refIdxL1 = −1		(G-26)

      - predFlagL1 = 0		(G-27)

  - The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

  - If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp [Ed. (GT) There is some redundancy in draft and software since ivpMvDispLX is derived for each list, although it is always equal for both lists.]

  - availableFlagIvMC = = 1

  - MergeIdx = = 0

  - predFlagLXIvMC = = 1

- [Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

- Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.

        - Derivation process for a luma motion vector for motion parameter inheritance mode

This process is only invoked when MotionInhFlag[ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variable textureComp is set equal to the texture view component with view order index equal to ViewIdx and DepthFlag equal to 0. The variable texturePU is set equal to the PU in textureComp that covers the luma location ( xP, yP ).

- For X being replaced by 0 or and 1, the following applies.

  - If texturePU is coded in an intra prediction mode, the following applies.

    - Both components of mvLX are set equal to 0.

    - If slice_type is equal to B or X is equal to 0, the following applies.

      - refIdxLX is set equal to 0.

      - predFlagLX is set equal to 1.

    - Otherwise (slice_type is equal to P and X is equal to 1), the following applies.

      - refIdxLX is set equal to −1.

      - predFlagLX is set equal to 0.

  - Otherwise (texturePU is not coded in an intra prediction mode), the following applies.

    - mvLX is set equal to MvLX[ xP ][ yP ] of textureComp.

    - refIdxLX is set equal to RefIdxLX[ xP ][ yP ] of the textureComp.

    - predFlagLX is set equal to PredFlagLX[ xP ][ yP ] of textureComp.

- G.8.5.2.x.x Derivation process for the texture merging candidate

This process is not invoked when depthflag is equal to 0.

Inputs to this process are:

–	a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

–	variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

–	a flag availableFlagTexture specifying whether the texture merging candidate is available,

–	the prediction utilization flags predFlagL0Texture and predFlagL1Texture,

–	the reference indices refIdxL0Texture and refIdxL1Texture (if availableFlagTexture is equal to 1),

–	the motion vectors mvL0Texture and mvL1Texture (if availableFlagTexture is equal to 1).

The variable availableFlagTexture is set equal to 0. The variables predFlagL0Texture and predFlagL1Texture are set equal to 0. The variables refIdxL0Texture and refIdxL1Texture are set equal to −1. Both components of the motion vectors mvL0Texture and mvL1Texture are set equal to 0. The variables TextureMvFlagL0[ xP, yP ] and TextureMvFlagL1[ xP, yP ] are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 )  )	(G-28)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 )  )	(G-29)

The variables availableFlagTexture, predFlagLXTexture, mvLXTexture, and refIdxLX are derived as following ordered step.

- Let refPredFlagLY, with Y being replaced by 0 and 1, be the variables predFlagLY the texture picture with ViewIdx equal to current ViewIdx. Let refRefIdxLY, with Y being replaced by 0 and 1, be the variables refIdxLY for the texture picture with ViewIdx equal to current ViewIdxthe texture picture with ViewIdx equal to current ViewIdx. Let refMvLY, with Y being replaced by 0 and 1, be the variables mvLY for the texture picture with ViewIdx equal to current ViewIdx. Let refRefPicListLY, with Y being replaced by 0 and 1, be the reference picture list RefPicListLY for the texture picture with ViewIdx equal to current ViewIdx.

- When refPredFlagL0[ xRef ] [ yRef ] is equal to 1, the following apply,

mvL0Texture[ 0 ] = (refMvL0[ xRef ] [ yRef ] [ 0 ] + 2 )>>2	(G-30)
mvL0Texture[ 1 ] = (refMvL0[ xRef ] [ yRef ] [ 1 ] + 2 )>>2	(G-31)
refIdxL0 = refRefIdxL0[ xRef ] [ yRef ]	(G-32)
predFlagL0Texture = 1	(G-33)

availableFlagTexture = 1                                        (G-34)

- When the current slice is a B slices and refPredFlagL1[ xRef ] [ yRef ] is equal to 1, the following apply,

mvL1Texture[ 0 ] = (refMvL1[ xRef ] [ yRef ] [ 0 ] + 2 )>>2	(G-35)
mvL1Texture[ 1 ] = (refMvL1[ xRef ] [ yRef ] [ 1 ] + 2 )>>2	(G-36)
refIdxL1 = refRefIdxL1[ xRef ] [ yRef ]	(G-37)
predFlagL1Texture = 1	(G-38)

availableFlagTexture = 1                                        (G-39)