| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_additional_layer_operation_points | u(8) |
| num_additional_profile_level_sets_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| reserved_zero_4bits_num_types[ i ] | u(4) |
| reserved_zero_4bits_type[ i ] | u(4) |
| view_id[ i ] | u(8) |
| if ( i>0 ) |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ] && i; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| for( i = 0; i <= num_additional_profile_level_sets_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= num_additional_layer_operation_points; i++ ) { |  |
| op_point( i ) |  |
| if (num_additional_profile_level_sets) |  |
| profile_level_idx[ i ] |  |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if ( (i ! = 0) && !( i % 2 ) ) { |  |
| multi_view_mv_pred_flag[ i ] | u(1) |
| multi_view_residual_pred_flag[ i ] | u(1) |
| } |  |
| if ( i % 2 ) { |  |
| enable_dmm_flag[ i ] | u(1) |
| use_mvi_flag[ i ] | u(1) |
| lim_qt_pred_flag[ i ] | u(1) |
| dlt_flag[ i ] | u(1) |
| if( dlt_flag[ i ] ) { |  |
| num_depth_values_in_dlt[ i ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt ; j++) { |  |
| dlt_depth_value[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the disparity vectors ivpMvDispL0 and ivpMvDispL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1 are derived as specified by the following ordered steps:

  - The derivation process for spatial merge ing candidates from neighboring prediction unit partitions as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

      - availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(G-1) 
predFlagLXCol = availableFlagLXCol		(G-2)

  - Depending on multi_view_mv_pred_flag, the following applies.

  - If multi_view_mv_pred_flag is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

  - Otherwise (multi_view_mv_pred_flag is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause  is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the output is assigned to the view order index of the reference view refViewIdx, the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC (with X being 0 or 1, respectively), and the disparity vector mvDisp.

  - Depending on DepthFlag, the following applies.

–	If DepthFlag is equal to 0, the variable availableFlagTexture is set equal to 0.

–	Otherwise (DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause 8.5.2.x.x is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH as the inputs and the outputs are the flag availableFlagTexture, the prediction utilization flags predFlagL0Texture and predFlagL1Texture, the reference indices refIdxL0Texture and refIdxL1Texture, and the motion vectors mvL0Texture and mvL1Texture.

  - The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC and the variable numMergeCand is increased by 1.

  - When availableFlagTexture is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to Texture and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1 and one or more of the following conditions is true,

  - availableFlagIvMC  = = 0,

  - predFlagLXIvMC ! = predFlagLXA1, (with X being replaced by 0 and 1),

  - mvLXIvMC ! = mvLXA1 (with X being replaced by 0 and 1),

  - refIdxLXIvMC ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1 and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1, and one or more of the following conditions is true ,

  - availableFlagIvMC = = 0,

  - predFlagLXIvMC ! = predFlagLXB1, (with X being replaced by 0 and 1),

  - mvLXIvMC ! = mvLXB1 (with X being replaced by 0 and 1),

  - refIdxLXIvMC ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1 and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to B0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

  - availableFlagA1 = = 0,

  - predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

  - mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

  - refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

- availableFlagB1 = = 0,

- predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

- mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

- refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0 and the variable numMergeCand is increased by 1.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to B2 and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + multi_view_mv_pred_flag, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

  - The variable numMergeCand and numOrigMergeCand are is set equal to numMergeCand to the number of merging candidates in the mergeCandList.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

  - The variable MergeIdx is derived as follows.

  - If use_mvi_flag is equal to 0, MergeIdx is set equal to merge_idx[ xP][ yP ].

  - Otherwise (use_mvi_flag is equal to 1), MergeIdx is set equal to merge_idx[ xP][ yP ] −  1.

  - The following assignments are made with N being the candidate at position MergeIdx merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ MergeIdx merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(G-3)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(G-4)

      - refIdxLX = refIdxLXN		(G-5)

      - predFlagLX = predFlagLXN		(G-6)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

      - refIdxL1 = −1		(G-7)

      - predFlagL1 = 0		(G-8)

  - The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

  - If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp [Ed. (GT) There is some redundancy in draft and software since ivpMvDispLX is derived for each list, although it is always equal for both lists.]

  - availableFlagIvMC = = 1

  - MergeIdx = = 0

  - predFlagLXIvMC = = 1

- [Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

- Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.

- G.8.5.2.x.x Derivation process for the texture merging candidate

This process is not invoked when depthflag is equal to 0.

Inputs to this process are:

–	a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

–	variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

–	a flag availableFlagTexture specifying whether the texture merging candidate is available,

–	the prediction utilization flags predFlagL0Texture and predFlagL1Texture,

–	the reference indices refIdxL0Texture and refIdxL1Texture (if availableFlagTexture is equal to 1),

–	the motion vectors mvL0Texture and mvL1Texture (if availableFlagTexture is equal to 1).

The variable availableFlagTexture is set equal to 0. The variables predFlagL0Texture and predFlagL1Texture are set equal to 0. The variables refIdxL0Texture and refIdxL1Texture are set equal to −1. Both components of the motion vectors mvL0Texture and mvL1Texture are set equal to 0. The variables TextureMvFlagL0[ xP, yP ] and TextureMvFlagL1[ xP, yP ] are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by

xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 )  )	(G-9)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 )  )	(G-10)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the texture picture with ViewIdx equal to current ViewIdx.

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply:

- Let refPredFlagLY, with Y being replaced by 0 and 1, be the variables predFlagLY for the prediction unit refPU. Let refRefIdxLY, with Y being replaced by 0 and 1, be the variables refIdxLY for the prediction unit refPU. Let refMvLY, with Y being replaced by 0 and 1, be the variables mvLY for the prediction unit refPU. Let refRefPicListLY, with Y being replaced by 0 and 1, be the reference picture list RefPicListLY for the prediction unit refPU in the texture picture with ViewIdx equal to current ViewIdx.

- When refPredFlagL0 is equal to 1, the following apply,

mvL0Texture[ 0 ] = refMvL0[ 0 ]	(G-11)
mvL0Texture[ 1 ] = refMvL0[ 1 ]	(G-12)
refIdxL0 = refRefIdxL0	(G-13)
predFlagL0Texture = 1	(G-14)

availableFlagTexture = 1                                        (G-15)

- When the current slice is a B slices and refPredFlagL1 is equal to 1, the following apply,

mvL1Texture[ 0 ] = refMvL1[ 0 ]	(G-16)
mvL1Texture[ 1 ] = refMvL1[ 1 ]	(G-17)
refIdxL1 = refRefIdxL1	(G-18)
predFlagL1Texture = 1	(G-19)

availableFlagTexture = 1                                        (G-20)