| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0138 |
| --- | --- |

| Title: | CE4.h results on removal of parsing dependency for inter-view residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents CE4.h results on the inter-view residual prediction (IVRP) in JCT3V-B0093, where two methods are proposed. One method is to solve the parsing dependency problem by removing the conditionally explicit signaling of the IVRP enable flag and applying IVRP to only CUs that select the temporal inter-view merging candidate of merge or skip mode with 2Nx2N partition. The other method is to reduce the complexity of IVRP by removing residual interpolation. The experimental results reportedly show that the proposed methods do not cause any coding efficiency loss.

# Introduction

In HTM, inter-view residual prediction (IVRP) is applied to reduce redundancy between the reference view residual and the current view residual. An IVRP enable flag is conditionally signaled for each coding unit (CU) when all the following conditions are true:

- The current CU belongs to an inter slice, a texture picture, and a dependent view.

- The current CU has at least one prediction unit (PU) using motion-compensated prediction [1].

- One or more transform units (TUs) covered or partially covered by the reference block in the reference view are inter coded and contain at least one nonzero coded block flag (CBF) [2].

In order to decide whether the IVRP enable flag should be decoded or not in the current CU, the decoder has to access the reference index data (for checking whether motion-compensated prediction or disparity-compensated prediction is selected) of the current CU, the CBF data of the region (one or more TUs) covered by the reference block in the reference view, and the prediction mode data (for checking intra or inter) of the region (one or more CUs) covered by the reference block in the reference view. This causes a parsing throughput issue and a parsing dependency issue.

For a CU where the IVRP enable flag is true, each PU in the CU has to determine whether to enable the IVRP or not according to the PU’s reference index data as follows:

- If the current PU only uses motion-compensated prediction, the IVRP is enabled in the PU.

- If the current PU only uses disparity-compensated prediction, the IVRP is disabled in the PU.

- If the current PU is bi-prediction and one reference index indicates motion-compensated prediction and the other reference index indicates disparity-compensated prediction, the IVRP is enabled, but the reference residual used in IVRP is multiplied by 1/2.

Drawbacks of the IVRP in the current HTM5.0.1 are listed as follows.

- The parsing of the IVRP enable flag depends on the information (prediction modes and CBFs) of the reference view. If one piece of information in the reference view is not decoded correctly, the entire picture of the current view may not be parsed correctly, which is clearly undesirable.

- The parsing of the IVRP enable flag requires checking the reference index data (motion-compensated prediction or disparity-compensated prediction) of the current CU, which will decrease the parsing throughput and is undesirable for parsing.

- When the disparity vector points to a sub-sample location, the residual prediction signal is obtained by interpolating the residual samples of the reference view using a bi-linear filter. This will increase the complexity at both encoders and decoders.

In JCT3V-B0093 [3], the first two problems were solved by removing the conditionally explicit signaling of the IVRP enable flag and applying IVRP to only CUs that satisfy a predefined situation. The third problem was solved by simply removing the residual interpolation. In this contribution, the proposed two methods are tested on top of HTM5.0.1.

# Proposed methods

## Removal of parsing dependency

To avoid the parsing dependency and parsing throughput issues, we remove the conditionally explicit signaling of the IVRP enable flag and apply IVRP to only CUs that select the temporal inter-view merging candidate of merge or skip mode with 2Nx2N partition. With this approach, there is no IVRP side information within CUs. Therefore, the parsing problem is totally solved.

## Removal of residual interpolation

To further simplify the IVRP, we propose to remove the residual interpolation. If the disparity vector of the current PU points to a sub-sample location in the reference view, the residual prediction signal is obtained by rounding the sub-sample location to the nearest integer location in the reference view.

# Experimental results

The above methods are integrated and tested on HTM-5.0.1. Three tests are conducted under the common test condition [4]. Test1 is conducted for the proposed method in Section 2.1 to remove the parsing dependency and throughput problems. Test2 is conducted for the proposed method in Section 2.2 to remove the residual interpolation. Test3 is for the combination of Test1 and Test2. The summary of experimental results is listed in Table 1. The results of Test1, Test2, and Test3 are listed in Table 2, Table 3, and Table 4, respectively. As can be seen, the coding efficiency differences between the proposed methods and the anchor are negligible. A side benefit of the first method (Test1) is reduction of the rate-distortion optimization (RDO) operations caused by the removal of the explicit signaling. The encoding time is reduced by 6%.

Table 1.  Summary of test results

| Test cases | Algorithm summary | Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | v1 | v2 | c. & s. | enc. T. | dec. T |
| Test1 | Removal of parsing dependency (Section 2.1) | -0.1% | -0.2% | 0.0% | 94% | 100% |
| Test2 | Removal of residual interpolation (Section 2.2) | 0.2% | 0.2% | 0.0% | 99% | 99% |
| Test3 | Test1 + Test2 | 0.0% | 0.0% | 0.0% | 94% | 100% |

Table 2.  Results of Test1

Table 3.  Results of Test2

Table 4.  Results of Test3

# Conclusion

In this contribution, the proposed methods to remove the parsing dependency and reduce the complexity of the IVRP proposed in JCT3V-B0093 are tested. The conditional parsing flag for the IVRP is removed by applying IVRP to a CU only when the CU is coded by skip or merge mode with the temporal inter-view merging candidate selected and 2Nx2N partition. Compared to the explicit signaling method, the proposed method can totally solve the parsing dependency problem and achieve the same coding efficiency as the current HTM with 6% encoding time saving. The removal of the residual interpolation of the IVRP is also tested to reduce the complexity without any coding loss.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Liu, J. Jia, J. Heo, “Restricted inter-view residual prediction,” Document of ISO/IEC JTC 1/SC 29/WG 11, m24766, April, 2012.

- X. Li, Y. Chen, M. Karczewicz, “3DV-HEVC: Simplified inter-view residual prediction,” Document of ISO/IEC JTC 1/SC 29/WG 11, m24938, April, 2012.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “Removal of the parsing dependency of inter-view residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0093, October, 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, October, 2012.