| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0140 |
| --- | --- |

| Title: | 3D-CE5.a related: Direct MVP derivation with reduced complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In ATM-6.0, the reference index for the spatial motion vector predictor (MVP) in Direct mode is derived as the minimum reference index of three neighboring blocks, while the reference index for the spatial MVP in Skip mode is set to zero. This contribution proposes to set the reference index to 0 in Direct mode also. Such a simplification can improve the cache efficiency of buffering reference blocks, reduce the memory bandwidth for the Direct mode motion estimation, reduce the latency in the spatial MVP derivation, and unify the reference index in Skip and Direct modes. The experimental results reportedly show this simplification brings no coding efficiency loss when compared to the ATM-6.0.

# Introduction

In the current ATM-6.0 design [1], the motion vector predictor (MVP) in the Skip/Direct mode is derived based on a predefined derivation order: the inter-view MVP and the median of three spatial MVPs derived from the neighboring blocks A, B and C, as shown in Figure 1. In the derivation of three spatial MVPs, a target reference index has to be derived firstly. In the current design, the target reference index is set to zero in Skip mode, as shown in the lower part of Figure 2. However, the target reference index of the spatial MVPs for Direct mode is derived as the minimum reference index of the neighboring blocks, as shown in the upper part of Figure 2.

There are several disadvantages of deriving the reference index for the spatial MVP derivation based on the minimum reference index of neighboring blocks.

- Firstly, it takes more cycles to fetch the reference indexes of neighboring blocks and to calculate the minimum reference index among them and thus has higher latency to derive the spatial MVP.

- Secondly, for the motion estimation of Direct mode on the encoder side in which the rate-distortion cost of the spatial MVP is calculated, changing the reference picture for Direct mode from MB to MB will reduces cache efficiency and increases memory bandwidth. In the Direct mode, the target reference picture varies depending on the neighboring blocks. In the worst cases, it will load reference blocks from different reference pictures from MB to MB.

Figure 1.  The spatial neighboring blocks used to derive the spatial candidate in ATM-6.0

Figure 2.  The derivation procedure of the spatial MVP in Skip and Direct mode

# Proposal

In this contribution, we propose to set the target reference index to zero for the spatial MVP derivation in Direct mode. The proposed simplification could reduce the latency in spatial MVP derivation, improve the cache efficiency of buffering reference blocks, reduce the memory bandwidth for the Direct mode motion estimation, and unify the reference index in Skip and Direct modes.

# Experimental results

Experiments were conducted according to the common test conditions [2], and the software is based on ATM-6.0 [1]. Compared to ATM-6.0, the coding performance of the proposed scheme is given in Table 1. The experimental results show that the proposed simplification brings no coding efficiency compared to the current design.

Table 1.  Results of the proposed simplification

# Conclusion

- In this proposal, a simplification is proposed in the spatial MVP derivation to set the target reference index to zero for the spatial MVP derivation in Direct mode. The experimental results reportedly shows that the proposed simplification brings no coding efficiency loss while the latency in spatial MVP derivation is reduced and the cache efficiency for the motion data access is improved.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ATM-6.0, http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv6.0/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, October, 2012.