J.8.3.1.3 Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

Inputs to this process are current macroblock partition index mbPartIdx and subMbPartIdx.

Outputs of this process are the reference indices refIdxL0, refIdxL1, the motion vectors mvL0 and mvL1, the motion vector count variable subMvCnt, and the prediction list utilization flags, predFlagL0 and predFlagL1.

For the derivation of output, the following ordered steps are specified:

- Let the variable currSubMbType be set equal to sub_mb_type[ mbPartIdx ].

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 0 as input and the output is assigned to the motion vector mvL0 and the reference index refIdxL0.

- The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 1 as input and the output is assigned to the motion vector mvL1 and the reference index refIdxL1.

- When both reference indices refIdxL0 and refIdxL1 are equal to -1, the following applies:

- The reference index refIdxL0 is set to 0.

–	The reference indices refIdxL0 and refIdxL1 are derived by:

refIdxL0 = MinPositive( refIdxL0A, minPositive( refIdxL0B, refIdxL0C ) )

refIdxL1 = MinPositive( refIdxL1A, minPositive(refIdxL1B, refIdxL1C ) )

where

MinPositive(x, y) is specified to return Min( x, y ) if both x and y are greater than or equal to 0, or Max ( x,y ) otherwise.

–	When both reference indices refIdxL0 and refIdxL1 are less than 0, refIdxL0 is set equal to 0 and refIdxL1 is set equal to 0.

–	The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, refIdxLX (with X being 0 or 1), and currSubMbType as the inputs and the output being mvLX.