| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0141 |
| --- | --- |

| Title: | 3D-CE2.h related: Improved DV searching order |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1,3, Jicheng An1, Kai Zhang1,Yi-Wen Chen2, Jian-Liang Lin2, Shawmin Lei2, Siwei Ma4, Debin Zhao3, Wen Gao4 1MediaTek Beijing, North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 3 Department of Computer Science and Technology, Harbin Institute of Technology, Harbin, China 150001 4School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {na.zhang, jicheng.an, kai.zhang, yiwen.chen, jl.lin, shawmin.lei} @mediatek.com {swma, dbzhao, wgao} @jdl.ac.cn |
| Source: | MediaTek Inc, Harbin Institute of Technology, Peking University. |  |  |

_____________________________

# Abstract

This contribution proposes a new searching order for the DV derivation. The current order is spatial DCP blocks, temporal DCP blocks, and spatial DV-MCP blocks, while the proposed order is temporal DCP blocks, spatial DCP blocks, and spatial DV-MCP blocks. Experimental results show that the proposed order can bring 0.1% overall coding performance gain compared to HTM-5.0.1 without introducing any complexity increase.

# Introduction

In HTM-5.0.1 [1], a disparity vector (DV) is derived to specify the location of a corresponding block in an inter-view reference picture for the inter-view motion parameter prediction and inter-view residual prediction. The DV is derived from a spatial or temporal neighboring block. Several spatial and temporal neighboring blocks, as shown in Figure 1, are checked in a given order, and the DV is derived as the first available one. The searching order is defined as the following: spatial disparity-compensated prediction (DCP) blocks (A1->B1->B0->A0->B2), temporal DCP blocks (RB or Ctr for non-image boundary, LT or Ctr for image boundary), and spatial disparity vector motion compensated prediction (DV-MCP) blocks (A0->A1->B0->B1->B2).

Figure 1.  Candidate blocks for DV derivation; (a) spatial neighboring blocks; (b) temporal neighboring blocks

# Proposed schemes

In this contribution, we propose to search the temporal neighboring blocks first. The proposed searching order for the DV derivation is: temporal DCP blocks, spatial DCP blocks, spatial DV-MCP blocks.

# Experimental results

The proposed scheme has been integrated into HTM-5.0.1 [1], and the test is conducted under the common test conditions [2]. As shown in Table1, the proposed scheme can bring 0.1% overall coding performance gain compared to HTM-5.0.1 without any complexity increase.

Table 1.  Results of the proposed searching order

|  | Video 1 | Video 2 | Video only | Synthesized only | Coded & synthesized | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.2% | -0.5% | -0.2% | -0.1% | -0.1% | 98.4% | 98.5% | 99.8% |
| Kendo | -0.7% | -0.6% | -0.3% | -0.2% | -0.2% | 97.8% | 95.9% | 94.2% |
| Newspapercc | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 98.3% | 98.6% | 98.2% |
| GhostTownFly | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 98.6% | 103.9% | 98.9% |
| PoznanHall2 | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 98.4% | 101.7% | 97.9% |
| PoznanStreet | -0.4% | 0.0% | -0.1% | -0.1% | -0.1% | 99.5% | 95.5% | 100.4% |
| UndoDancer | -0.3% | 0.3% | 0.0% | -0.1% | -0.1% | 97.9% | 97.4% | 98.9% |
| 1024x768 | -0.4% | -0.4% | -0.2% | -0.1% | -0.1% | 98.2% | 97.6% | 97.3% |
| 1920x1088 | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 98.6% | 99.5% | 99.0% |
| average | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 98.4% | 98.7% | 98.3% |

- Conclusion

This contribution proposes a new searching order for the DV derivation. Experimental results show that the proposed order can bring 0.1% overall coding performance gain compared to HTM-5.0.1 without introducing any complexity increase.

# Patent rights declaration(s)

MediaTek Inc., Harbin Institute of Technology and Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HTM-5.0.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1110, October, 2012.