| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0144 |
| --- | --- |

| Title: | 3D-CE1.h cross-check results on Backward Projection based View Synthesis Prediction using Derived Disparity Vector (JCT3V-C0100) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Lin Chang +886-3-5670766 ext. 25314 yulin.chang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check result for the proposal of NTT on backward projection based view synthesis prediction in 3D-HEVC. The test is cross-checked in this contribution. The verification tasks were done successfully, and the results match those provided by NTT.

# Verification

NTT provided the HTM source codes of their proposed codec described in JCT3V-C0100 [1]. The HTM source code is based on HTM 5.1 which enables the functionality of VSP. There is only one test case in JCT3V-C0100. It is cross-checked in this contribution. The simlations are conducted on the common test condition Error: Reference source not found.

# Results

The experiment result is shown in Table 1. The anchor is HTM 5.1 with common test condition. The BD-rates from our experimental results match those provided by NTT. The encoding time/decoding time of the anchor and the tests were measured on the same platform. The ratio also matches those provided by NTT. The detailed results can be seen in JCT3V-C0144_CE1_NTT_TEST.xls and JCT3V-C0144_CE1_NTT_TEST_new_template.xls for using new excel template.

Table 1 Experiment results

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.5% | 2.6% | 0.7% | 0.7% | 1.2% | 108.2% | 98.2% | 97.5% |
| Kendo | 0.0% | 0.7% | 1.2% | 0.5% | 0.2% | 0.5% | 106.3% | 106.3% | 101.2% |
| Newspaper_CC | 0.0% | 1.9% | 2.3% | 0.9% | 0.6% | 1.4% | 109.1% | 107.4% | 102.3% |
| GT_Fly | 0.0% | -7.0% | -6.1% | -1.6% | -1.6% | -0.7% | 106.9% | 104.4% | 103.7% |
| Poznan_Hall2 | 0.0% | 0.3% | 0.9% | 0.4% | 0.3% | 1.6% | 108.4% | 106.6% | 99.5% |
| Poznan_Street | 0.0% | -2.2% | -1.4% | -0.5% | -0.4% | 0.0% | 110.6% | 102.0% | 102.4% |
| Undo_Dancer | 0.0% | -10.8% | -7.9% | -2.6% | -2.4% | 0.4% | 109.6% | 103.0% | 101.9% |
| 1024x768 | 0.0% | 1.0% | 2.0% | 0.7% | 0.5% | 1.0% | 107.9% | 103.9% | 100.3% |
| 1920x1088 | 0.0% | -4.9% | -3.6% | -1.1% | -1.0% | 0.3% | 108.8% | 104.0% | 101.8% |
| average | 0.0% | -2.4% | -1.2% | -0.3% | -0.4% | 0.6% | 108.4% | 103.9% | 101.2% |

# Conclusion

The result presented by NTT in JCT3V-C0100 is confirmed.

# References

- S. Shimizu, S. Sugimoto, H. Kimata, “CE1.h: Backward Projection based View Synthesis Prediction using Derived Disparity Vector”, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0100, Jan. 2013, Geneva.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B1100, October, 2012.