| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0148 |
| --- | --- |

| Title: | 3D-CE5.h: Additional merge candidates derived from shifted disparity candidate predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christine Guillemot Laurent Guillo | Tel: Email: | +33 299847100 Christine.guillemot@inria.fr Laurent.guillo@inria.fr |
| Source: | INRIA, France. |  |  |

_____________________________

# Abstract

The coding performance of the HTM 5.0.1 can be improved by inserting in the candidate merge list two new candidates, the maximal length of list still being equal to 6. These candidates are derived from the first disparity candidate vector found in the list, which is  then horizontally shifted by +4 and -4. Bit rate gains are respectively 0.4% and 0.5% for the side views 1 and 2. The overall bit rate gain is 0.2%.

# Introduction

In 3D-HEVC, up to six merging candidates are made available for selection when inter-view motion prediction is enabled. The efficiency of this approach relies on how relevant vectors present in the list are. In HTM 5.0.1, several adoptions improved the candidate merge list by pruning duplicated candidates and by inserting, when IMPV is available, disparity motion vectors derived from disparity vectors [1][2].

It is still possible to improve the coding performance by adding two new candidates derived from the first disparity candidate vector found in the candidate merge list in adding a horizontal shift of -4 or +4 to it. That is equivalent to a horizontal shift of -1 or +1 pixel.

The method described below proposes to replace existing candidates of the candidate merge list described in the text specification draft with two candidates pointing to the base view and horizontally shifted by the offset -4 and +4 respectively.

This proposal is the follow-up of JCT3V-B0080.

# Proposed method

First, the merge candidate list is constructed as described in the section “G.8.5.2.1.1 Derivation process for luma motion vectors for merge mode” [3]. Nothing is changed with respect to the way it is done in HTM 5.0.1 A possible candidate merge list could be as depicted in Figure 1:

Figure 1: Example of a candidate merge list.

In this list, each motion vector candidate is defined with:

- The luma motion vectors mvL0 and mvL1,

- The reference indices refIdxL0 and refIdxL1,

- The prediction list utilization flags predFlagL0 and predFlagL1.

Once the candidate merge list is constructed, the refinement of the list is a three step process:

- Search for  the 1st disparity vector,

- Creation of the two new candidates,

- Insertion of the new candidates into the candidate merge list.

These three steps are detailed below.

## Search for the 1st disparity vector

The candidate merge list is scanned from the first position (index 0) to the third one (index 2). The first candidate found in the list during this search which has either its L0 vector or L1 vector or both pointing to another view is marked as selected candidate  (a vector mvLX is pointing to another view if the viewId of current coding unit is not equal to the viewId of reference refIdxLX).

If no such candidate exists, the process ends.

## Creation of the two new candidates

Let m be the index of the selected candidate. This index m is higher or equal to 0 and lower or equal to 2. The selected candidate is defined by mvL0m, mvL1m, refIdxL0m, refIdxL1m, predFlagL0m, and predFlagL1m. Two refined candidates candDCPa and candDCPb are constructed as follows:

If mvL0m is pointing to another view:

mvL0candDCPa = mvL0m + (4, 0) // if current view is 2, +(-4, 0) otherwise

mvL1candDCPa = mvL1m

refIdxL0candDCPa = refIdxL0m

refIdxL1candDCPa = refIdxL1m

predFlagL0candDCPa = predFlagL0m

predFlagL1candDCPa = predFlagL1m

mvL0candDCPb = mvL0m + (- 4, 0) // if current view is 2, +(4, 0) otherwise

mvL1candDCPb = mvL1m

refIdxL0candDCPb = refIdxL0m

refIdxL1candDCPb = refIdxL1m

predFlagL0candDCPb = predFlagL0m

predFlagL1candDCPb = predFlagL1m

Else  (mvL1m vector is pointing to another view):

mvL0candDCPa = mvL0m

mvL1candDCPa = mvL1m + (4, 0) // if current view is 2, +(-4, 0) otherwise

refIdxL0candDCPa = refIdxL0m

refIdxL1candDCPa = refIdxL1m

predFlagL0candDCPa = predFlagL0m

predFlagL1candDCPa = predFlagL1m

mvL0candDCPb = mvL0m

mvL1candDCPb = mvL1m + (- 4, 0) // if current view is 2, +(4, 0) otherwise

refIdxL0candDCPb = refIdxL0m

refIdxL1candDCPb = refIdxL1m

predFlagL0candDCPb = predFlagL0m

predFlagL1candDCPb = predFlagL1m

## Insertion of the new candidates into the candidate merge list

The two new candidates will be added at the 4th position (index 3) and 5th position (index 4). The previous 4th and 5th candidates might be overwritten. However, the previous 4th candidate will be moved to the 6th position (index 5) first. These two operations are depicted in the following figures and the pseudo code listed after.

Figure 2: Moving of the fourth candidate to the last position in the candidate list.

Figure 3: Overwriting of the fourth and fifth candidates by newly generated candidates.

// insertion of the two new candidates

mvL05  = mvL03

mvL15  = mvL13

refIdxL05 = refIdxL03

refIdxL15 = refIdxL13

predFlagL05 = predFlagL03

predFlagL15 = predFlagL13

mvL03  = mvL0candDCPa

mvL13  = mvL1candDCPa

refIdxL03 = refIdxL0candDCPa

refIdxL13 = refIdxL1candDCPa

predFlagL03 = predFlagL0candDCPa

predFlagL13 = predFlagL1candDCPa

mvL04  = mvL0candDCPb

mvL14  = mvL1candDCPb

refIdxL04 = refIdxL0candDCPb

refIdxL14 = refIdxL1candDCPb

predFlagL04 = predFlagL0candDCPb

predFlagL14 = predFlagL1candDCPb

# Experimental results

This section provide results of the proposed method in comparison with the 3D-HTM anchor. The 3D-HTM 5.0.1 [4] is utilized and the method is implemented on top of it. All tests are run based on the common tests conditions [5].

Only the method TComDataCU::getInterMergeCandidates in  TComDataCU.cpp is modified (and also TypeDef.h to activate the macro INRIA_NEW_MERGE_CAND).

Results are provided in table 1. The overall average bitrate reduction is 0.2%, 0.1%, 0.2% for decoded texture views, synthesized views, coded and synthesized views, respectively.

No added complexity is reported.

Table 1: Coding gains with respect to anchor for 3-view case

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,2% | 0,0% | 0,0% | 0,0% | 0,0% | 95,5% | 102,0% | 106,3% |
| Kendo | 0,0% | -0,4% | -0,7% | -0,3% | -0,1% | -0,1% | 97,4% | 101,9% | 110,7% |
| Newspapercc | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 103,3% | 98,2% | 113,8% |
| GhostTownFly | 0,0% | -0,5% | -0,9% | -0,2% | -0,2% | -0,2% | 101,6% | 102,4% | 106,0% |
| PoznanHall2 | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% | 99,4% | 99,0% | 104,4% |
| PoznanStreet | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 102,9% | 101,1% | 106,7% |
| UndoDancer | 0,0% | -0,3% | -0,4% | -0,1% | -0,2% | -0,2% | 101,6% | 101,5% | 104,8% |
| 1024x768 | 0,0% | -0,3% | -0,4% | -0,2% | -0,1% | -0,1% | 98,7% | 100,7% | 110,2% |
| 1920x1088 | 0,0% | -0,4% | -0,5% | -0,2% | -0,2% | -0,2% | 101,4% | 101,0% | 105,5% |
| average | 0,0% | -0,4% | -0,5% | -0,2% | -0,1% | -0,2% | 100,2% | 100,9% | 107,5% |

# Conclusion

This proposal provides an overall bit rate gain of 0.2% with no added complexity. It is proposed to adopt it.

# References

- L. Zhang, Y. Chen, "3D-CE5.h: Merge candidates derivation from disparity vector", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0048, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

- E. Mora, B. Pesquet-Popescu, M. Cagnazzo, J. Jung, "3D-CE5.h related: Modification of the Merge Candidate List for Dependant Views in 3DV-HTM", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0069, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 2”, ", ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1005, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

- 3DV-HTM version 5.0.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-5.0.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B1110, 2nd Meeting: Shanghai, CN, 13–19 Oct 2012.

# Patent rights declaration(s)

INRIA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).