| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0150 |
| --- | --- |

| Title: | The report on cross-check of simplifications for adaptive luminance compensation in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alexey Fartukov Igor Kovliga Mikhail Mishurovskiy | Tel: Email: | +7-495-7972500 3756 a.fartukov@samsung.com +7-495-7972500 3765 i.kovliga@samsung.com +7-495-7972500 3760 mike.mishky@samsung.com |
| Source: | Samsung R&D Russia Institute, Samsung Electronics Co. LTD |  |  |

_____________________________

# Abstract

This contribution summarizes cross check results on Qualcomm’s simplifications for adaptive luminance compensation in 3D-AVC (JCT3V-C0054). Performance evaluation results reported by proponent are confirmed. The simulation results (BD Rates and PSNR’s) received by Samsung - Moscow are matched with the results provided by Qualcomm.

# Test results

Software provided by the proponent has been used for cross check. Common test conditions specified in JCT3V-B1100 [1] is used for the evaluation. In the simulation, the anchor is 3DV-ATM v6.0r1. Simulation results for this configuration are shown in the Table 1.

Table 1. Test results for Qualcomm’s simplifications for adaptive luminance compensation.

| Sequences | dBR%, PSNR of decoded views, total rate (depths + views) | dBR%, PSNR of synthesized views, total rate (depths + views) |
| --- | --- | --- |
| S01 (Poznan Hall2) | 0.00 | 0.08 |
| S02 (Poznan Street) | 0.02 | 0.00 |
| S03 (Undo Dancer) | 0.01 | 0.01 |
| S04 (GT Fly) | -0.01 | -0.01 |
| S05 (Kendo) | 0.07 | 0.02 |
| S06 (Balloons) | 0.09 | 0.04 |
| S08 (Newspaper) | 0.09 | 0.04 |
| Average | 0.04 | 0.02 |
| Encoding Time (%) | 100 |  |
| Decoding Time (%) | 100.02 |  |

# Summary

Cross check of proposed adaptive illumination compensation simplification was done by running simulations of Qualcomm’s software. BD-Rates values are matched perfectly with the results provided by Qualcomm. Small differences in the synthesized results are observed that are probably due-to different HTM Renderer versions. Yet, small deviations in execution times are observed. It is supposed that these deviations are due-to different computational platforms used for simulations (Samsung uses Windows-based PC, while Qualcomm uses Linux-based computer).

# Reference

- D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, The 2nd JCT-3V meeting, JCT3V-B1100, Shanghai, China, October 2012.