| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C0151 |
| --- | --- |

| Title: | 3D-CE5.h: Cross check of JCT3V-C0148 on additional merge candidates derived from shifted disparity candidate predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, E. Mora 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results for the contribution JCT3V-C00148. It has been verified that common conditions are followed and that results provided by the proponent (Inria) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Proposal verification

This proposal is the follow-up of JCT3V-B0080.

In 3D-HEVC, up to six merging candidates are made available for selection when inter-view motion prediction is enabled.

The proposal adds two new candidates derived from the first disparity compensated candidate found in the candidate merge list in adding a horizontal shift of -4 or +4 to it. That is equivalent to a horizontal shift of -1 or +1 pixel.

The method described below proposes to replace existing candidates of the candidate merge list described in the text specification draft with two candidates pointing to the base view and horizontally shifted by the offset -4 and +4 respectively.

Once the candidate merge list is constructed, the refinement of the list is a three step process:

- Search of the 1st disparity compensated predictor,

The candidate merge list is scanned from the first position (index 0) to the third one (index 2). The first candidate found in the list during this search which has either its L0 vector or L1 vector or both pointing to another view is marked as selected candidate. If no such candidate exists, the process ends.

- Creation of the two new candidates,

Two refined candidates candDCPa and candDCPb are constructed, based on the selected candidate

- Insertion of the new candidates into the candidate merge list.

The two new candidates will be added at the 4th position (index 3) and 5th position (index 4). The previous 4th and 5th candidates might be overwritten. However, the previous 4th candidate will be moved to the 6th position (index 5) first. These two operations are depicted in the following figures and the pseudo code listed after.

Figure 1: Moving of the fourth candidate to the last position in the candidate list.

Anchor: HTM5.0.1 with common conditions.

Tested: anchor with proposed method

The software has been verified and compiled. It does not raise any issue.  Point décodeur soulevé par EM ?

Results obtained by Inria and Orange Labs are reported in the following table.

Results obtained by Inria:

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,2% | 0,0% | 0,0% | 0,0% | 0,0% | 95,5% | 102,0% | 106,3% |
| Kendo | 0,0% | -0,4% | -0,7% | -0,3% | -0,1% | -0,1% | 97,4% | 101,9% | 110,7% |
| Newspapercc | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 103,3% | 98,2% | 113,8% |
| GhostTownFly | 0,0% | -0,5% | -0,9% | -0,2% | -0,2% | -0,2% | 101,6% | 102,4% | 106,0% |
| PoznanHall2 | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% | 99,4% | 99,0% | 104,4% |
| PoznanStreet | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 102,9% | 101,1% | 106,7% |
| UndoDancer | 0,0% | -0,3% | -0,4% | -0,1% | -0,2% | -0,2% | 101,6% | 101,5% | 104,8% |
| 1024x768 | 0,0% | -0,3% | -0,4% | -0,2% | -0,1% | -0,1% | 98,7% | 100,7% | 110,2% |
| 1920x1088 | 0,0% | -0,4% | -0,5% | -0,2% | -0,2% | -0,2% | 101,4% | 101,0% | 105,5% |
| Average | 0,0% | -0,4% | -0,5% | -0,2% | -0,1% | -0,2% | 100,2% | 100,9% | 107,5% |

Results obtained by Orange Labs:

|  | video 0 | video 1 | video 2 | video only | synthesized only | coded & synthesized | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,2% | 0,0% | 0,0% | 0,0% | 0,0% | 112,6% | 98,7% | 98,2% |
| Kendo | 0,0% | -0,4% | -0,7% | -0,3% | -0,1% | -0,1% | 114,3% | 100,8% | 98,3% |
| Newspapercc | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 104,6% | 100,1% | 116,0% |
| GhostTownFly | 0,0% | -0,5% | -0,9% | -0,2% | -0,2% | -0,2% | 102,5% | 99,6% | 100,2% |
| PoznanHall2 | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% | 97,7% | 99,4% | 98,4% |
| PoznanStreet | 0,0% | -0,7% | -0,6% | -0,3% | -0,2% | -0,2% | 108,1% | 96,8% | 100,1% |
| UndoDancer | 0,0% | -0,3% | -0,4% | -0,1% | -0,2% | -0,2% | 115,4% | 101,7% | 101,5% |
| 1024x768 | 0,0% | -0,3% | -0,4% | -0,2% | -0,1% | -0,1% | 110,4% | 99,8% | 103,8% |
| 1920x1088 | 0,0% | -0,4% | -0,5% | -0,2% | -0,2% | -0,2% | 105,6% | 99,4% | 99,7% |
| average | 0,0% | -0,4% | -0,5% | -0,2% | -0,1% | -0,2% | 106,4% | 98,5% | 100,3% |

Perfect match was achieved.

# Conclusion

This verification contribution confirms the results obtained by Inria.